3GPP TSG RAN WG1 Meeting #109-e	R1-2204878

e-Meeting, May 9th – 20th, 2022

Agenda item:		8.9

Source:	Nokia, Nokia Shanghai Bell

Title:	Support of 16-QAM in NB-IoT TDD

Document for:		Discussion and Decision

# Introduction

In this contribution, we address the issue of 16-QAM transmission in TDD special subframes.

# 16-QAM Transmission in TDD Special Subframe

In [1], the issue of using 16-QAM in DwPTS was highlighted with the observation that further rate matching would be needed to accommodate reduced number of available symbols in the DwPTS. Since Rel-15, the following agreements have been specified with respect to TDD special subframe –

- For NPDSCH

  - Transmission in DwPTS is supported for in-band when the number of OFDM symbols in DwPTS is greater than 3.

  - Transmission in DwPTS is supported for guard-band and standalone for all DwPTS configurations.

- Do not map transmissions to DwPTS and UpPTS in special subframe configuration 10.

- UpPTS is not used for NPUSCH and NPRACH.

Note that since UpPTS cannot be used for NPUSCH, only NPDSCH transmission on DwPTS using 16-QAM needs to be discussed. Unlike for QPSK, for 16-QAM, repetition cannot be used. Therefore, the transmission must be self-decodable (e.g. coding rate lower than 0.932) within the DwPTS since NPDSCH uses convolutional coding with chase combining (and therefore re-transmission cannot lower the code rate). There are several options on how to handle this –

- Leave it to eNB implementation to ensure the DwPTS is self-decodable (i.e. do not schedule NPDSCH to the UE if it cannot be decoded after rate matching to DwPTS).

- Prohibit the use of 16-QAM in DwPTS in some special subframe configurations (e.g. #0 and #5 in case of standalone and guard-band since they only have 3 OFDM symbols).

- Specify TBS adjustment like LTE such that NDSCH transmission in DwPTS is self-decodable. For instance, in eMTC the PRB is adjusted by a factor of 0.375 for special subframe configuration 9 and 0.75 for other configurations prior to determining the TBS.

In our view, this issue is not unique to 16-QAM as similar problem can also happen when QPSK is scheduled without repetition. In Rel-15, no special handling was specified and it was left to eNB implementation to ensure that the DwPTS is self-decodable. Our preference is to follow the same principle for 16-QAM in DwPTS.

In addition, 16-QAM is also supported with multi-TB transmissions. In this case, it is up to the eNB to ensure that both TBs are self-decodable when one of them will be transmitted on the DwPTS.

Proposal 1: It is up to the eNB to ensure that NPDSCH transmission on DwPTS using 16-QAM is self-decodable (e.g. coding rate lower than 0.932) by the UE after rate matching. There is no need for specification change.

# Conclusions

In this contribution, we consider NPDSCH transmission using 16-QAM in DwPTS and make the following proposal –

Proposal 1: It is up to the eNB to ensure that NPDSCH transmission on DwPTS using 16-QAM is self-decodable (e.g. coding rate lower than 0.932) by the UE after rate matching. There is no need for specification change.

# References

- R1-2112363, “On the support of 16-QAM for unicast in UL and DL in TDD NB-IoT,” Ericsson, RAN1#107-e, electronic meeting.