3GPP TSG RAN WG1 #109-e	R1-2204881

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.7.1

Source:	Ericsson

Title:	Modeling and evaluation methodology for network energy saving

Document for:	Discussion and Decision

# Introduction

In RAN#94, it was agreed to start a Rel 18 SI on network energy savings (latest WID in [7]), with the following objectives.

- Definition of a base station energy consumption model [RAN1]

- Adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 to the base station side, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, base station load, etc), sleep states and the associated transition times, and one or more reference parameters/configurations.

- Definition of an evaluation methodology and KPIs [RAN1]

- The evaluation methodology should target for evaluating system-level network energy consumption and energy savings gains, as well as assessing/balancing impact to network and user performance (e.g. spectral efficiency, capacity, UPT, latency, handover performance, call drop rate, initial access performance, SLA assurance related KPIs), energy efficiency, and UE power consumption, complexity. The evaluation methodology should not focus on a single KPI, and should reuse existing KPIs whenever applicable; where existing KPIs are found to be insufficient new KPIs may be developed as needed.

Note: WGs will decide KPIs to evaluate and how.

- Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include:

- How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2]

- Information exchange/coordination over network interfaces [RAN3]

Note: Other techniques are not precluded

The study should prioritize idle/empty and low/medium load scenarios (the exact definition of such loads is left to the study), and different loads among carriers and neighbor cells are allowed.

In this contribution, we provide inputs for objectives 1 and 2, i.e., modelling of gNB energy consumption, evaluation methodology and KPIs.

# Discussion

## gNB power model

In this section, general aspects of gNB power model are discussed, followed by power model structure, scaling and approach for deriving the energy consumption for NR channel/signal transmission/reception.

### General aspects

The power model should reflect practical gNB technology at a few years’ time horizon. Both overly conservative estimates, assuming older technology, and highly optimistic estimates presuming technology disruptions should be avoided. Furthermore, both hardware and software should be considered in developing the appropriate model.

In previous discussions, a model [1] from 2015 has been used and companies have broadly considered it a qualitative and quantitative reference. The power levels and the time scales in the paper, shown in Figure 3 (in Annex), have been used in previous discussions. The presentation structure of the model is somewhat different from the UE power model in Rel-16/17. It is therefore desirable to introduce an improved model that better reflects the current trends and the analysis structure anticipated in the SI.

Both energy efficiency (EE) during active operation and energy consumption in the absence of traffic are important. Nevertheless, preliminary analysis of energy consumption patterns indicates that most gratuitous energy expenditure occurs in “idle”; when the gNB is not communicating with UEs. The modeling should thus particularly ensure that aspects related to “idle” operations (e.g. SSB transmissions and PRACH monitoring, etc) and related sleep modes and transitions are well covered.

Model details are expected to differ for a variety of gNB types, e.g., macro/micro/small BS, AAS/indoor designs, etc. If multiple model agreements are not feasible in the SI, the modeling effort should at least consider AAS-type gNBs which are common in current deployments.

The gNB power model should ensure detailed support for low-load and “idle” mode modeling, and be applicable to at least AAS-type gNB designs reflecting current technology trends and evolutions in a few years horizon.

### Power model structure

In Rel-16/17, UE power consumption analysis was based on the model agreed in the SI TR [2]. The model includes following aspects that were used in UE EE work:

- Power levels associated with different UE operations, e.g., specific channel reception

- Power levels associated with multiple sleep states

- Transition times and transition energies associated with entering and leaving sleep states

- Power consumption scaling depending on the active number of antennas, BW

- Applicable to FR1 and FR2

It is desirable that similar model features and a similar presentation, especially related to the sleep transitions, be provided for gNB power and energy consumption analysis.

Develop a gNB power model structurally similar to the UE model in TR38.840 that includes power and/or energy levels of relevant operations, sleep states and transitions, as well as scaling for TX and RX BW and the number of active antenna ports.

Similar to UE power consumption model in TR 38.840 [1], the model should cover 100% PRB in active transmission and reception, and multiple sleep states with different transition times. In contrast to UE where all operations can be linked to single UE-gNB communication and specific channel processing, a gNB operation may include communication or broadcast to a varying number of UEs and on different channels and with different signals. Furthermore, in a typical TDD implementation, TX/RX hardware are entangled, and thus the radio may only adopt/operate in a sleep mode when there is nothing to transmit or receive, e.g., in a symbol. In case there is something to transmit or receive, then the gNB may not be able to choose a sleep mode. As such, UL and DL network energy consumption modelling at least for TDD scenarios should be considered together.

In typical TDD AAS scenario, UL and DL implementations are entangled and thus, it is preferred to model them together. If there is something to transmit or receive in a symbol, the gNB can not go to sleep.

Based on these discussions, we can consider at least the following states and their associated transition times to active transmission or reception as being essential in modelling of network energy consumption.

- Active transmission (100% PRB)

  - the transmitted symbol occupies 100% of available PRBs and all hardware resources are utilized to maximize spectral efficiency and baseband processing capability.

- Active reception (100% PRB)

  - this models UL reception at the gNB, i.e., PRACH, SRS, PUCCH and PUSCH.

- “Micro” sleep

  - the gNB does not need to operate TX/RX within the current symbol. PA or LNA can be turned off.

- “Light” sleep

  - the gNB does not need to operate TX/RX at least within the next X1 ms (~[5] ms). Additional hardware components, i.e., in addition to PA can be turned off, e.g., transceiver chains. Power should be lower compared to micro sleep.

- “Deep” sleep

  - the gNB does not need to operate TX/RX at least within the next X2 ms (~[50] ms). Even more hardware components can be turned off with respect to light sleep, and the minimum can be kept ON, e.g., the clock. Power should be lower compared to light sleep.

- “Hibernate” sleep

  - the gNB does not need to operate TX/RX within the next X3 ms (~ [1000] ms). All hardware/software components can be turned off. Power should be lower compared to deep sleep.

From connection perspective, the gNB and UE remain connected during micro-, light- and deep sleep, i.e., UE is camped on the cell either in RRC_connected or RRC_idle/inactive, while the connection becomes lost for the hibernate mode, i.e., the cell is totally off from a UE’s perspective. Figure 1 shows the described states and the transition from each sleep mode to the active state. The active state is applicable to both transmission and reception at the gNB side.

Figure 1 State transition model

The gNB power model should include at least following states:  active transmission (100% PRB operation),  active reception (100%PRB), micro sleep, light sleep, deep sleep, and hibernate sleep.

In some reference modelling, another state is also defined called an idle or no load or 0%PRB. In this case, the hardware components are active but are not transmitting/receiving. This state does not exist by its own but defines a reference floor for the power consumption when gNB transmits or receives something. In other words, this can be considered as a pseudo-state when the transceiver is ready to transmit or receive in a symbol.

Also, in line with the UE power model, the power and energy levels provided by the model should be relative. As in the UE model [2], the deep sleep level may be used as the reference unit for gNB power model and power levels of other states and operations may be expressed as multiples of it. This mitigates the effect of systematic differences in measurement or estimation methodology.

The gNB power model should be expressed in relative units, related to the deep sleep power level.

Table 1 shows the power states and power levels that can be used for gNB power model. The values are given considering AAS gNB.

Table 1. Power level of different states at gNB.

| Operation/state | Power level, relative units |
| --- | --- |
| Active transmission (100% PRB) | [200] |
| Active reception (100% PRB) | [100] |
| “Micro”-sleep | [60] |
| “Light”-sleep | [25] |
| “Deep”-sleep | 1 |
| “Hibernate”-sleep | [0.1] |

Figure 2  shows the relative powers of Table 1 using a bar graph.

Figure 2. Illustration of the power levels for different operations/states.

We note that deep sleep is not the lowest available power level; further significant power savings are possible when the gNB is in hibernate mode, i.e., the cell is off and is not transmitting anything including SSBs. The cell is known to other cells, but not to the UE at least in the current setups.

Next, energy for state transition is discussed.  A gNB consumes energy to transition to different sleep modes from active and vice versa. These energy consumptions should not be omitted since a gNB may take hundreds of ms to fully reactivate from sleep mode (e.g. in hibernate), and the power consumed in sleep mode may be still comparable to active transmission or reception as seen in Table 1 and Table 2. Hence, it is necessary to include additional transition energy when establishing and evaluating the gNB power model.

The gNB power model should include at least transition energy between different gNB sleep states and active transmission/reception state.

The additional transition energy from active state to sleep mode (SM) k and vice versa can be calculated using i=1k-12∙Ti+Ti+12∙PSM,i-PSM,i+1 where PSM,i, Ti is the power level and half the transition time of ith sleep mode, respectively [3]. The definition of additional transition energy is the same for UE power saving model in 38.840 [2].

Table 2. Energy and transition time from active to different sleep mode and vice versa.

| Transitions | Additional transition energy (Relative power x ms) | Transition time (ms) (incl. ramp down/up) |
| --- | --- | --- |
| Active ó microsleep | 0 | 0 |
| Active ó light-sleep | [90] | [5] |
| Active ó deep-sleep | [750] | [50] |
| Active ó hibernate sleep | [1220] | [1000] |

### Power model scaling

Like UE power model, scaling can be applied for gNB power model based on transmission and reception BW, carrier aggregation, and the number of antenna ports. This is shown in Table 3. Regarding Transmission Bandwidth (DL), more BW entails more output power if a fixed power density spectrum is used, resulting in more power consumed by the PAs. In addition, when PRB utilization is reduced to 0 percent, the current power level falls to 50% of the power level of Active transmission (100% PRB). In contrast to transmission, the power level in reception is around [100] and reducing PRBs barely changes the power level since the power consumed by the LNAs is not as high as the power consumed by the PAs.

The gNB power model should consider the scaling of relative power levels at least for active transmission/reception and at least for BW, CA, and antenna ports adaptations.

  - FFS: If and how the maximum output power scales the active transmission power consumption.

Table 3. Scaling for gNB power model.

| Scaling factor | Proposal | Comments |
| --- | --- | --- |
| Transmission Bandwidth (DL) | Scaling of X MHz = [0.5] + [0.5] * X /100 | relative to active transmission (100% PRB). |
| Reception Bandwidth (UL) | Scaling of X MHz = [0.8] + [0.2] * X / 100 | relative to active reception (100%PRB). |
| CA (DL) | 2CC = [1.7] CC |  |
| CA (UL) | 2CC = [1.7] CC |  |
| Number of antennas ports | [0.4] + [0.6]*(x/64) where x is either 64, 32, 16 that represents the number of antennas | Antenna ports are reduced for both UL and DL. Antenna ports adaptation impacts active transmission/reception and micro sleep. |
| Maximum output power | FFS: If and how the maximum output power scales the active transmission power consumption |  |

### Energy consumption for NR channels/signals

Next, we discuss how total energy can be calculated for NR channels/signals.

The model in previous section can be used at the symbol level. Therefore, in a very simple way irrespective of the signal, or channel, a formula can be used to determine the total power consumption in a transmitted symbol. Denoting a as 0%PRB power consumption, and b as the 100%PRB power consumption, the total energy consumption for X%PRB utilization per symbols is a+X(b-a)/100. In case of the model presented in the previous section, the calculated energy consumption per symbols of X%PRB utilization is 100+X. When it comes to a reception symbol, this is simply [100] for all the UL channels, if the total BW is used, otherwise, the UL BW scaling as in Table 3 can be applied.

When it comes to slot level, we can also easily extend the above formula, i.e., in a simple way, if all the symbols in the slot are used for transmission/reception, then we can use the above formulas and scaling. However, if some symbols are not used for transmission or reception, then depending on the available time to sleep, the appropriate sleep mode (typically microsleep if only symbol level duration) becomes applicable. E.g., if in one slot only one SSB is transmitted, this means 4 symbols out of 14 symbols are transmitting and the other 10 symbols will be in micro sleep. Considering SSB to have for example, 20 PRBs per symbol, this means 20/275 or almost 7% PRB utilization. Therefore, the total power consumption for transmission of only one SSB over a slot is (4x107+10x60)/14~73.

Similar examples can be applied to other types of signals and channels. A gNB can transmit multiple signals/channels in the same slot, therefore, in our opinion a methodic example as in this section suits better for network energy consumption evaluations than per channel definitions. For example, a BS may decide to transmit SSB and PDSCH in the same slot or transmits PDCCH to multiple UEs in the same slot.

## Evaluation methodology

In this section, evaluation methodology including target scenarios and traffic models are discussed for evaluating network energy saving techniques.

### Target scenarios

To have a starting point, it is necessary to come up with common reference configurations or target scenarios for FR1 and FR2 when analysing the network energy consumption model so that the model, simulation results, etc. will be in-line with each other. In addition, the network scenarios or references should also be in-line with UE power saving analyses in TR 38.840 [2] as much as possible. In addition, target scenarios should be also in-line with the current deployments for NR. For example, urban micro based TDD massive MIMO for FR1, or beam based massive MIMO for FR2. Table 4 shows the target scenario of the study. The baseline should be separated for FR1 and FR2 to have a separate gNB power model like the UE power model in TR 38.840 [2].

Table 4. Target Scenarios

| Reference configurations | FR1 | FR2 |
| --- | --- | --- |
| Subcarrier spacing (SCS) | 30 kHz | 120 kHz |
| Number of carriers | 1CC | 1CC |
| System Bandwidth | 100 MHz | 100 MHz |
| SSB periodicity | 20 ms | 20 ms |
| Number of SSB | up to 6 | up to 48 |
| Paging in idle | One PO configured per frame and 10% are used | One PO configured per frame and 10% are used |
| SI assumption | SIB1 every 20ms, 4 SI blocks every 640ms | SIB1 every 20 ms, 4 SI blocks every 640ms |
| Multiplexing | TDD | TDD |

In addition to the above, some other parameters for system level simulation (e.g., the BS and UE setup) may need to be agreed. For example, it will be beneficial to have an alignment on the parameter such as BS height, carrier frequency, BS distance, etc. Similar with the UE power saving work, the simulation assumption for the system level simulation may use table A2.1-1 of TR 38.802. It should be noted, however, different values on the parameter can also be used whenever reasonable and justified.

Table A2.1-1 of TR 38.802 can be used as the baseline for system level simulation assumptions. Other assumptions are not precluded.

### Traffic model

From the gNB perspective, in one period, the energy consumption will highly depend on the number of PRBs occupied during the transmission. Therefore, the average resource utilization (RU) of the simulation should be agreed as it will heavily affect the energy saving gain evaluation of the techniques discussed in the study item. In addition, it has been agreed in the SID that the network energy saving SI will put emphasis on the low and medium load scenarios.

At least an average resource utilization of 10% and 30%, is included for the network energy-saving evaluations. Other values are not precluded.

In the UE power saving SI and WI, several traffic models were agreed each with its associated DRX setups. For example, the following setups are agreed for FTP traffic and IM traffic.

Table 5. Traffic model

| Parameters | FTP traffic | Instant messaging |
| --- | --- | --- |
| Model | FTP model 3 | FTP model 3 |
| Packet size | 0.5 Mbytes | 0.1 Mbytes |
| Mean inter-arrival time | 200 ms | 2 sec |
| DRX setting | Period = 160 ms Inactivity timer = 100 ms On-duration = 8 ms | Period = 320 ms Inactivity timer = 80 ms On-duration = 10 ms |

The above assumption can also be used for network energy saving SI as a baseline. As mentioned above, however, the network energy consumption will very much depend on the RU during the transmission. Therefore, the above traffic should be used in the simulation such that it results in the agreed RU assumption. This can be done, e.g., by setting up the number of users. If needed, a modification on the packet data size or the data inter-arrival time may also be used.

The traffic models and DRX setup of the UE power-saving WI may be used as a reference. The overall traffic (from the served UEs) shall result in the agreed RU assumption.

## KPIs

While the focus of this study is to obtain mechanism which supports network energy saving, it shall be understood that the proposed mechanisms can also impact KPIs other than network energy to some extent. Therefore, besides of the energy saving gain, the impact to the other KPI caused by the proposed mechanism should be also be considered. Furthermore, we should consider that while some KPIs such as cell level throughput may be impacted, at the end what is important is that the UE is happy with the experience, and thus when it comes to specific KPIs, the emphasis can be more on UE level KPIs. Therefore, for network energy saving, the following may be considered as the KPI to be reported in the evaluation:

Energy saving gain

The network energy saving schemes are generally proposed to reduce the overall energy consumption. Considering this, the evaluation in the study shall be conducted in the sense of average power consumption in a duration (e.g., at least a duration corresponds to several FTP sessions) instead of reducing the instantaneous power consumption. For example, a transmission with full PRBs in one time instance may consume more power compared to a transmission with lower PRB occupancy. However, for a given packet size, transmission with full PRB may require a shorter time to complete the transmission. Hence, although the instantaneous power consumption is higher, the total energy consumption might be lower by using the full PRB transmission due to the shorter transmission time.

Evaluation of the energy saving gain should consider overall network energy usage for performing a certain operation (e.g., equal to several FTP sessions) by considering the per symbol/per slot power consumption and transition energy values from the gNB power model.

Network energy saving may be achieved, e.g., by increasing the sleep time of the network and/or by deploying smaller resources for the transmission (e.g., smaller number of antenna or ports, smaller BW, etc.). Intuitively, this mechanism will directly impact the network performance on the throughput and/or latency. Considering this, beside of the energy saving gain, the system level simulation(SLS) should also capture the average UPT loss and/or additional latency. Note that for certain schemes, e.g., the scheme related to the initial access improvement, instead of SLS, a pen and paper or qualitative analysis may be sufficient.

SLS should also capture the UPT impact and/or additional latency of the network energy saving schemes.

Due to the energy-saving schemes employed in the network, it is possible that there is impact on UE performance and hence the impact on UE should also be considered. For example, the power consumption may increase, the throughput may decrease, and the latency may also increase. Therefore, the network energy saving should give a minimum impact to the UE performance. In other words, to justify the worthiness of the proposed energy saving techniques, the KPI (e.g., the power consumption, throughput, and latency) evaluation from the UE perspective may also be added.

To give a proper consideration to evaluate network energy saving techniques, KPI on the UE side is also considered.

Multiple cells may be used in the SLS. In some cases, it is possible that the reduction on the energy consumption of a certain cell brings an impact to the increase of average energy consumption in other cells. Considering this, the average energy consumption should at least be the average energy consumption of all cells in the simulation. For a certain case, it is possible that information on the average energy consumption of each (or a certain) cell may be worth to be included. Thus, this information may also optionally be added if needed. In addition, other statistical values, e.g., percentile, CDF, etc., may also be added if needed. Note that besides of the energy consumption, the same approach may also be taken for other KPIs, e.g., throughput loss, additional latency, etc.

At least the average value across multiple cells can be considered for the qualitative analysis via SLS. Average values of each cell and other statistics may also be added if needed.

In addition to the above, several other KPIs may also be considered. For example, the resource overhead, number of delivered data per energy, spectral efficiency, cell-drop rate, successful handover rate, etc. It should be noted, however, those KPIs may be very use case specific. For example, not all of proposed schemes requires handover to see the advantage or drawbacks. In another example, the spectral efficiency KPI may be more suitable for the high load traffic case which is not the focus of this SI. Considering this, other KPIs than the above shall be optional. In addition, for these KPIs, pen and paper analysis or qualitative analysis may be sufficient.

In addition to the KPIs mentioned above, one popular KPI is energy efficiency. This KPI is quite vague and has different definitions from consumed energy per maximum capacity to consumed energy per delivered bits and so on. In case, this is also a KPI to study, first we should find a definition which reflects the intention of the SI, e.g., following ETSI definitions it can be defined as consumed energy per delivered bit. And next, we should note that temporary high energy efficiency is not a good measure for evaluating the network energy consumption, and thus it should be evaluated over a longer time, e.g., 24 hours.

# Conclusion

In previous sections, the following observations and proposals were made:

Observation 1	In typical TDD AAS scenario, UL and DL implementations are entangled and thus, it is preferred to model them together. If there is something to transmit or receive in a symbol, the gNB can not go to sleep.

Proposal 1	The gNB power model should ensure detailed support for low-load and “idle” mode modeling, and be applicable to at least AAS-type gNB designs reflecting current technology trends and evolutions in a few years horizon.

Proposal 2	Develop a gNB power model structurally similar to the UE model in TR38.840 that includes power and/or energy levels of relevant operations, sleep states and transitions, as well as scaling for TX and RX BW and the number of active antenna ports.

Proposal 3	The gNB power model should include at least following states:  active transmission (100% PRB operation),  active reception (100%PRB), micro sleep, light sleep, deep sleep, and hibernate sleep.

Proposal 4	The gNB power model should be expressed in relative units, related to the deep sleep power level.

Proposal 5	The gNB power model should include at least transition energy between different gNB sleep states and active transmission/reception state.

Proposal 6	The gNB power model should consider the scaling of relative power levels at least for active transmission/reception and at least for BW, CA, and antenna ports adaptations.

a.	FFS: If and how the maximum output power scales the active transmission power consumption.

Proposal 7	Table A2.1-1 of TR 38.802 can be used as the baseline for system level simulation assumptions. Other assumptions are not precluded.

Proposal 8	At least an average resource utilization of 10% and 30%, is included for the network energy-saving evaluations. Other values are not precluded.

Proposal 9	The traffic models and DRX setup of the UE power-saving WI may be used as a reference. The overall traffic (from the served UEs) shall result in the agreed RU assumption.

Proposal 10	Evaluation of the energy saving gain should consider overall network energy usage for performing a certain operation (e.g., equal to several FTP sessions) by considering the per symbol/per slot power consumption and transition energy values from the gNB power model.

Proposal 11	SLS should also capture the UPT impact and/or additional latency of the network energy saving schemes.

Proposal 12	To give a proper consideration to evaluate network energy saving techniques, KPI on the UE side is also considered.

Proposal 13	At least the average value across multiple cells can be considered for the qualitative analysis via SLS. Average values of each cell and other statistics may also be added if needed.

# References

B. Debaillie, C. Desset and F. Louagie, "A Flexible and Future-Proof Power Model for Cellular Base Stations," 2015 IEEE 81st Vehicular Technology Conference (VTC Spring), 2015, pp. 1-7, doi: 10.1109/VTCSpring.2015.7145603.

3GPP TR 38.840, 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Study on User Equipment (UE) power saving in NR (Release 16)

P. Lähdekorpi, M. Hronec, P. Jolma and J. Moilanen, "Energy efficiency of 5G mobile networks with base station sleep modes," 2017 IEEE Conference on Standards for Communications and Networking (CSCN), 2017, pp. 163-168, doi: 10.1109/CSCN.2017.8088616.

R1-2009202, High-level UE energy consumption profiling, Ericsson, RAN1#103-e, Oct-Nov 2020.

S. Tombaz et al. “Energy Performance of 5G-NX Wireless Access Utilizing Massive Beamforming and an Ultra-Lean System Design”, IEEE Globecom, San Diego, CA, USA, 2015.

R1-1611900, “Long Network DTX in NR”, Ericsson, 3GPP TSG-RAN WG1 Meeting #87, Nov 2016.

RP-220297, 3GPP TSG RAN Meeting #95e, Electronic Meeting, March 17-23, 2022; Revised SI: Study on network energy savings for NR, Huawei

- ANNEX (BS power model from reference [1])

Base station power model which has been used in the past discussions from reference [1].

Figure 3 Power consumption as the system deactivates and enters successive sleep modes