3GPP TSG-RAN WG1 #109e	Tdoc R1-2204883

Electronic meeting, May 9 – May 20, 2022

Agenda Item:	9.7.3

Source:	Ericsson

Title:	Other aspects related to network energy saving

Document for:	Discussion, Decision

# 1	Introduction

In RAN#94, it was agreed to start a Rel 18 SI on network energy savings (latest WID in [1]), with the following objectives related to the techniques for improving network energy savings.

- Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include:

- How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2]

- Information exchange/coordination over network interfaces [RAN3]

Note: Other techniques are not precluded

In our RAN#93 contributions with potential SI on network energy saving and corresponding motivation document [2][3], the following aspects were described, particularly more detailed examples related to UE assistance (relative to the final SI in [1]).

- Study NW energy saving techniques in frequency domain e.g., CA/DC adaptation using additional information such as UE assistance information.

- Study and identify UE assistance information helping NW energy saving e.g., UE traffic type, priority, time criticality, expected volume, coverage, mobility status, location.

This contribution discusses the benefits of using UE assistance information for network energy saving which is also in-line with the Rel 18 network energy savings SID objective [2]. NR is designed to be lean, nevertheless, since the gNB typically does not have much information about the UE, e.g., its traffic type, time criticality and so on, it may need to keep the transceivers ON most of the time even if not necessary and thus cannot go to deeper sleep modes than micro sleep to gain meaningful energy savings.

# 2	Discussion

There are diverse types of UE assistance information that the UE can provide to help network energy savings. Besides the UE assistance information examples listed in [2][3], there are other candidates for the UE assistance information that could be useful not only for the network energy savings but also for the UE power savings as we will discuss later. These UE assistance information candidates include the UE type (e.g., smart vehicle, smartphone, healthcare, industrial, Redcap, or environmental IoT device), the UE battery specifications such as capacity, type of battery, charge/discharge rate, and the remaining life cycle. It is important to note that due to their intrinsic nature, diverse types of information could have different validity periods, and thus they may need to be provided to the network with different frequentness. On the one hand, information such as the UE type and the UE battery specifications have a long-term validity period and hence, they can be provided to the network seldomly. Consequently, providing this type of information could be beneficial for the overall system without being a burden for the UE in terms of signalling. On the other hand, information such as the ones indicated in Objective 2 may be both, long-term and short-term valid depending on the particular scenario.

Providing the UE assistance information for network energy savings does not have to be a burden for the UE in terms of signalling.

Such UE assistance information could also be useful to save UE power. For instance, with the current available UE assistance information, in periods of low traffic the UE may inform the network that it prefers to have a smaller bandwidth, the network may then decide to change the active BWP of one or more serving cells to a smaller bandwidth, which should save power for both the UE and the network, particularly if the traffic is low, and thus a higher BW may be redundant as the data arrivals are sparse and UL can also be configured to be narrow BW. Similarly, if the UE informs its preferred number of active SCells, power could be saved for both the UE and the network if SCells that are not required in a particular moment are deactivated. Additionally, the UE can also inform the network explicitly of which SCells to be deactivated, and thus the network can make a better decision and save additional power. E.g., if the UE rarely use an FR2 SCell, the network can deactivate that SCell and go to deep sleep mode or even totally turn that cell off to save significant power.

Another useful UE assistance information is for the network to become aware of the UE time criticality, for example if a UE can tolerate delay of 20ms or higher, then the network can collect all the PRBs to be transmitted in one stack, and transmit them and then go to sleep until the next scheduling instance. Here, for example, the network can align the transmissions with SSB in order to able to sleep in between two SSB transmissions.

Further mechanisms of UE assistance information can be studied which could result in relaxed scheduling requirements for the network and may also trigger network actions that could save UE power e.g., SCG deactivation, relaxed RLM, CA reconfiguration. Another assistance which can also motivate the network to configure more UE power saving features is for the UE to report if it uses specific power saving functionalities, e.g., PEI.

UE assistance information can be beneficial for network energy savings.

Network actions based on UE assistance information related to network energy saving may also save UE power.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Providing the UE assistance information for network energy savings does not have to be a burden for the UE in terms of signalling.

Observation 2	UE assistance information can be beneficial for network energy savings.

Observation 3	Network actions based on UE assistance information related to network energy saving may also save UE power.

# References

RP-220297, 3GPP TSG RAN Meeting #95e, Electronic Meeting, March 17-23, 2022; Revised SI: Study on network energy savings for NR, Huawei

RP-212423, Proposal for New SID on NR Network Energy Saving, Ericsson, RAN#93-e, September 2021

RP-212422, Motivation for Network Energy Saving in Rel-18, Ericsson, RAN#93-e, September 2021