3GPP TSG-RAN WG1 #109-e		R1-2204887

eMeeting, May 9- 20, 2022

Source:	Ericsson

Title:	Other aspects related to DSS enhancements

Agenda Item:	9.9.3

Document for:	Discussion and Decision

# Introduction

In this document, we discuss additional enhancement to improve DSS performance.

This document is a revised version of R1-2112156 submitted to RAN1#107-e with updates reflecting current RAN/RAN1 discussions.

# Discussion

## CRS Rate-matching enhancement

RAN1 is currently specifying UE support for overlapping CRS rate matching patterns in Rel18 [1]. This enables configuration of two rate-matching patterns for the UE within same carrier e.g., one corresponding to serving cell and one corresponding to neighbor cell. This can be extended further to introduce dynamic adaptation of the configured patterns to better match the pattern of strongest interferer.

Figure 1. Impact of CRS interference level and potential gains from rate-matching around it

Figure 1 shows UE throughput for different cases of CRS rate-matching around a neighbor cell interferer.

- ‘baseline’ case in the figure is UE throughput without inter-cell CRS interference (though with rate-matching around the own CRS), i.e. the performance that would be expected with colliding CRS between cells.

- ‘ratematched CRS RE’ corresponds to RE level rate-matching around the interferer CRS REs only

- ‘ratematched symbol’ corresponds to rate-matching around the full symbols with CRS interference.

- SIR levels in the legend refer to cases with CRS interference with different signal to CRS interference levels.

As illustrated in the figure, CRS interference limits performance especially when noise and interference from other sources is low (medium/high SNR in the plot). For such cases, rate-matching around neighbor cell CRS REs is useful to improve performance. The gain depends on the strength of the interferer, with larger gains for stronger interferer (lower SIR in the figure). For the simulated scenario, RE-level rate-matching provides around ~20% throughput gain compared to symbol-level rate-matching. The gain can vary based on total available PDSCH REs.

Currently, the CRS rate-matching pattern can only be configured via RRC which limits flexibility compared to symbol/PRB level rate-matching that can be adapted by DCI based L1 signaling. The level of CRS interference and the strongest interferer itself (i.e., v-shift) can change dynamically. Considering this, supporting dynamic adaptation between different CRS rate-matching patterns via DCI would be beneficial as it provides fast adaptation without need for frequent RRC signaling to turn on/off rate-matching or to match the pattern of strongest interferer.

It should be noted that existing L1/L3 CSI/RRM measurements framework can used to estimate the appropriate CRS-RM pattern without need to introduce new measurements.

# References

- RP-213575 - “New WI: Enhancement of NR Dynamic spectrum sharing (DSS)”, Ericsson, RAN#94-e, Dec 2021.

# Annex A: Simulation assumptions

## Link level

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2 GHz |
| SCS | 15 kHz |
| Bandwidth | 20 MHz |
| Channel model | TDL-A |
| Delay spread | 100 ns |
| UE speed | 3 km/h |
| Correlation | Medium |
| Symbols reserved for PDCCH | Symbol 0 and 1 reserved for LTE PDCCH, symbol 2 reserved for NR PDCCH |
| Number of BS antennas | 4 Tx (Two cross-polarized antenna pairs) |
| Number of UE antennas | 2 Rx (One cross-polarized antenna pair) |
| Channel estimation | Practical |
| Receiver type | MRC |
| DMRS | Type A, 1+1 symbol, no FDM with data |
| CRS | 4 port CRS |
| DCI payload (excluding CRC) | 55 bits |
| Interleaving | Interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 PRBS |

## System level

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2 GHz |
| SCS | 15 kHz |
| Simulation bandwidth | 20 MHz |
| BS antenna height | 25 m |
| UE height | 1.5m |
| TRP transmit power | 46 dBm for 10MHz |
| Scenario | Urban Macro |
| ISD | 500m |
| TRP antenna configuration | (M,N,P,Mg,Ng;Mp,Np)= (2,8,2,1,1;1,1) |
| UE antenna configuration | (M,N,P,Mg,Ng;Mp,Np)= (1,1,2,1,1;1,1) |
| Device deployment | 80% indoor, 20% outdoor |
| UE speeds | Indoor users: 3km/h |
|  | Outdoor users (in-car): 30 km/h |
| BS noise figure | 5 dB |
| BS antenna element gain | 8 dBi |
| UE noise figure | 9 dB |
| Thermal noise level | -174 dBm/Hz |
| Traffic | Full Buffer |
| Macro sites | 19 |
| Downtilt | 102° |
| Minimum BS to UE distance | 35m |