3GPP TSG RAN WG1 #109-e	R1-2204888

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.10.1

Source:	Ericsson

Title:	Multi-cell PUSCH/PDSCH scheduling with a single DCI

Document for:	Discussion and Decision

# Introduction

In RAN#94, it was agreed to start a Rel 18 WI on multi-carrier enhancements, with the following objectives for multi-cell PUSCH/PDSCH scheduling (one PDSCH/PUSCH per cell) with a single DCI.

Specify a solution for multi-cell PUSCH/PDSCH scheduling (one PDSCH/PUSCH per cell) with a single DCI [RAN1]

- Identify the maximum number of cells that can be scheduled simultaneously

- Consider both intra-band and inter-band CA operation

- Consider both FR1 and FR2

- The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling

In this paper, we discuss the various aspects to consider in specifying this feature.

# Discussion

In NR releases until Rel-17, a single DCI can schedule PUSCH/PDSCH on one cell and single DCI can schedule multiple PUSCH/PDSCH on single cell. The new Rel-18 WI objective is to specify a feature where a single DCI can be used for multi-cell PUSCH/PDSCH scheduling (one PUSCH/PDSCH per cell).

## High-level design aspects

In current Rel-15/16 framework, a PDCCH can schedule PDSCH on only one cell using DCI formats 1-0/1-1/1-2. In case of self-carrier scheduling, the PDCCH and PUSCH/PDSCH are received on the same serving cell. For cross-carrier scheduling, a PDCCH is received on a scheduling cell (cell A) and the corresponding PUSCH/PDSCH may be received on a scheduled cell which may be same as the scheduling cell (cell A) or on another cell (cell B).

For single DCI scheduling PUSCH/PDSCH on multiple cells, one PDCCH DCI format on the scheduling cell (e.g. cell A) has to schedule a PUSCH/PDSCH e.g. for the scheduling cell (cell A) and also one other PUSCH/PDSCH for another scheduled cell (cell B). As stated in the WID justification, trade-off between overhead saving and scheduling restriction must be taken into account.

CA is a native feature of NR supported from Rel15 and it is used for aggregating carriers with a wide variety of attributes depending on the specific deployment scenario. This flexibility provided by CA should be retained even when single DCI scheduling PUSCH/PDSCH on multiple cells (or mc-DCI) is used.

### Maximum number of cells scheduled by mc-DCI

The first main issue to address is following from WI objective:

- Identify the maximum number of cells that can be scheduled simultaneously.

The maximum number of NR carriers for CA and DC is 16 from RAN1 specification perspective. Thus, in principle, the mc-DCI could potentially schedule a maximum of 16 cells.

For a single DCI scheduling single cell (e.g. DCI format 1_1), take an example typical DCI payload of about 56 information bits, which along with a 24-bit CRC gives a DCI size of 80 bits. Assuming no compression of DCI payload (for sake of illustration), the estimated DCI size is as shown below.

Table 1. DCI size comparison for different number of cells

| Maximum number of cells in mc-DCI | mc-DCI size (assuming no compression of DCI payload) | Compression relative to one DCI scheduling one cell |
| --- | --- | --- |
| 1 | 80 | 1 |
| 3 | 192 | 2 |
| 4 | 248 | 3 |
| 8 | 472 | 6 |
| 16 | 920 | 12 |

The maximum NR DCI size is 164 bits. It is clear from the table that some choices (such as 16) would need a huge amount of compression. This is in principle possible, but it will lead to excessive scheduling restrictions (e.g. in TDRA/FDRA fields, etc). On the other hand, the WID also says the design should be optimized for 3 or more cells. Of the various scenarios of interest, one is intra-band FR2 with four or eight carriers. Thus, 4 or 8 are good alternatives to consider further for mc-DCI design.

- Maximum number of cells scheduled by a mc-DCI is selected from {4,8}.

### SCS combinations scheduled by mc-DCI

Like the number of cells, the SCSes scheduled by a mc-DCI can also influence the DCI format design, and more importantly how such DCI will be used in typical scheduling. The number of SCSes increased to 7 in Rel-17 (considering FR2-2). Even if mc-DCI schedules a maximum of 4 or 8 cells, typically, the processing timeline of control/data for different numerologies can be widely varying, and hence mixing too many SCSes in to a single mc-DCI can increase complexity. Therefore, our proposal is as follows.

- At least the case where all PUSCH/PDSCH scheduled by a mc-DCI have same SCS is supported.

DCI design for case where PUSCH/PDSCH scheduled by a mc-DCI have two different SCS can be considered next.

### Scheduled/scheduling cells for mc-DCI

One more issue to address is whether the scheduling cell containing an mc-DCI can also be one of the cells that are scheduled by the scheduled cells or not (see figure 1), i.e. below alternatives.

- Alt 1: mc-DCI on a scheduling cell can be used to schedule PUSCH/PDSCH on that scheduling cell.

- Alt 2: mc-DCI on a scheduling cell cannot be used to schedule PUSCH/PDSCH on that scheduling cell.

A main scenario where an mc-DCI is beneficial is in the intra-band case and in such cases, it is natural to benefit from the PDCCH overhead reduction by using mc-DCI to schedule PUSCH/PDSCH on the scheduling cell also. Alt 2 reduces opportunity for PDCCH overhead reduction. Therefore, Alt 1 is preferred.

- mc-DCI on a scheduling cell can be used to schedule PUSCH/PDSCH on that scheduling cell, at least when all cells have same SCS.

Figure 1. Illustration of two alternatives : Alt 1 scheduling cell can be scheduled by mc-DCI and Alt 2 : scheduling cell cannot be scheduled by mc-DCI.

The mc-DCI design should allow sufficient scheduling flexibility, considering that not all cells will be always scheduled by the mc-DCI at all instances (e.g. retransmissions depending on HARQ feedback, CSI reporting, etc). It is also beneficial to also support individual scheduled cells using existing single cell DCI format as it provides the full flexibility/feature support in scheduling (i.e. without compressed DCI fields like in mc-DCI). When UE monitors the DCI format for single DCI scheduling PUSCH/PDSCH on multiple cells, the UE can be configured to also monitor existing DCI format(s) scheduling PUSCH/PDSCH on single cell (i.e. 1-0/1-1/1-2).

- When mc-DCI is configured for scheduling PUSCH/PDSCH on multiple cells, a mc-DCI can schedule PUSCH/PDSCH on all of the cells or a subset of those cell (including single cell).

- When mc-DCI is configured for scheduling PUSCH/PDSCH on multiple cells, for each of those cells, UE can also be configured to monitor existing single cell DCI format(s) scheduling PUSCH/PDSCH (i.e. 1_1/1_2/0_1/0_2).

### DCI size handling and budgets

Next, we discuss DCI size handling and budgets. For a scheduled cell, UE is not expected to handle more than 4 DCI sizes for cell and more than 3 DCI sizes with a C-RNTI. The DCI size alignment procedure can become quite messy (7.3.1.0 of 38.212) and hence it is preferred if the size alignment procedure can be kept simple for mc-DCI. Like other DCI formats (such as 2_0, 2_6, 2_7, etc), explicit configuration of the mc-DCI size can make the specification simple (e.g. gNB can simply by configuration ensure that the 4 + 3(C-RNTI)) budget is maintained when UE is configured with mc-DCI. This can also help when the scheduling cell schedules more than one group of cells.

mc-DCI configuration for uplink and downlink should also be addressed. Typically, the number of aggregated carriers can be different between downlink and uplink. Moreover, it is possible (and perhaps more likely) that the DCI size for downlink and uplink is different. In such a case, it is preferable to keep independent the configuration of mc-DCI for PUSCH and PDSCH as well as to configure the BDs for those separately. For example, if UE is configured with four cells in downlink and only two cells (or even one cell) in the uplink, then a mc-DCI for downlink makes sense, but there may be no need to use mc-DCI for the uplink or perhaps assign separate BDs for mc-DCI for uplink and downlink. Based on above, the below three proposals are made.

- When mc-DCI is configured for scheduling PUSCH/PDSCH on multiple cells, existing Rel-17 DCI size budget is maintained for each scheduled cell.

- Size of mc-DCI is explicitly configured by higher layers.

- Support independent configuration of mc-DCI for PUSCH and PDSCH.

### mc-DCI format design

Below we list some aspects that need to be considered for mc-DCI. We note the high-level aspects (as discussed in previous sections) would influence the detailed design of mc-DCI. Hence some progress on high-level aspects is preferable before starting detailed mc-DCI design. Nonetheless, some principles for mc-DCI design and simplifications (for certain fields such as UL-related information for PDSCH) can be considered until then.

It needs to be discussed whether mc-DCI format supports the functionality of all the DCI fields specified for existing DCI formats (i.e. 1_1/1_2/0_1/0_2) or whether it supports only a limited subset of DCI fields.

For each DCI field of the mc-DCI format, at least there can be following option(s):

- DCI field jointly indicates functionalities for all the PUSCHs/PDSCHs

  - E.g. carrier selection information, etc

- Separate DCI fields are used for each PUSCH/PDSCH

  - E.g. FDRA, etc

For PDSCH, certain other fields can be common such as TPC command for PUCCH, PUCCH resource indicator, PDSCH-to-HARQ ACK timing, and DAI fields – in principle this assumes that there is a common feedback timing for both PDSCHs scheduled by the mc-DCI, which is not necessarily true in all cases (e.g. URLLC), but is reasonable for eMBB scenario. For URLLC use case, it makes more sense to rely on single cell DCI formats such as 1_2/0_2.

- For mc-DCI scheduling PDSCH on multiple cells, at least the following fields are common for the multiple scheduled PDSCHs

  - Downlink assignment index

  - TPC command for scheduled PUCCH

  - PUCCH resource indicator

  - PDSCH-to-HARQ-feedback timing indicator

# Conclusion

In previous sections, the following observations and proposals were made:

Proposal 1	Maximum number of cells scheduled by a mc-DCI is selected from {4,8}.

Proposal 2	At least the case where all PUSCH/PDSCH scheduled by a mc-DCI have same SCS is supported.

Proposal 3	mc-DCI on a scheduling cell can be used to schedule PUSCH/PDSCH on that scheduling cell, at least when all cells have same SCS.

Proposal 4	When mc-DCI is configured for scheduling PUSCH/PDSCH on multiple cells, a mc-DCI can schedule PUSCH/PDSCH on all of the cells or a subset of those cell (including single cell).

Proposal 5	When mc-DCI is configured for scheduling PUSCH/PDSCH on multiple cells, for each of those cells, UE can also be configured to monitor existing single cell DCI format(s) scheduling PUSCH/PDSCH (i.e. 1_1/1_2/0_1/0_2).

Proposal 6	When mc-DCI is configured for scheduling PUSCH/PDSCH on multiple cells, existing Rel-17 DCI size budget is maintained for each scheduled cell.

Proposal 7	Size of mc-DCI is explicitly configured by higher layers.

Proposal 8	Support independent configuration of mc-DCI for PUSCH and PDSCH.

Proposal 9	For mc-DCI scheduling PDSCH on multiple cells, at least the following fields are common for the multiple scheduled PDSCHs

a.	Downlink assignment index

b.	TPC command for scheduled PUCCH

c.	PUCCH resource indicator

d.	PDSCH-to-HARQ-feedback timing indicator

# References

RP-213577, New WID on Multi-carrier enhancements, NTT DOCOMO, RAN#94-e, December 2021

R1-1805782,  Reply LS on maximum number of CCs for NR CA, RAN1, RAN1#92-bis, Sanya(CN), April 2018