3GPP TSG-RAN WG1 Meeting #109-e	R1- 2204889

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.10.2

Source:	Ericsson

Title:	Multi-carrier UL Tx switching

Document for:	Discussion, Decision

# Introduction

In RAN#94, it was agreed to start a Rel-18 WI on multi-carrier enhancements with the following objective on enhancements of UL Tx switching [1]:

2. Study and if necessary specify following enhancements for multi-carrier UL operation [RAN1, RAN2, RAN4] - UL Tx switching schemes across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs, including mechanisms to enable more configured UL bands than its simultaneous transmission capability and to support dynamic Tx carrier switching across the configured bands for both single TAG and multiple TAGs configurations (RAN1, RAN4) - UE capability and RRC configuration related signalling (RAN2) - Note: strive for RAN1/2 design agnostic with the number of bands, i.e., common design between 3 and 4 bands - Note: no additional TAG is introduced for UL transmission on a carrier without corresponding DL carrier - Note: this objective does not target to extend the SUL framework to support more than 1 SUL for 1 NUL - Switching time and other RF aspects, and RRM requirements for above UL Tx switching schemes across up to 3 or 4 bands (RAN4) - Note: Prioritize UL Tx switching across up to 3 bands is to be addressed first and then that for up to 4 bands can also be addressed

In this paper, we discuss our view regarding the second objective that focuses on the potential enhancements for UL Tx switching operation.

# Discussion

TX switching was proposed originally for two carriers, SUL in one band and TDD transmissions using two TX connectors in another band for devices only supporting 2TX chains, but subsequently also included EN-DC and UL CA. For all these cases the carriers are assumed to be co-located belonging to the same TAG. Two cases for UL switching were supported in Rel-16 with an extension to three cases in Rel-17. More information on the corresponding scenarios is provided in Appendix.

Table 1: Rel-17 dynamic UL Tx switching cases

|  | Number of Tx chains (band A + band B) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

Furthermore, two options for dynamic UL Tx switching for inter-band UL CA for case 1 are supported:

- Option 1 (uplinkTxSwitchingOption = switchedUL): For case 1, simultaneous transmission on both carriers is not supported,

- Option 2 (uplinkTxSwitchingOption = dual UL): For case 1, simultaneous transmission on both carriers is supported.

In the following, first we start the discussion by sharing our view on the extension of UL Tx switching for multiple-TAG in case of UL CA including 2 bands and continue with the extension of UL Tx switching feature to 3 or 4 bands.

## Extension to multiple-TAG

By the current UL TX switching Rel-17 feature, i.e. switching across two TX chains or switching pairs of TX chains, combinations of UL-MIMO features and UL CA can be supported and tested for co-located scenarios with the single-TAG assumption. However, it is not possible for non-co-located deployments requiring support of multiple-TAG. CA band combinations with cells in bands below 2 GHz and cells in bands around 3.5 GHz are common and non-colocation therefore a common scenario due to the different cell sizes used in these bands. Therefore, it is important to extend the deployment scenarios for UE TX switching to include non-co-located scenarios for UL CA with multiple-TAG. This would represent a most useful enhancement of the UE TX switching feature.

The UL TX switching feature for UL CA is currently supported for co-located scenarios with the single-TAG assumption.

Non-co-located scenarios for UL CA with multiple-TAG are common deployment scenarios in particular for CA band combinations with cells in bands below 2 GHz and cells in bands around 3.5 GHz.

Extending the deployment scenarios for UE TX switching to non-co-located scenarios for UL CA with multiple-TAG would be a useful enhancement of the UL TX switching feature.

We have provided an assessment of the expected specification efforts to extend the functionality to multi-TAG even in Rel-17, see e.g. [2]. In general, we discuss that the UL timing on a carrier is derived from the DL of the same carrier according to 38.211 and 38.133. This includes the effect of timing advance for each carrier (i.e. also for the co-located case).

From RAN1 perspective, for uplink switching the following is currently specified in 38.214:

If an uplink switching is triggered for an uplink transmission starting at T0, after T0-Toffset, the UE is not expected to cancel the uplink switching, or to trigger any other new uplink switching occurring before T0 for any other uplink transmission that is scheduled after T0-Toffset, where Toffset is the UE processing procedure time defined for the uplink transmission triggering the switch given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213].

That implies that when the gNB schedules or configures a UL transmission, it should accommodate the Toffset required including any effect of timing differences between the switched carriers. In the general case, the processing time includes the effect of timing advance and the timing difference between component carriers as given by 38.133 (the MRTD and MTTD requirements) when the UE is configured with multiple active component carriers. The switching period  (Tswitch ) is added to this as specified in 38.214.

From RAN4 perspective, the changes are limited to lifting the restriction to single-TAG operation only for the time masks specified for TX switching. Modified time masks are added for UEs supporting supportedNumberTAG. For UEs supporting supportedNumberTAG and a switching band pair, the switching period during which the UE is not expected to transmit on any of the two carriers would precede the time T0 (the start of the uplink transmission) just as for the single-TAG case; in the field the gNB scheduler would ensure that there are no transmissions in the switching period by not scheduling the last UL symbol of the trailing carrier (of smaller TA value) before the switching gap, a similar action by the test system in conformance testing.

The timing aspects are illustrated in Figure 1 where the upper part of Figure 1 shows the existing single-TAG case when the switching period is taken in carrier 1 (e.g. in a low band), the timing advance NTA,1 of carrier 1 is the same as NTA,2 of carrier 2 (e.g. in a 3.5 GHz band). The UE is not expected to transit in the shaded symbols, and the time T0 is always at the start of a slot of carrier 2 when the switching period is taken in carrier 1.

The lower part of Figure 1 shows the non-collocated case with different timing advance on the two carriers, NTA,1 > NTA,2, carrier 2 is trailing carrier 1. Then the gNB would blank the last symbol of carrier 2 that overlaps with the switching period in carrier 1. The maximum delay is one symbol at SCS = 30k, corresponding to the maximum receive timing difference (MRTD) for CA.

We remark that timing differences between the two UL carriers may occur also for the collocated case with a single TAG. The timing of a transmission at T0 at an UL carrier is determined by the corresponding DL carrier; hence there may be a timing shift between the two UL carriers due to DL timing errors in addition to other UL timing errors.

The timing requirements on T0 – Toffset w r t the received DCI specified in 38.214 are not modified. Moreover, the MRTD for CA would not imply changes of the DL interruptions allowed. This asserts that the multiple-TAG case can be accommodated by modifying the time masks in the RAN4 specifications alone.

Therefore, regarding UL TX switching with 2 bands in our view specification efforts are minimum and involve a change of the time masks used for conformance testing with a timing offset between the UL carriers. The modified masks are subject to capability, multiple TAG requires support of supportedNumberTAG with at least two TAGs irrespective of UL TX switching.

Minor specification impact is expected to extend the support of UL TX switching across 2 bands for UL CA with multiple-TAG.

Therefore, since UL TX switching for 2 bands is already supported, we propose to lift the single-TAG restriction for UL CA.

Support UL TX switching across 2 bands for UL CA with multiple-TAG.

Moreover, we in our view any potential extension of the feature to 3 or 4 bands should be applicable to both single and multiple-TAG. In fact, with more bands for UL CA, it is more realistic to have deployments with multiple-TAG as previously discussed. Therefore, the study should not prioritize one co-located scenarios versus non-co-located scenarios.  Therefore, we propose to consider a unified framework for UL CA during the study and potential specification and refrain from any TAG restriction for UL CA band combinations.

Study on extension of ULTX switching to 3 or 4 bands should refrain from any TAG restriction for UL CA band combinations.

Figure 1: co-located case (upper) and the non-collocated case (lower) with carrier 2 lagging in time as seen at the UE, the last symbol of carrier 2 blanked.

## UL Tx switching for 3 or 4 bands

Extension of UL TX switching to more bands provide more flexibility. However, proper understanding of the scenarios and switching combinations is crucial to perform a realistic assessment. We start with listing the possible cases for 3 and 4 bands in Table 2 and Table 3 respectively. In each table, maximum number of Tx chains per band is shown, together with number of antenna ports with different options.

Table 2: Cases of number of Tx chains across 3 bands (A+B+C) for UL Tx switching

|  | Number of Tx chains | Number of Antenna ports (Option 1) | Number of Antenna ports (Option 2) |
| --- | --- | --- | --- |
| Case 1 | 1T+1T+0T | - | 1P+1P+0P 0P+1P+0P 1P+0P+0P |
| Case 2 | 0T+2T+0T | 0P+2P+0P 0P+1P+0P | 0P+2P+0P 0P+1P+0P |
| Case 3 | 2T+0T+0T | 2P+0P+0P 1P+0P+0P | 2P+0P+0P 1P+0P+0P |
| Case 4 | 1T+0T+1T | - | 1P+0P+1P 1P+0P+0P 0P+0P+1P |
| Case 5 | 0T+1T+1T | - | 0P+1P+1P 0P+1P+0P 0P+0P+1P |
| Case 6 | 0T+0T+2T | 0P+0P+2P 0P+0P+1P | 0P+0P+2P 0P+0P+1P |

Table 3: Cases of number of Tx chains across 4 bands (A+B+C+D) for UL Tx switching

|  | Number of Tx chains | Number of Antenna ports (Option 1) | Number of Antenna ports (Option 2) |
| --- | --- | --- | --- |
| Case 1 | 1T+1T+0T+0T | - | 1P+1P+0P+0P 0P+1P+0P+0P 1P+0P+0P+0P |
| Case 2 | 0T+2T+0T+0T | 0P+2P+0P+0P 0P+1P+0P+0P | 0P+2P+0P+0P 0P+1P+0P+0P |
| Case 3 | 2T+0T+0T+0T | 2P+0P+0P+0P 1P+0P+0P+0P | 2P+0P+0P+0P 1P+0P+0P+0P |
| Case 4 | 1T+0T+1T+0T | - | 1P+0P+1P+0P 1P+0P+0P+0P 0P+0P+1P+0P |
| Case 5 | 0T+1T+1T+0T | - | 0P+1P+1P+0P 0P+1P+0P+0P 0P+0P+1P+0P |
| Case 6 | 0T+0T+2T+0T | 0P+0P+2P+0P 0P+0P+1P+0P | 0P+0P+2P+0P 0P+0P+1P+0P |
| Case 7 | 1T+0T+0T+1T | - | 1P+0P+0P+1P 1P+0P+0P+0P 0P+0P+0P+1P |
| Case 8 | 0T+1T+0T+1T | - | 0P+1P+0P+1P 0P+1P+0P+0P 0P+0P+0P+1P |
| Case 9 | 0T+0T+1T+1T | - | 0P+0P+1P+1P 0P+0P+1P+0P 0P+0P+0P+1P |
| Case 10 | 0T+0T+0T+2T | 0P+1P+0P+2P 0P+0P+0P+1P | 0P+1P+0P+2P 0P+0P+0P+1P |

There are ambiguity cases the UE transmits on one antenna port in a same band. The color-codes entries show the ambiguity cases where the same color mark different cases that transmission is done similarly on the same antenna port in band regardless of switch.

Another observation is the large number of cases for 3, and specially 4 bands. It is important to study whether all these cases should be considered for study or only a subset of them. From our perspective, it is important to focus on few promising cases for study based on some practical assumptions.

From our perspective a useful UL CA scenario is a low band combined with non-contiguous CA (two carriers) in mid-band where the low-band can belong to a different TAG [the non-contiguous supported by 1T + 1T like mid-band].

One can also consider assuming the cases that only one TX chain can be switched, or for switching across 4 bands switching can be applied only between two exclusive 2 bands, e, g from Band A&B to Band C&D while switching within A&B bands or C&D bands is based on Rel-17 procedures.

Therefore, in order to have a manageable and meaningful investigations for extension of the UL Tx switching feature to 3 or 4 bands, it is important to select early during the study phase few applicable cases.

Considering all switching combinations across 3 or 4 bands results in large number of cases and with increased ambiguity as compared to switching across two bands.

Prioritize to reduce the number of cases based on few reasonable assumptions. Investigate at least the impact of ambiguous switching cases and corresponding switching time.

For UL CA, consider the scenario when a low band combined with non-contiguous CA (two carriers) in mid-band, the low-band can belong to a different TAG.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The UL TX switching feature for UL CA is currently supported for co-located scenarios with the single-TAG assumption.

Observation 2	Non-co-located scenarios for UL CA with multiple-TAG are common deployment scenarios in particular for CA band combinations with cells in bands below 2 GHz and cells in bands around 3.5 GHz.

Observation 3	Extending the deployment scenarios for UE TX switching to non-co-located scenarios for UL CA with multiple-TAG would be a useful enhancement of the UL TX switching feature.

Observation 4	Minor specification impact is expected to extend the support of UL TX switching across 2 bands for UL CA with multiple-TAG.

Observation 5	Considering all switching combinations across 3 or 4 bands results in large number of cases and with increased ambiguity as compared to switching across two bands.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Support UL TX switching across 2 bands for UL CA with multiple-TAG.

Proposal 2	Study on extension of ULTX switching to 3 or 4 bands should refrain from any TAG restriction for UL CA band combinations.

Proposal 3	Prioritize to reduce the number of cases based on few reasonable assumptions. Investigate at least the impact of ambiguous switching cases and corresponding switching time.

Proposal 4	For UL CA, consider the scenario when a low band combined with non-contiguous CA (two carriers) in mid-band, the low-band can belong to a different TAG.

# References

RP-220834		Revised WID on Multi-carrier enhancements; NTT DOCOMO, INC.

R4-2204604		Extending the deployment scenarios for UE TX switching: completing the RAN4 specification for non-colocation; Ericsson

# Appendix

- Tx switching between different cases across carriers based on SUL and NR inter-band uplink CA for UE supporting maximum two concurrent transmissions for the following scenarios:

    - For Tx switching based on SUL band combination, or uplink CA band combination

|  | Number of Tx chains (carrier 1 + carrier 2) |
| --- | --- |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

    - For Tx switching based on uplink CA band combination

|  | Number of Tx chains in WID (carrier 1 + carrier 2) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

- Tx switching between cases, where 1 carrier on band A and 2 contiguous aggregated carriers on band B, and band A is for SUL or non-SUL and band B is a non-SUL band for the following scenarios:

    - For Tx switching based on SUL band combination, or uplink CA band combination

|  | Number of Tx chains (band A + band B) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |

and

|  | Number of Tx chains (band A + band B) |
| --- | --- |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

    - For Tx switching based on uplink CA band combination

|  | Number of Tx chains (band A + band B) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |