3GPP TSG-RAN WG1 Meeting #109-e	R1-2204894

e-Meeting, May 9th – 20th, 2022

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on PDCCH Blind Detection in CA

Document for:	Discussion and Decision

Introduction

This paper discusses the PDCCH blind detection in case of carrier aggregation (CA). The purpose is to answer the incoming LS R1-2202901 from RAN2.

Discussion

In this section we give our views on the questions asked by RAN2 in the LS R2-2204055 [1].

- Question 1: Is UE required to report both pdcch-BlindDetectionMCG-UE-Mixed-r16 for MCG and pdcch-BlindDetectionSCG-UE-Mixed-r16 for SCG

There are two separate PDCCH blind detection parameters described in TS38.331 [2], one for MCG (pdcch-BlindDetectionMCG-UE-Mixed-r16) and one for SCG (pdcch-BlindDetectionSCG-UE-Mixed-r16).

According to current ASN.1 coding, there is no requirement for a UE supporting NR-DC to report both the capabilities for MCG and SCG. The corresponding parameter descriptrion in TS 38.331 is copied below:

-- R1 11-2e: Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel. 16 and -- Rel. 15 PDCCH monitoring capabilities on different carriers pdcch-BlindDetectionMCG-UE-Mixed-r16 SEQUENCE { pdcch-BlindDetectionMCG-UE1-r16 INTEGER (0..15), pdcch-BlindDetectionMCG-UE2-r16 INTEGER (0..15) } OPTIONAL, pdcch-BlindDetectionSCG-UE-Mixed-r16 SEQUENCE { pdcch-BlindDetectionSCG-UE1-r16 INTEGER (0..15), pdcch-BlindDetectionSCG-UE2-r16 INTEGER (0..15) } OPTIONAL,

However, according to the definitions in section 10.1 in TS 38.213 [3], both of the two above mentioned parameters are needed for PDCCH monitoring on MCG and SCG when the UE is configured for NR-DC operation with Rel-16 PDCCH monitoring capability on all the serving cells. The related specification is shown as follows.

When a UE is configured for NR-DC operation and is provided monitoringCapabilityConfig = r15monitoringcapability for at least one downlink cell and monitoringCapabilityConfig = r16monitoringcapability for at least one downlink cell where the UE monitors PDCCH, the UE may indicate, through pdcch-BlindDetectionMCG-UE1 and pdcch-BlindDetectionSCG-UE1, respective maximum values for pdcch-BlindDetection3 for the MCG and pdcch-BlindDetection3 for the SCG, and through pdcch-BlindDetectionMCG-UE2 and pdcch-BlindDetectionSCG-UE2 respective maximum values for pdcch-BlindDetection2 for the MCG and pdcch-BlindDetection2 for the SCG.

In addition, in Rel-15, the FG6-25a has been introduced in TS38.822 [4] for MCG and for SCG in synchronous NR-NR DC operation. Its counterpart from TS38.331 is copied below. It is noted that the parameter pdcch-BlindDetectionNRDC with SEQUENCE type includes one pdcch-BlindDetectionMCG-UE field and one pdcch-BlindDetectionSCG-UE field, which means both parameters must be reported by the UE. A similar ASN.1 coding should be applied for the Rel-16 NR-DC operation.

pdcch-BlindDetectionNRDC SEQUENCE { pdcch-BlindDetectionMCG-UE INTEGER (1..15), pdcch-BlindDetectionSCG-UE INTEGER (1..15) } OPTIONAL,

Proposal 1: Take the following as the answer to Question 1:

- For Rel-16 NR-DC operation with a mix of Rel-15 and Rel-16 PDCCH monitoring capability on different carriers, the UE is required to report both pdcch-BlindDetectionMCG-UE-Mixed-r16 for MCG and pdcch-BlindDetectionSCG-UE-Mixed-r16 for SCG in Rel-16.

- Question 2: How to interpret it if the UE reports both of FG 11-2c and FG 11-2g, or both of FG 11-2a and FG 11-2f

In the existing RAN2 specification, there is no restriction that only one of FG 11-2c and FG 11-2g is reported if supported, and no restriction that only one of FG 11-2a and FG 11-2f is reported if supported. However, in our understanding the RAN2 specification is not aligned with the agreements/understanding in RAN1. For example, according to the RAN1 agreement achieved in the RAN1#104-e meeting [5] below, it is clear that only one of FG 11-2c and FG 11-2g should be reported, and only one of 11-2a and FG 11-2f should be reported.

Agreements: - No change of pre-requisite for both FG 11-2d and FG 11-2e; - For FG 11-2d, add a note “If a UE supports FG 11-2a or FG 11-2f, then the capability defined by FG 11-2a or FG 11-2f is applied to FG 11-2d”. - For FG 11-2e, add a note “If a UE supports FG 11-2c or FG 11-2g, then the capability defined by FG 11-2c or FG 11-2g is applied to FG 11-2e”.

To further clarify,

- If UE only supports aligned span for all cases, UE can just report FG 11-2c with “aligned spans only”.

- If UE supports aligned span and non-aligned span with restriction, then UE can just report FG 11-2g.

- If UE supports aligned span and non-aligned span without restriction, then UE can just report FG 11-2c with “aligned spans and non-aligned spans”.

Therefore, there is no motivation to report both FG 11-2a and FG 11-2f. Similar situation for FG 11-2a and FG 11-2f.

Proposal 2: Take the following as the answer to Question 2:

- The UE is only required to report either FG 11-2a or FG11-2f.

- The UE is only required to report either FG 11-2c or FG11-2g.

Based on the above analysis, a draft LS reply R1-2203077 [6] is provided accordingly for further discussion in RAN1.

Conclusions

In this contribution, we discuss the LS reply to RAN2 about the PDCCH blind detection in CA and we are making the following observations and proposal:

Proposal 1: Take the following as the answer to Question 1:

- For Rel-16 NR-DC operation with a mix of Rel-15 and Rel-16 PDCCH monitoring capability on different carriers, the UE is required to report both pdcch-BlindDetectionMCG-UE-Mixed-r16 for MCG and pdcch-BlindDetectionSCG-UE-Mixed-r16 for SCG in Rel-16.

Proposal 2: Take the following as the answer to Question 2:

- The UE is only required to report either FG 11-2a or FG11-2f.

- The UE is only required to report either FG 11-2c or FG11-2g.

- References

R2-2204055, “Reply LS on PDCCH Blind Detection in CA”, February 21st - March 3rd 2022.

3GPP TS 38.331 V16.7.0 Radio Resource Control (RRC) protocol specification (Release 16)

3GPP TS 38.213 V16.8.0 Physical layer procedures for control (Release 16)

- 3GPP TR 38.822 V15.0.1 User Equipment (UE) feature list (Release 15)

Final Report of 3GPP TSG RAN WG1 #104-e v1.0.0, January 25th - February 5th 2021.

R1-2203077, “Draft reply LS on PDCCH Blind Detection in CA”, Huawei/HiSilicon, RAN1#109-e, May 9th-20th, 2022