3GPP TSG-RAN WG1 Meeting #109-e                                                                                    R1-2204896

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	8.2.5

Source:	Huawei, HiSilicon, SIA

Title:	Remaining issues of PUCCH enhancement and beam management enhancement for 52-71GHz spectrum

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the remaining issues for PUCCH and beam management enhancement for 52.6-71GHz spectrum.

Discussion

Potential CRs for multi-RB PUCCH format 4

In RAN1#104-e, the following agreement was made:

Agreement: - The configured number of RBs for enhanced PF 0/1/4 is denoted NRB - The minimum value of NRB is 1 for PF 0/1/4 for all subcarrier spacings - The maximum value of NRB depends on subcarrier spacing - FFS: maximum value for each SCS and each of PF0/1/4 - FFS: Allowed values of NRB within the [min/max] range - FFS: Details of indication of NRB by cell-specific (for PF0/1) and dedicated signaling (PF0/1/4) - FFS: Whether or not multiplexing of users with misaligned RB allocations is supported, where "misaligned" also includes users with different # of RBs. - For PF4: - The actual number of RBs used for a PUCCH transmission is equal to NRB, i.e., the actual number of RBs does not vary dynamically based on PUCCH payload - NRB fulfils the following: N=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers - Note: if frequency hopping is enabled, NRB is the number of RBs per hop - Note: decisions on the maximum value of NRB for each SCS and PUCCH format shall take into account link budgets based at least on the agreed evaluation assumptions

The actual number of PRBs used is equal to the configured number of PRBs for enhanced PUCCH format 4, which means that the actual number of PRBs does not vary dynamically based on the UCI of enhanced PUCCH format 4.

The description of rate matching for enhanced PUCCH format 4 in TS 38.212 [1] is shown below.

Clause 6.3.1.4 in TS 38.212 for rate matching of uplink control information on PUCCH For PUCCH formats 2/3/4, the total rate matching output sequence length is given by Table 6.3.1.4-1, where , , and are the number of symbols carrying UCI for PUCCH formats 2/3/4 respectively; , and NPRBPUCCH,4 are the number of PRBs that are determined by the UE for PUCCH formats 2/3/4 transmission respectively according to Clause 9.2 of [5, TS38.213]; and NSFPUCCH,2, NSFPUCCH,3, and are the spreading factors for PUCCH format 2, PUCCH format 3, and PUCCH format 4, respectively. Table 6.3.1.4-1: Total rate matching output sequence length PUCCH format; Modulation order. QPSK; π/2-BPSK. PUCCH format 2; 16∙Nsymb,UCIPUCCH,2∙NPRBPUCCH,2/NSFPUCCH,2; N/A. PUCCH format 3; 24∙Nsymb,UCIPUCCH,3∙NPRBPUCCH,3/NSFPUCCH,3; 12∙Nsymb,UCIPUCCH,3∙NPRBPUCCH,3/NSFPUCCH,3. PUCCH format 4; 24∙Nsymb,UCIPUCCH,4∙NPRBPUCCH,4NSFPUCCH,4; 12∙Nsymb,UCIPUCCH,4∙NPRBPUCCH,4NSFPUCCH,4.

However, despite the above clause, there is no description in Clause 9.2 of TS 38.213 [2] to determine the actual number of PRBs used for enhanced PUCCH format 4. This clause only provides the description to determine the actual number of PRBs used for PUCCH format 2/3. Given this and considering the agreement in RAN1#104-e that the actual number of PRBs does not vary dynamically based on the UCI of enhanced PUCCH format 4, we propose the following TP:

Proposal 1: Support the following TP#1 in clause 9.2.3 and clause 9.2.5.1 of TS 38.213.

TP#1 for TS 38.213 Clause 9.2.3 and Clause 9.2.5.1 < Unchanged parts are omitted > ### 9.2.3 UE procedure for reporting HARQ-ACK If a UE transmits a PUCCH with OACK HARQ-ACK information bits and OCRC bits using PUCCH format 4 in a PUCCH resource that includes MRBPUCCH PRBs, the UE determines a number of PRBs for the PUCCH transmission to be the configured number of PRBs, that is equal to a number of PRBs MRBPUCCH provided by nrofPRBs of PUCCH-format4. If a UE transmits a PUCCH with OACK HARQ-ACK information bits and OCRC bits using PUCCH format 2 or PUCCH format 3 in a PUCCH resource that includes MRBPUCCH PRBs, the UE determines a number of PRBs MRB,minPUCCH for the PUCCH transmission to be the minimum number of PRBs, that is smaller than or equal to a number of PRBs MRBPUCCH provided respectively by nrofPRBs of PUCCH-format2 or nrofPRBs of PUCCH-format3 and start from the first PRB from the number of PRBs, that results to OACK+OCRC≤MRB,minPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r and, if MRBPUCCH>1, OACK+OCRC>MRB,minPUCCH-1⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r, where Nsc,ctrlRB, Nsymb-UCIPUCCH, Qm, and r are defined in clause 9.2.5.2. For PUCCH format 3, if MRB,minPUCCH is not equal 2α2⋅3α3⋅5α5 according to [4, TS 38.211], MRB,minPUCCH is increased to the nearest allowed value of nrofPRBs [12, TS 38.331]. If OACK+OCRC>MRBPUCCH-1⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r, the UE transmits the PUCCH over MRBPUCCH PRBs. < Unchanged parts are omitted > ### 9.2.5 UE procedure for reporting multiple UCI types #### 9.2.5.1 UE procedure for multiplexing HARQ-ACK or CSI and SR in a PUCCH If a UE transmits a PUCCH with OACK HARQ-ACK information bits, OSR=log2K+1 SR bits, and OCRC CRC bits using PUCCH format 4 in a PUCCH resource that includes MRBPUCCH PRBs, the UE determines a number of PRBs for the PUCCH transmission to be the configured number of PRBs, that is equal to a number of PRBs provided by nrofPRBs in PUCCH-format4. If a UE transmits a PUCCH with OACK HARQ-ACK information bits, OSR=log2K+1 SR bits, and OCRC CRC bits using PUCCH format 2 or PUCCH format 3 in a PUCCH resource that includes MRBPUCCH PRBs, the UE determines a number of PRBs MRB,minPUCCH for the PUCCH transmission to be the minimum number of PRBs, that is smaller than or equal to a number of PRBs provided by nrofPRBs in PUCCH-format2 or nrofPRBs in PUCCH-format3 and starts from the first PRB from the number of PRBs, that results to OACK+OSR+OCRC≤MRB,minPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r and, if MRBPUCCH>1, OACK+OSR+OCRC>MRB,minPUCCH-1⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r, where Nsc,ctrlRB, Nsymb-UCIPUCCH, Qm, and r are defined in clause 9.2.5.2. For PUCCH format 3, if MRB,minPUCCH is not equal 2α2⋅3α3⋅5α5 according to [4, TS 38.211], MRB,minPUCCH is increased to the nearest allowed value of nrofPRBs [12, TS 38.331]. If OACK+OSR+OCRC>MRBPUCCH-1⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r, the UE transmits the PUCCH over the MRBPUCCH PRBs. < Unchanged parts are omitted >

Scheduling restrictions to address beam switching latency

Although the beam switch time at gNB is [59ns] according to the reply from RAN4 [3], 200 ns beam switch time at the UE side is suggested in RAN4 [4]:

- Proposals: - Option 1: 200 nsec for all SCS (Huawei, vivo, Apple, MediaTek, Qualcomm, OPPO) - Option 2: 50 nsec for all SCS (Nokia, Ericsson) - Option 3: 200 ns is too long, this requires more discussion (Ericsson) - Recommended WF by Moderator in the first round: - Option 1 - Tentative agreement based on majority view: - Option 1 (200 nsec for all SCS) NOTE: no agreement was reached on this topic

Even if UE side beam switch time is less than but comparable to the CP length of 73ns (146ns) in 960kHz (480kHz), in a realistic scenario, effects of channel dispersion, synchronization errors, and gNB MIMO TAE may be added to the beam switching time; making the overall latency not be absorbable within the CP of the OFDM symbol. Note also that although the UE receives DL signals/channels during the beam switch time, its beam is uncontrollable before the beam is steady. If any parts of the beam switch interval falls outside of the CP time into the OS, demodulation performance will be negatively impacted. As such, a full symbol gap needs to be considered for 960k Hz and 480 kHz if UE beam switch time is comparable to the CP time. Therefore, we propose the following:

Proposal 2: Regarding beam switch time, support the following for 960k Hz and 480 kHz SCSs

- UE is not expected to be scheduled/configured with a signal/channel on one symbol before to and one symbol after of another signal/channel if the signals/channels have two different QCL-D assumptions.

Conclusions

In this paper, we discussed the remaining issues above 52.6GHz with the following proposals.

Proposal 1: Support the following TP#1 in clause 9.2.3 and clause 9.2.5.1 of TS 38.213.

Proposal 2: Regarding beam switch time, support the following for 960k Hz and 480 kHz SCSs

- UE is not expected to be scheduled/configured with a signal/channel on one symbol before to and one symbol after of another signal/channel if the signals/channels have two different QCL-D assumptions.

- References

- 3GPP TS 38.212, “NR; Multiplexing and channel coding”

- 3GPP TS 38.213, “NR; Physical layer procedures for control”

- R4-2107985, LS on 60GHz time-related issues, Apple

- R4-2120063, WF on 60GHz UE timing, Apple