3GPP TSG-RAN WG1 Meeting #109-e 	R1-2204903

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.5.2

Source:	Huawei, HiSilicon

Title:	Maintenance of Rel-17 positioning latency and efficiency improvements

Document for:	Discussion and Decision

Introduction

In this paper, we discuss the maintenance of PRS measurement in the PRS processing window and INACTIVE state SRS transmission.

PRS-data collision detection timeline

In RAN1#108-e, there was discussion on the PRS-data collision detection timeline [2].

With the discussion, the following observations can be concluded.

- Observation 1: For PRS processing window type-1A

If PRS is higher priority than the DL signals/channels, UE is not expected to receive any DL signals/channels within the PRS processing window.

If PRS is lower priority than the DL signals/channels, UE is not expected to receive any PRS within the window if UE determines that the DL signals/channels are present in the PRS processing window.

The presence of PDCCH monitoring that may not have the actual PDCCH transceiving is also considered as presence of the DL signals/channels.

- Observation 2: For PRS processing window type-2

If PRS is higher priority than the DL signals/channels, UE is not expected to receive the DL signals/channels on the PRS symbols within the PRS processing window.

If PRS is lower priority than the DL signals/channels, UE is not expected to receive any PRS on the symbols on which UE determines that the DL signals/channels are present in the PRS processing window.

The presence of PDCCH monitoring that may not have the actual PDCCH reception is also considered as present of the DL signals/channels.

The PRS-data collision timeline actually refers to the case when UE will be able to determine whether the DL signals/channels of higher priority than PRS are present in the PRS processing window.

In the following example where we only consider PDCCH decoding time for a type-2 PRS processing window for the sake of simplicity, the collision detection timeline actually exists.

Assuming the DCI in the PDCCH schedules PDSCH in the same slot, in which UE is about to receive PRS on symbol 5 to symbol 10. The scheduled PDSCH is on symbol 2 to symbol 13, which is high priority than PRS. Due to the DCI decoding latency, it was not until symbol 9 that UE is aware of the presence of the PDSCH, and before that UE already receives PRS on symbol 5 to symbol 8.

It should be noted that the UE strategy of processing PRS symbols and communication symbols could be different, including AGC, baseband buffering method, and etc. However, in general, a symbol that is already determined to be processed as PRS symbol should not be fed back into the communication processing engine, and there may even be switching time between communication and positioning processing.

PRS

PDCCH

PRS

DCI decoding outcome

0

PRS that will be dropped

1

2

3

4

5

6

7

8

9

10

11

12

13

PRS that will be measured

PDSCH

PDSCH

Observation 3: The collision detection timeline exists at least for PRS and high priority communication signals/channels.

For PRS being higher priority, the PRS reception is deterministic, and UE will discard all communication receptions on the PRS symbols.

With regards to the communication traffic, it should be noted that for periodic/semi-persistence traffic, UE should be able to know in advance when to receive them, so there should be no need to define the “collision detection timeline”.

For SPS, the first occasion upon activation should be considered the same as aperiodic traffic, while the succeeding occasions should be considered the same as the semi-persistent traffic.

For SP CSI-RS, it is activated by a DL MAC CE, we prefer to treat all instances of SP CSI-RS (including the first one upon activation) as semi-persistent traffic. The reason is that even though the MAC CE takes effect at 3ms after the corresponding HARQ-ACK transmission by the UE, we do not think it is worthwhile to define another timeline starting from 3ms after the HARQ-ACK transmission, as it can be easily avoided by gNB implementation.

With the above discussion and observations, we have the following proposal.

- Proposal 1: Define the PRS collision detection timeline for the case when PRS may be lower priority than the dynamically scheduled DL traffic, which is applicable to all PRS processing window types (1A, 1B, 2).

The first PDSCH for SPS is considered as dynamically scheduled DL traffic.

On the timeline details, we think a simple way is to introduce a UE capability. For example, UE may report T in the unit of ms for a band.

The UE behaviour is summarized in the table in the following proposal.

Proposal 2: Agree with the following UE behaviour for both high priority PRS and low priority PRS.

UE declares a duration Tdetection per band in the capability signaling for detecting the collision between PRS and higher priority DL signals/channels.

|  | Case 1: PRS measurement is of higher priority | Case 2: PRS measurement is of lower priority |
| --- | --- | --- |
| 1A | UE is not expected to receive the DL signals and channels within the PRS processing window on all serving cells including SCG. | The UE is not expected to receive the scheduled DL signals/channels in the PRS processing window on all serving cells including SCG, if the corresponding DCI is later than Tdetection before the start of the PRS processing window and there are no DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with the scheduling DCI earlier than Tdetection before the start of the PRS processing window on any serving cell including SCG; The UE is not expected to receive the DL PRS within the PRS processing window if there are DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with the scheduling DCI earlier than Tdetection on any serving cell including SCG. |
| 1B | UE is not expected to receive the DL signals/channels within a PRS processing window on the serving cells in the same band as the DL PRS. | The UE is not expected to receive the scheduled DL signals/channels in the PRS processing window on the serving cells in the same band as the DL PRS, if the corresponding DCI is later than Tdetection before the start of the PRS processing window and there are no DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with DCI earlier than Tdetection before the start of the PRS processing window on serving cells in the same band as the DL PRS; The UE is not expected to receive the DL PRS within the PRS processing window if there are DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with the scheduling DCI earlier than Tdetection before the start of the PRS processing window on any serving cell in the band as the DL PRS. |
| 2 | UE is not expected to receive any DL signals/channels on a DL PRS symbol within the PRS processing window on the impacted serving cells | The UE is not expected to receive the scheduled DL signals/channels on the DL PRS symbol on the impacted serving cells, if the corresponding DCI is later than a threshold before the symbol and there are no DL signals/channels configured on the symbol on the impacted serving cells; The UE is not expected to receive the DL PRS on the symbol within the PRS processing window if there are DL signals/channels configured on the symbol or scheduled on the symbol with the scheduling DCI earlier than Tdetection before the symbol on the impacted serving cells. |

A TP is also provided to capture the above proposals in the specification.

Proposal 3: Endorse the following TP to TS 38.214.

Reason of change: The current description is not complete in terms of describing the UE behaviour to handle the collision between PRS of low priority and communication signals/channels of high priority.

Summary of change: Add UE behaviour to handle the low priority PRS, including the collision detection timeline.

Consequences if not approved: The UE behaviour for receiving PRS of low priority is not specified.

- ==================== Start of the TP ====================

When the UE is expected to measure the DL PRS outside the measurement gap in a configured PRS processing window with [Type-1A]

-	if the DL PRS is determined to be higher priority than the DL signals and channels, the UE is not expected to receive the DL signals and channels within the PRS processing window on all serving cells including SCG;

-	if the DL PRS is determined to be lower priority than the DL signals and channels

-	the UE is not expected to receive the scheduled DL signals/channels in the PRS processing window on all serving cells including SCG, if the corresponding DCI is later than [detectionTime] before the start of the PRS processing window and there are no DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with the scheduling DCI earlier than [detectionTime] before the start of the PRS processing window on any serving cell including SCG;

-	the UE is not expected to receive the DL PRS within the PRS processing window if there are DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with the scheduling DCI earlier than [detectionTime] before the start of the PRS processing window on any serving cell including SCG.

When the UE is expected to measure the DL PRS outside the measurement gap in a configured PRS processing window with [Type-1B]

-	if the DL PRS is determined to be higher priority than the DL signals and channels, the UE is not expected to receive the DL signals/channels within a PRS processing window on the serving cells in the same band as the DL PRS;

-	if the DL PRS is determined to be lower priority than the DL signals and channels

-	the UE is not expected to receive the scheduled DL signals/channels in the PRS processing window on the serving cells in the same band as the DL PRS, if the corresponding DCI is later than [detectionTime] before the start of the PRS processing window and there are no DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with DCI earlier than [detectionTime] before the start of the PRS processing window on serving cells in the same band as the DL PRS;

-	the UE is not expected to receive the DL PRS within the PRS processing window if there are DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with the scheduling DCI earlier than [detectionTime] before the start of the PRS processing window on any serving cell in the band as the DL PRS.

When the UE is expected to measure the DL PRS outside the measurement gap in a configured PRS processing window with [Type-2]

-	if the DL PRS is determined to be higher priority than the DL signals and channels, the UE is not expected to receive any DL signals/channels on a DL PRS symbol within the PRS processing window on the impacted serving cells;

-	if the DL PRS is lower priority than the DL signals and channels

-	the UE is not expected to receive the scheduled DL signals/channels on the DL PRS symbol on the impacted serving cells, if the corresponding DCI is later than [detectionTime] before the symbol and there are no DL signals/channels configured on the symbol on the impacted serving cells;

-	the UE is not expected to receive the DL PRS on the symbol within the PRS processing window if there are DL signals/channels configured on the symbol or scheduled on the symbol with the scheduling DCI earlier than [detectionTime] before the symbol on the impacted serving cells;

-	the impacted serving cells refer to the serving cell on which the [PRSProcessingWindow] is configured for a frequency range 1 band, and all the serving cells in the same band as the DL PRS for a frequency range 2 band.

- ==================== End of the TP ====================

Low latency PRS processing capability

Regarding whether and how a low latency PRS processing capability is defined, after so many rounds of discussion, we believe that a simple solution is to add another indicator in the {N, T} reporting that whether UE assumes that the T ms of processing is within the PRS processing window.

It is worth noting that RAN4 already made the agreement on reducing Tlast for MG-based PRS measurements, which can be applicable to MG-less.

Issue 1-3-1: Optimization of PRS measurements with gaps Agreements: - Define Tlast as T+MGL when all of the PRS resources to be measured are available in the same MG occasion during Tavailabe.

With the above discussion, we have the following proposal.

Proposal 4: Introduce an indicator in the existing UE capability signaling for the PRS processing capability of {N, T} that indicates whether the reported T ms is assumed within the PRS processing window or not.

The indicator is associated with any processing type that UE supports.

The {N, T} component should take that from FG 13-1, and values {1, 2, 4} ms are added to the existing T values.

Simultaneously activated PRS processing windows

RAN1#108-e agreed that the activated PRS processing window do not overlap in time. [1]

Agreement - The PRS processing window is configured per DL BWP. - Processing type, to be selected from 1A, 1B and 2, will be provided associated with the PRS processing window if and only if multiple processing types per band in the UE capability signaling is supported. - No need to provide band ID and CC ID associated with the PRS processing window. - A single priority indicator is provided for a PRS processing window, which applies to all PRS within the PRS processing window for the corresponding DL BWP. - The maximum number of activated PRS processing windows per DL BWP is 1. - The maximum number of activated PRS processing windows across all active DL BWPs is 4. - The maximum number of activated PRS processing windows overlapping in time across all active DL BWPs is 1

The agreement made assumed that UE should support multiple PRS processing windows being activated without capability signaling at all. To our understanding, a UE capability should be defined for that. For the most typical cases of latency reduction, a single PRS processing window and a single positioning frequency layer are configured. UE may choose to optimize the processing for that case, but may not support more activated PRS processing windows.

Proposal 5: Introduce a UE capability on the support of more than one activated PRS processing windows.

Per UE

Candidate values {2, 3, 4}.

The activated PRS processing windows do not overlap in time.

SUL support of SRS transmission in RRC_INACTIVE state

In RAN1#108-e, the following agreement was made on Option 2 SRS transmission for SUL.

Agreement For Option 2 of SRS for positioning configuration, - The feature is supported at least for NUL in Rel.17 - The SRS for positioning is configured in the same band and CC as the initial UL BWP - Signaling details are up to RAN2 - The following is up to UE capability indication - Support of different SCS, CP type from the initial UL BWP - Support a different center frequency between the SRS for positioning and the initial UL BWP - Whether bandwidth of SRS for positioning may not include bandwidth of the CORESET#0 and SSB

We do not think support of this feature on SUL should require additional effort in terms of the signaling.

First, there seemed to be no controversy over SUL on Option 1 SRS transmission, which implies that transmission of SRS for Option 1 on SUL is already supported.

Second, the SRS resource capability for RRC_INACTIVE state is already reported per band, and if UE does not support this feature, UE may simply not report the SUL band entry for the SRS capability.

Third, if UE supports NUL+SUL band combination that corresponds to FG 6-16, and FG 6-17 as shown in the Appendix, with the reported SRS capability for RRC_INACTIVE state on an SUL band, there should be no additional effort in the UE implementation to support SRS transmission in RRC_INACTIVE on a SUL band for supported NUL+SUL band combination.

Based on the above reasoning, we think that SUL should be supported for Option 2.

Proposal 6: The feature of Option 2 SRS transmission in RRC_INACTIVE state is supported for SUL.

Network identifies UE support of the feature by

  - The SRS resource capability reported for an SUL band, and

  - The supported NUL+SUL band combination that follows the capability reporting of FG 6-16 and FG 6-17

Unlike initial access and SDT, there shouldn’t be any carrier selection based on RSRP for SRS transmission on SUL. It means that UE should follow the configuration to determine on which carrier the SRS is transmitted.

For the collision between NUL data and SUL SRS and vice versa, we believe the existing FG 6-19 as shown in the Appendix should be used, meaning only if UE supports FG 6-19 for the feature set then UE should be able to simultaneously transmit NUL data and SUL SRS and vice versa.

Proposal 7: For the transmission of SRS (option 1 and option 2) associated with SUL

No UL carrier selection is specified.

Simultaneous transmission on NUL and SUL on RRC_INACTIVE state follows FG 6-19.

SRS switching time

In RAN1#108-e, the following working assumption was made on Option 2 SRS switching time.

Working assumption For Option 2 of SRS for positioning transmission in RRC_INACTIVE, a UE capability for switching time between SRS Tx and other Tx in initial UL BWP or Rx in initial DL BWP is introduced - The capability is reported per band - The capability applies at least to TDD - FFS: FDD - The switching time value(s) are left up to RAN4 discussion - If the transmission of SRS for positioning with the switching time overlaps/collides in time domain with other DL reception or UL transmission at least for TDD, the SRS for positioning transmission is dropped in the symbol(s) where the overlap/collision occurs - Note: Transmission of SRS for positioning with the switching time covers the following example TDD cases: - “switching after SRS” (i.e., transmission of SRS + switching time) - “switching before SRS” (i.e., switching time + transmission of SRS)

There was FFS on the FDD part because for FDD, transmission of SRS should not have impact on DL.

It should also be clarified that the behaviour of FDD also applies to SUL bands, on which no collision between DL and SUL will be assumed.

With above discussion, we have the following proposal.

- Proposal 8: For Option 2 of SRS for positioning transmission in RRC_INACTIVE, the UE capability of switching time between SRS Tx and other Tx in initial UL BWP is introduced also for FDD and SUL bands.

If the transmission of SRS for positioning with the switching time collides in time domain with other UL transmission for FDD and SUL, the SRS for positioning transmission is dropped in the symbols where the collision occurs.

Conclusion

In this contribution, we have discussed the remaining issues of PRS measurement in the PRS processing window and INACTIVE state SRS transmission, with the following observations and proposals.

- Observation 1: For PRS processing window type-1A

If PRS is higher priority than the DL signals/channels, UE is not expected to receive any DL signals/channels within the PRS processing window.

If PRS is lower priority than the DL signals/channels, UE is not expected to receive any PRS within the window if UE determines that the DL signals/channels are present in the PRS processing window.

The presence of PDCCH monitoring that may not have the actual PDCCH transceiving is also considered as presence of the DL signals/channels.

- Observation 2: For PRS processing window type-2

If PRS is higher priority than the DL signals/channels, UE is not expected to receive the DL signals/channels on the PRS symbols within the PRS processing window.

If PRS is lower priority than the DL signals/channels, UE is not expected to receive any PRS on the symbols on which UE determines that the DL signals/channels are present in the PRS processing window.

The presence of PDCCH monitoring that may not have the actual PDCCH reception is also considered as present of the DL signals/channels.

Observation 3: The collision detection timeline exists at least for PRS and high priority communication signals/channels.

- Proposal 1: Define the PRS collision detection timeline for the case when PRS may be lower priority than the dynamically scheduled DL traffic, which is applicable to all PRS processing window types (1A, 1B, 2).

The first PDSCH for SPS is considered as dynamically scheduled DL traffic.

Proposal 2: Agree with the following UE behaviour for both high priority PRS and low priority PRS.

UE declares a duration Tdetection per band in the capability signaling for detecting the collision between PRS and higher priority DL signals/channels.

|  | Case 1: PRS measurement is of higher priority | Case 2: PRS measurement is of lower priority |
| --- | --- | --- |
| 1A | UE is not expected to receive the DL signals and channels within the PRS processing window on all serving cells including SCG. | The UE is not expected to receive the scheduled DL signals/channels in the PRS processing window on all serving cells including SCG, if the corresponding DCI is later than Tdetection before the start of the PRS processing window and there are no DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with the scheduling DCI earlier than Tdetection before the start of the PRS processing window on any serving cell including SCG; The UE is not expected to receive the DL PRS within the PRS processing window if there are DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with the scheduling DCI earlier than Tdetection on any serving cell including SCG. |
| 1B | UE is not expected to receive the DL signals/channels within a PRS processing window on the serving cells in the same band as the DL PRS. | The UE is not expected to receive the scheduled DL signals/channels in the PRS processing window on the serving cells in the same band as the DL PRS, if the corresponding DCI is later than Tdetection before the start of the PRS processing window and there are no DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with DCI earlier than Tdetection before the start of the PRS processing window on serving cells in the same band as the DL PRS; The UE is not expected to receive the DL PRS within the PRS processing window if there are DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with the scheduling DCI earlier than Tdetection before the start of the PRS processing window on any serving cell in the band as the DL PRS. |
| 2 | UE is not expected to receive any DL signals/channels on a DL PRS symbol within the PRS processing window on the impacted serving cells | The UE is not expected to receive the scheduled DL signals/channels on the DL PRS symbol on the impacted serving cells, if the corresponding DCI is later than a threshold before the symbol and there are no DL signals/channels configured on the symbol on the impacted serving cells; The UE is not expected to receive the DL PRS on the symbol within the PRS processing window if there are DL signals/channels configured on the symbol or scheduled on the symbol with the scheduling DCI earlier than Tdetection before the symbol on the impacted serving cells. |

Proposal 3: Endorse the following TP to TS 38.214.

Reason of change: The current description is not complete in terms of describing the UE behaviour to handle the collision between PRS of low priority and communication signals/channels of high priority.

Summary of change: Add UE behaviour to handle the low priority PRS, including the collision detection timeline.

Consequences if not approved: The UE behaviour for receiving PRS of low priority is not specified.

- ==================== Start of the TP ====================

When the UE is expected to measure the DL PRS outside the measurement gap in a configured PRS processing window with [Type-1A]

-	if the DL PRS is determined to be higher priority than the DL signals and channels, the UE is not expected to receive the DL signals and channels within the PRS processing window on all serving cells including SCG;

-	if the DL PRS is determined to be lower priority than the DL signals and channels

-	the UE is not expected to receive the scheduled DL signals/channels in the PRS processing window on all serving cells including SCG, if the corresponding DCI is later than [detectionTime] before the start of the PRS processing window and there are no DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with the scheduling DCI earlier than [detectionTime] before the start of the PRS processing window on any serving cell including SCG;

-	the UE is not expected to receive the DL PRS within the PRS processing window if there are DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with the scheduling DCI earlier than [detectionTime] before the start of the PRS processing window on any serving cell including SCG.

When the UE is expected to measure the DL PRS outside the measurement gap in a configured PRS processing window with [Type-1B]

-	if the DL PRS is determined to be higher priority than the DL signals and channels, the UE is not expected to receive the DL signals/channels within a PRS processing window on the serving cells in the same band as the DL PRS;

-	if the DL PRS is determined to be lower priority than the DL signals and channels

-	the UE is not expected to receive the scheduled DL signals/channels in the PRS processing window on the serving cells in the same band as the DL PRS, if the corresponding DCI is later than [detectionTime] before the start of the PRS processing window and there are no DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with DCI earlier than [detectionTime] before the start of the PRS processing window on serving cells in the same band as the DL PRS;

-	the UE is not expected to receive the DL PRS within the PRS processing window if there are DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with the scheduling DCI earlier than [detectionTime] before the start of the PRS processing window on any serving cell in the band as the DL PRS.

When the UE is expected to measure the DL PRS outside the measurement gap in a configured PRS processing window with [Type-2]

-	if the DL PRS is determined to be higher priority than the DL signals and channels, the UE is not expected to receive any DL signals/channels on a DL PRS symbol within the PRS processing window on the impacted serving cells;

-	if the DL PRS is lower priority than the DL signals and channels

-	the UE is not expected to receive the scheduled DL signals/channels on the DL PRS symbol on the impacted serving cells, if the corresponding DCI is later than [detectionTime] before the symbol and there are no DL signals/channels configured on the symbol on the impacted serving cells;

-	the UE is not expected to receive the DL PRS on the symbol within the PRS processing window if there are DL signals/channels configured on the symbol or scheduled on the symbol with the scheduling DCI earlier than [detectionTime] before the symbol on the impacted serving cells;

-	the impacted serving cells refer to the serving cell on which the [PRSProcessingWindow] is configured for a frequency range 1 band, and all the serving cells in the same band as the DL PRS for a frequency range 2 band.

- ==================== End of the TP ====================

Proposal 4: Introduce an indicator in the existing UE capability signaling for the PRS processing capability of {N, T} that indicates whether the reported T ms is assumed within the PRS processing window or not.

The indicator is associated with any processing type that UE supports.

The {N, T} component should take that from FG 13-1, and values {1, 2, 4} ms are added to the existing T values.

Proposal 5: Introduce a UE capability on the support of more than one activated PRS processing windows.

Per UE

Candidate values {2, 3, 4}.

The activated PRS processing windows do not overlap in time.

Proposal 6: The feature of Option 2 SRS transmission in RRC_INACTIVE state is supported for SUL.

Network identifies UE support of the feature by

  - The SRS resource capability reported for an SUL band, and

  - The supported NUL+SUL band combination that follows the capability reporting of FG 6-16 and FG 6-17

Proposal 7: For the transmission of SRS (option 1 and option 2) associated with SUL

No UL carrier selection is specified.

Simultaneous transmission on NUL and SUL on RRC_INACTIVE state follows FG 6-19.

- Proposal 8: For Option 2 of SRS for positioning transmission in RRC_INACTIVE, the UE capability of switching time between SRS Tx and other Tx in initial UL BWP is introduced also for FDD and SUL bands.

If the transmission of SRS for positioning with the switching time collides in time domain with other UL transmission for FDD and SUL, the SRS for positioning transmission is dropped in the symbols where the collision occurs.

- Reference

- RAN1, Chair’s Notes RAN1#108-e v27, RAN1#108-e, e-Meeting, 21st Feb.  – 3rd Mar., 2022.

- R1-2202515, Summary #3 of [108-e-R17-ePos-04] latency improvements, Moderator (Huawei) , RAN1#108-e, e-Meeting, 21st Feb.  – 3rd Mar., 2022.

- R4-2206979, WF on NR Positioning Enhancements (Part 1), RAN4#102-e, Electronic Meeting, 21st Feb.  – 3rd Mar., 2022

- Appendix

- The figure groups associated SUL in Rel-15.

| 6. CA/DC, BWP, SUL | 6-16 | Supplemental uplink | 1) RACH, PUSCH, PUCCH, SRS operations in a band combination including SUL 2) Supplemental uplink with same numerology between SUL and non SUL carriers | 6-15 | supportedBandCombinationList | RF-Parameters | n/a | n/a | This is conditioned on the support of SUL band combination(s). | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6-17 | Supplemental uplink with different numerologies between SUL and non SUL carriers | Different numerologies between SUL and non SUL | 6-16 | supportedBandCombinationList | RF-Parameters | n/a | n/a | This is conditioned on the support of SUL band combination(s). | Mandatory with capability signalling |
|  | 6-18 | Supplemental uplink with dynamic switch | DCI based selection of PUSCH carrier | 6-16 | dynamicSwitchSUL | FeatureSetUplink | n/a | n/a | his is conditioned on the support of SUL band combination(s). | Optional with capability signalling |
|  | 6-19 | Simultaneous transmission of SRS on an SUL/non-SUL carrier and PUSCH/PUCCH/SRS on the other UL carrier in the same cell | Simultaneous transmission of SRS on an SUL/non-SUL carrier and PUSCH/PUCCH/SRS on the other UL carrier in the same cell | 6-16 | simultaneousTxSUL-NonSUL | FeatureSetUplink | n/a | n/a |  | Optional with capability signalling |