3GPP TSG-RAN WG1 Meeting #109e	R1-2204904

e-Meeting, May 9 – May 20, 2022

Agenda Item:	8.1.3

Source:	Huawei, HiSilicon

Title:	Remaining issues on SRS enhancement in Rel-17

Document for:	Discussion and Decision

Introduction

In previous meeting, some SRS enhancements including antenna switching enhancement and capacity enhancement was introduced in Rel-17[1], while there are still some remaining issues for the new feature. In this paper we share our views on the remaining issues and provide corresponding text proposals.

On SRS antenna switching

Text proposal on SRS configuration for T=R antenna switching in 38.214

Reason of changes:

For antenna switching T=R case, in Rel-15, up to two SRS resource sets are supported, including 2 periodic, or two aperiodic, or 1 periodic+1 aperiodic, etc. In Rel-17, when UE has the capability of “Maximum 2 SP and 1 periodic SRS sets for antenna switching”, the legacy configurations should be maintained, i.e., the configurations of 2 periodic, or 2 aperiodic, or 1 periodic+1 aperiodic should be supported. However, the mentioned legacy SRS configurations are missing for Rel-17 UE in the current wording of spec.

Consequence without changes:

For T=R, the legacy configurations of 2 periodic, or 2 aperiodic, or 1 periodic+1 aperiodic are missing for Rel-17 UE with the capability of “Maximum 2 SP and 1 periodic SRS sets for antenna switching”.

Proposed changes:

Text Proposal 1: We have the following text proposal for TS 38.214 V17.1.0

< Start of the text proposal > #### 6.2.1.2 UE sounding procedure for DL CSI acquisition < Unchanged parts are omitted > - For 1T=1R, or 2T=2R, or 4T=4R, up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 1, 2, or 4, if the UE is not indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets]. Two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' can be configured and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or up to two SRS resource sets can be configured, if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2, or 4, or < End of the text proposal >

Text proposal on definition of inter-set guard period for antenna switching in 38.214

Reason of changes:

In current 38.214 6.2.1, statement “UE shall use the same priority rules as defined above during the guard period as if SRS was configured” shows that when the collision happens between UL channel/signal with higher priority and guard period, UE should drop the guard period and transmit the UL channel/signal. According to the previous agreement, the collision handling rule above only applies to intra-set guard period, i.e., the words “guard period” is used to refer to “intra-set guard period”.

However, in the definition of inter-set guard period in 38.214 6.2.1.2, “a guard period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot”, the words “guard period” is also used to refer to “inter-set guard period”, which may lead to the misunderstanding that the collision handling rule above also applies to inter-set guard period, which violates the reached agreement and will cause conflict. To avoid the misunderstanding, it’s necessary to distinguish the definition of “guard period” and “inter-set guard period”.

Consequence without changes:

Whether the collision handling rule mentioned above should be used for inter-set guard period is unclear. Misunderstanding violating the reached agreement as well as causing conflict may occur.

Proposed changes:

Text Proposal 2: We have the following text proposal for TS 38.214 V17.1.0

< Start of the text proposal > #### 6.2.1.2 UE sounding procedure for DL CSI acquisition < Unchanged parts are omitted > The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. For two SRS resource sets of an antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, an inter-set guard period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot. < End of the text proposal >

On SRS capacity/coverage enhancement

Text proposal on parameter alignment for start RB hopping in 38.211 and 38.214

Reason of changes:

In Rel-17, start RB hopping was supported for partial sounding. Accordingly, a new parameter was introduced to define the hopping pattern. In 38.211, the hopping pattern is denoted as khop, while in 38.214, the hopping pattern is denoted as khopping. So it’s necessary to align it in the different spec.

Consequence without changes:

The parameter name for start RB hopping pattern is not aligned in 38.211 and 38.214, which may lead to the misunderstanding.

Proposed changes:

Text Proposal 3: We have the following text proposal for TS 38.214 V17.1.0:

< Start of the text proposal > #### 6.2.1 UE sounding procedure < Unchanged parts are omitted > - Defining start RB index hopping for partial frequency sounding in different SRS frequency hopping periods for aperiodic/periodic/semi-persistent SRS based on the hopping pattern khopas described in clause 6.4.1.4.3 in [4, TS 38.211. If not configured, then start RB hopping is not enabled and khopis fixed to be 0 for all SRS symbols. < End of the text proposal >

Conclusion

In this contribution, we discussed the remaining issues on SRS enhancements in Rel-17 and provide following text proposals:

Text Proposal 1: We have the following text proposal for TS 38.214 V17.1.0

< Start of the text proposal > #### 6.2.1.2 UE sounding procedure for DL CSI acquisition < Unchanged parts are omitted > - For 1T=1R, or 2T=2R, or 4T=4R, up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 1, 2, or 4, if the UE is not indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets]. Two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' can be configured and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or up to two SRS resource sets can be configured, if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2, or 4, or < End of the text proposal >

Text Proposal 2: We have the following text proposal for TS 38.214 V17.1.0

< Start of the text proposal > #### 6.2.1.2 UE sounding procedure for DL CSI acquisition < Unchanged parts are omitted > The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. For two SRS resource sets of an antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, an inter-set guard period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot. < End of the text proposal >

Text Proposal 3: We have the following text proposal for TS 38.214 V17.1.0:

< Start of the text proposal > #### 6.2.1 UE sounding procedure < Unchanged parts are omitted > - Defining start RB index hopping for partial frequency sounding in different SRS frequency hopping periods for aperiodic/periodic/semi-persistent SRS based on the hopping pattern khopas described in clause 6.4.1.4.3 in [4, TS 38.211. If not configured, then start RB hopping is not enabled and khopis fixed to be 0 for all SRS symbols. < End of the text proposal >

- References

3GPP, “RAN1#108e Meeting Chairman’s Notes”, E-meeting, February 21 – March 3, 2022.