3GPP TSG-RAN WG1 Meeting #109-e	R1-2204905

e-Meeting, May 9th – 20th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SRS resource set with 'antennaSwitching' |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the RAN1#108-e meeting, the following agreement was achieved to clarify the number of configured SRS resource set(s) with usage ‘antennaSwitching’ configured in srs-ResourceSetToAddModListDCI-0-2. Agreement For the maximum number of SRS resource sets with usage ‘antennaSwitching’, adopt Option 2: - Opt 2 – The SRS resource set(s) with usage=”antennaswitching” configured in srs-ResourceSetToAddModListDCI-0-2 shall not be different from the SRS resource set(s) configured in srs-ResourceSetToAddModListDCI with the same usage Note: The need to make RAN1 specification change with regards to this agreement needs further discussion.. This approach avoids that the UE is configured with more SRS resource set(s) for 'antennaSwitching' than what are possible compared to Rel-15, which is not feasible at this late stage (and would require a new UE capability). To avoid a misunderstanding between gNB and UE and to avoid that the UE is configured with more SRS resource sets that it can support, the above agreement needs to be captured in the specification. |
| Summary of change: |  | It is clarified that when the higher layer parameter usage is set to 'antennaSwitching', the SRS resource set(s) configured in srs-ResourceSetToAddModListDCI-0-2 are either equal to or are a subset of the SRS resource set(s) with the same usage configured in srs-ResourceSetToAddModList. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR has isolated impact on determining SRS resource sets with usage ‘antennaSwitching’. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.2	UE reference signal (RS) procedure

6.2.1	UE sounding procedure

The UE may be configured with one or more Sounding Reference Signal (SRS) resource sets as configured by the higher layer parameter SRS-ResourceSet or SRS-PosResourceSet. For each SRS resource set configured by SRS-ResourceSet, a UE may be configured with SRS resources (higher layer parameter SRS-Resource), where the maximum value of K is indicated by UE capability [13, 38.306]. When SRS resource set is configured with the higher layer parameter SRS-PosResourceSet, a UE may be configured with K ≥1 SRS resources (higher layer parameter SRS-PosResource), where the maximum value of K is 16. The SRS resource set applicability is configured by the higher layer parameter usage in SRS-ResourceSet. When the higher layer parameter usage is set to 'beamManagement', only one SRS resource in each of multiple SRS resource sets may be transmitted at a given time instant, but the SRS resources in different SRS resource sets with the same time domain behaviour in the same BWP may be transmitted simultaneously.

When the higher layer parameter usage is set to 'antennaSwitching', the SRS resource set(s) configured in srs-ResourceSetToAddModListDCI-0-2 are either equal to or are a subset of the SRS resource set(s) with the same usage configured in srs-ResourceSetToAddModList.

For aperiodic SRS at least one state of the DCI field is used to select at least one out of the configured SRS resource set(s).

< Unchanged parts are omitted >