3GPP TSG-RAN WG1 Meeting #109-e 	R1-2204906

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.6.2

Source:	Huawei, HiSilicon

Title:	Remaining issues on RAN2 related issues

Document for:	Discussion and Decision

Introduction

This paper discusses remaining issues for R17 RedCap UEs which may be related to RAN2 discussion.

Discussion

2.1 SDT related issues

(1) SI change indication and measurement

In last RAN1 meeting, it has been agreed that SDT resources can be configured in the separate initial BWP for RedCap UEs in the SDT topic.

RAN1#108-e Agreement for SDT RAN1 confirms that the separate BWP in case of RedCap may still be considered as the initial BWP and SDT resources(both CG-SDT and RA-SDT) can hence be configured on this BWP for RedCap UEs. - Note: details can be further studied to ensure proper functionality of RedCap UE performing SDT.

RAN2 previously agreed that UEs should monitor SI change and PWS notification during SDT procedure, which seems to imply that UEs have to monitor paging during SDT procedure.

RAN2#116b-e Agreement for SDT 9. During the SDT procedure (i.e. while SDT timer is running), UE monitors SI change indication in any paging occasion at least once per modification period (i.e. same as legacy RRC_CONNECTED). 10. During the SDT procedure (i.e. while SDT timer is running), ETWS or CMAS capable Ues monitors PWS notification in any paging occasion at least once every defaultPagingCycle (i.e. same as legacy RRC_CONNECTED).

On one hand, it is not clear if RAN2 intends to allow this possibility that a UE does not monitor SI change or PWS notification in their previous agreements, which were made prior to the agreements that the separate DL BWP without Paging was adopted in RAN1 RedCap session. On the other hand, SDT transmission contains not only initial transmission, but also subsequent data transmission, the procedure may last for several seconds. If paging is not monitored is also allowed (pending clarification) in the above case, RedCap UEs may miss the important public safety message.

Another issue is about TA validation. SSB based TA validation should be made during CG-SDT, which means UEs have to do measurement based on SSB during CG-SDT as per the agreements.

RAN1#105-e Agreement for SDT The SSB subset for RSRP based TA validation is determined at least based on a configured absolute RSRP threshold.

For a BWP does not contain any SSB, it is unclear whether/how it can/is preferred to be used for SDT procedure. As there are willingness to implement both RedCap and SDT features, how they are interacted needs further clarification.

Proposal 1: RAN1 to clarify the RedCap separate BWP used for SDT procedure in INACTIVE mode, e.g. w.r.t paging monitoring, and the configurability.

(2) HARQ feedback

RAN1#106-e Agreement for SDT • RAN1 confirms that common PUCCH resources (i.e. those that are shared with non-SDT UEs) can also be used for HARQ-ACK feedback for Msg4 /MsgB and subsequent SDT transmissions. • RAN1 thinks there is no need for any other PUCCH resources than common PUCCH resources shared with non-SDT UEs.

During SDT procedure, only common PUCCH resource can be used. As defined in TS 38.21 [1], common PUCCH support format 0 and format 1, which can carry one or two bits. For FG 4-1 Basic UL control channel, “6) HARQ-ACK transmission once per slot with its resource/timing determined by using the DCI”. So at most 2 HARQ-ACK bits can be transmitted in one slot. If gNB schedule HARQ-ACK feedback associated with more than two TBs in the same slot, it should be specified which HARQ-ACK to feedback, or such case is not expected.

Proposal 2: RAN1 to clarify that during SDT procedure, UEs do not expect to be scheduled HARQ-ACK associated with more than two TBs in one slot.

2.2 Cell bar

The Rx branches/DL MIMO layer number for R17 RedCap UEs is 1 or 2, which may affect the network efficiency, so the gNB is allowed to have access control for RedCap UEs, as described in WID:

Objective of Core part WI [1]:

- Specify a system information indication to indicate whether a RedCap UE can camp on the cell/frequency or not; it shall be possible for the indication to be specific to the number of Rx branches of the UE. [RAN2, RAN1]

In TS 38.331 [2], whether to bar UEs with 1Rx branch and UEs with 2Rx branches are indicated separately in SIB1:

cellBarredRedCap-r17                SEQUENCE {

cellBarredRedCap1Rx-r17             ENUMERATED{barred, notBarred},

cellBarredRedCap2Rx-r17             ENUMERATED{barred, notBarred}

The indication can support four barred/not barred cases, including:

- Case1:{bar 1Rx, bar 2Rx}

- Case2:{bar 1Rx, not bar 2Rx}

- Case3:{not bar 1Rx, bar 2Rx}

- Case4:{not bar 1Rx, not bar 2Rx}

Although, this indication provide the flexibility for gNB to control the access of RedCap UEs with 1Rx and/or 2Rx, we think Case 3 is an unreasonable configuration. To get higher spectrum efficiency, operators may want to bar 1Rx only or bar 1Rx+2Rx, but there is no scenario for gNB to bar 2Rx while not bar 1Rx. Moreover, a UE with 2Rx may report itself as UE with 1Rx to access the cell, which make configuration Case3 have the same effect as Case4.

Proposal 3: It is an invalid case for gNB to bar 2Rx branches but not bar 1Rx branch.

Conclusions

In this contribution, we presented our views on the remaining issues besides bandwidth reduction for RedCap UEs:

Proposal 1: RAN1 to clarify the RedCap separate BWP used for SDT procedure in INACTIVE mode, e.g. w.r.t paging monitoring, and the configurability.

Proposal 2: RAN1 to clarify that during SDT procedure, UEs do not expect to be scheduled HARQ-ACK associated with more than two TBs in one slot.

Proposal 3: It is an invalid case for gNB to bar 2Rx branches but not bar 1Rx branch.

- References

- RP-211574, “Revised WID on support of reduced capability NR devices”.

- TS 38.331, “Radio Resource Control (RRC) protocol specification (Release 17)”.