3GPP TSG-RAN WG1 Meeting #109-e 	R1-2204910

e-Meeting, May 9 – 20, 2022

Agenda Item:	9.9.3

Source:	Huawei, HiSilicon

Title:	Discussion on additional evaluation results for NR PDCCH in collision with LTE CRS

Document for:	Discussion and Decision

Introduction

In RAN #94 meeting, a new WID on enhancement of NR dynamic spectrum sharing was approved for Release 18 [1], where RAN1 agreed to study solutions to improve the NR spectrum efficiency on the shared carriers:

The following objectives shall be included for improvement of NR spectrum efficiency for LTE-NR co-existence (RAN1): - Study and if needed specify NR PDCCH reception in symbols with LTE CRS REs. [RAN1] - Investigate enabling LTE CRS to puncture NR PDCCH, including the impact to NR PDCCH DMRS if there is the performance gain from the additional PDCCH resources. - Allow a UE to support, and be configured with, two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP [RAN1, RAN2]

Objective 1, i.e., NR PDCCH reception in symbols with LTE CRS REs is discussed in our companion contribution [2]. In this contribution, some additional evaluation results for Objective 1 is provided.

Discussion

PDCCH performances for UEs at 30 km/h are evaluated in [2]. In this contribution, PDCCH performances for UEs at 350 km/h are evaluated for two legacy cases and 3 R18-DSS cases below (i.e. the same cases as in [2]).

- For legacy DSS Case 1, Rel-18 PDCCH symbols can be increased to:

  - Case 1_enh1: CORESET is configured on 2 symbols as shown in Fig. 4, where one symbol does not overlap with the CRS (the 3rd symbol of a slot), and the other symbol overlaps with the CRS (the 2nd symbol of a slot).

  - Case 1_enh2: CORESET is configured on 3 symbols as shown in Fig. 5, where one symbol does not overlap with the CRS (the 3rd symbol of a slot), and the other two symbols overlaps with the CRS (the 1st and 2nd symbol of a slot).

- For legacy DSS Case 2, Rel-18 PDCCH symbols can be increased to:

- Case 2_enh: CORESET is configured on 3 symbols as shown in Fig. 6, where two symbols do not overlap with the CRS (the 2rd and 3rd symbol of a slot), and the other one symbol overlaps with the CRS (the 1rd symbol of a slot).

Fig. 1 Case 1 vs Case 1_enh1

Fig. 2 Case 1 vs Case 1_enh2

Fig. 3 Case 2 vs Case 2_enh

The following three figures show the PDCCH BLER performances of for Case 1 vs Case 1_enh1, Case 1 vs Case 1_enh2, and Case 2 vs Case 2_enh respectively.

Fig. 4 PDCCH BLER performance of Case 1 vs Case 1_enh1.

Fig. 5 PDCCH BLER performance of Case 1 vs Case 1_enh2.

Fig. 6 PDCCH BLER performance of Case 2 vs Case 2_enh.

Using the methodology introduced in [2], the PDCCH performances for Case 1 vs Case 1_enh1, Case 1 vs Case 1_enh2, and Case 2 vs Case 2_enh are provided and summarized in Table 1.

Table 1 PDCCH performances for Case 1 vs Case 1_enh1, Case 1 vs Case 1_enh2, and Case 2 vs Case 2_enh under 350km/h

|  | Occupied PDCCH symbols (consumed resources) | Capacity gain over baseline | Outage UE percentage |
| --- | --- | --- | --- |
| Case 1 (baseline) | 1-symbol | -- | 1% |
| Case 1_enh1 (taking Case 1 as baseline) | 2-symbol w/ 1 collision symbol (+100% resources) | 29.6% | 4% |
| Case 1_enh2 (taking Case 1 as baseline) | 3-symbol w/ 2 collision symbols (+200% resources) | 38.2% | 9% |
| Case 2 (baseline) | 2-symbol CORESET | -- | 1% |
| Case 2_enh (taking Case 2 as baseline) | 3-symbol w/ 1 collision symbol (+50% resources) | 15.7% | 4% |

The PDCCH capacity gains for Case 1 vs Case 1_enh1, Case 1 vs Case 1_enh2, and Case 2 vs Case 2_enh at 30 km/h evaluated in [2] are 36.2%, 55.7% and 19.1% respectively. Comparing the gains of these two speeds, it can be found that, the PDCCH capacity gain at 350 km/h is even lower than the gain at 30 km/h especially in Case 1 vs Case 1_enh2.  The reason is that at high speed, the channel changes rapidly with time, so the channel estimation performance would be decreased, and causes further reliability loss in the PDCCH reception.

Observation: For NR PDCCH reception in symbols with LTE CRS REs, the PDCCH capacity gain at high speed is lower than the PDCCH capacity gain at low speed.

Conclusion

According to the above discussions, we have the following observation:

Observation: For NR PDCCH reception in symbols with LTE CRS REs, the PDCCH capacity gain at high speed is lower than the PDCCH capacity gain at low speed.

- References

- RP-213575, “New WI: Enhancement of NR Dynamic spectrum sharing (DSS)”, TSG RAN Meeting #94e, Dec. 6 - 17, 2021

- R1-2203137, Discussion on NR PDCCH reception in symbols with LTE CRS REs, May, RAN1#109-e.

- Appendix

Table 2 Link level simulation assumptions

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2.1 GHz |
| SCS | 15 kHz |
| Bandwidth | 20 MHz |
| Channel model | TDL-A |
| Delay spread | 100 ns |
| UE speed | 350 km/h |
| Number of BS antennas | 4 Tx (Two cross-polarized antenna pairs) |
| Number of UE antennas | 4 Rx (Two cross-polarized antenna pair) |
| DCI payload (excluding CRC) | 60 bits |
| Interleaving | Non-Interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 PRBs |

Table 3 System level simulation assumptions

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2.1 GHz |
| SCS | 15 kHz |
| Simulation bandwidth | 20 MHz |
| BS antenna height | 25 m |
| UE height | 1.5m |
| TRP transmit power | 49 dBm |
| Scenario | Urban Macro |
| ISD | 500m |
| Device deployment | 80% indoor, 20% outdoor |
| UE speeds | Indoor users: 3km/h |
|  | Outdoor users (in-car): 30 km/h |
| BS noise figure | 5 dB |
| BS antenna element gain | 8 dBi |
| UE noise figure | 9 dB |
| Thermal noise level | -174 dBm/Hz |
| Traffic | Full Buffer |
| Macro sites | 19 |
| Downtilt | 102° |
| Minimum BS to UE distance | 35m |