3GPP TSG RAN WG1 Meeting #109-e	R1-2204911

e-Meeting, May 9 – 20, 2022

Agenda Item:	9.2.5

Source:	Huawei, HiSilicon

Title:	Discussions on other potential use cases of AI/ML for NR air interface

Document for:	Discussion and Decision

Introduction

In RAN #94 plenary meeting, a new SI to study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface is approved [1], and the objectives are attached in the Appendix. Based on the objective, three use cases should be studied to justify the attainable gain of AI/ML based techniques and also to identify the potential specification impact required to enable AI/ML techniques for the air-interface.

During the discussion of the RAN #94 plenary meeting [2], potential use cases for AI/ML for air interface have been raised. The popular use cases include CSI feedback, beam management, positioning, RS enhancement, mobility, etc. Due to the consideration of the workload, only the first three use cases are included in the SID of AI/ML for NR air interface. In order to provide some more information on other potential use cases empowered by AI/ML as future study directions, this contribution will discuss the AI/ML-based channel estimation enhancement.

AI/ML-based channel estimation enhancement

In Rel-15, PUSCH DM-RS configuration Type 1 and Type 2 have been specified to support 8 and 12 orthogonal DMRS ports, respectively. In order to satisfy the increasing need for multiplexing capacity, 24 orthogonal DMRS ports without increasing overhead will be studied in Rel-18 WI on MIMO evolution [3]. With the increasing of multiplexed stream number, the inter-stream interference is more serious, and lead to harmed performance of channel estimation as a result. AI/ML-based solution can learn the channel and interference characteristics that may not be easily handled by legacy channel estimation solutions. In the following, the evaluation methodology and results for AI/ML-based channel estimation enhancement are provided.

Evaluation methodology

The legacy channel estimation methods rely on orthogonal DMRS ports to decompose the signal of each stream. Least square (LS) channel estimation with a de-noise filter is a typical example to derive the channel response on DMRS REs. Based on that, the channel response of the neighboring data REs is estimated by performing filtering or interpolation based on the estimated channel of the DMRS REs. For frequency selective fading channel, the orthogonality of the received signals on DMRS ports may be impaired due to fading. As a consequence, the decomposed signal may contain some residual interference from other streams, leading to performance loss.

By introducing AI/ML-based channel estimation solution, the channel response of the data REs can be directly derived by taking the received DMRS signal as input of AI/ML model since it can learn the accurate channel characteristics. As long as the AI/ML model can also well learn the interference characteristics, it can estimate more accurate channel response on data REs based on the received DMRS signal which attributes to inter-stream interference suppression.

Procedure of AI/ML operation

One example of the procedure of AI/ML-based channel estimation is shown as Figure 1, where the AI/ML model is NW-oriented, i.e. the training and inference is performed by gNB, and UL channel estimation scenario is considered. In model training phase, training inputs should be collected. Supervised learning is considered, thus training input would contain ground-truth channel response, which can be achieved by high accurate channel estimation algorithms or training dedicated DMRS in realistic networks.

Figure 1 - Procedure of AI/ML-based channel estimation for uplink

The inference procedure of AI/ML-based channel estimation is shown as Figure 2. The input of AI/ML model is the received signal y containing hybrid signals of all uplink MU-MIMO transmission streams and the DMRS sequences of these streams. The output of AI/ML model is the estimated channel response of data REs for these streams.

Figure 2 – Inference procedure of AI/ML-based channel estimation

DMRS patterns for channel estimation

The scenario is assumed to be dense UE environment including in total 24 UEs for UL MU-MIMO and each UE has one transmission layer. Both of NR Type 2 DMRS supporting up to 12 ports and one candidate Rel-18 DMRS pattern with 24 orthogonal DMRS ports are evaluated as shown in Figure 3. For Rel-15 Type 2 DMRS pattern, 2 different pseudo-random sequences are used to generate DMRS for 24 UEs. For R18 candidate DMRS patterns, the length of frequency domain OCC is extended from 2 to 4 to support 24 orthogonal DMRS ports.

| - NR PUSCH DMRS configuration type 2 | - One candidate Rel-18 DMRS pattern |
| --- | --- |

Figure 3 – Two different DMRS patterns

Dataset construction

Link level simulation is performed to evaluate the channel estimation performance, and the dataset is generated from the simulated channel model. To guarantee the generalization of the AI/ML model, a mixed CDL-A, CDL-B and CDL-C channels are generated and collected into the dataset. Other assumptions for the dataset are shown as Table 1.

Table 1 – Training dataset assumptions

| Parameters | Values |
| --- | --- |
| Channel model | CDL channel according to 3GPP TR38.901 |
| Channel types | Mixed channel, including CDL-A, CDL-B and CDL-C, with delay spread range [100, 400]ns |
| Size | 50000 samples, where each sample contains 24 UEs |
| Other configurations | Uplink, 1T64R, 16RB, 24 UEs |

AI/ML model

The AI/ML model used for channel estimation is learned approximate message passing (LAMP). LAMP is a popular type of model-driven AI/ML models solving compressed sensing problems by unfolding the traditional AMP algorithm and treating some theoretical parameters as learnable parameters of AI/ML models [4]. We model the channel estimation problems in delay domain and it can be modeled as a compressed sensing problem so that LAMP can be used to solve the problem.

Baseline

The baseline is single UE-based channel estimation. Per UE signals are decomposed firstly by the orthogonal DMRS ports, and then least square (LS) channel estimation algorithm with a de-noise filter is applied to derive the channel response of the DMRS REs. Finally, the channel response of the data REs are estimated by interpolating the estimated channel of neighboring DMRS REs.

Evaluation assumptions

Other evaluation assumptions are shown as Table 2.

Table 2 – Other simulation assumptions

| Parameters | Values |
| --- | --- |
| Channel model | CDLC400 |
| Carrier Frequency | 3.5GHz |
| SCS | 30KHz |
| Bandwidth | 16RB |
| Number of UEs | 24 |
| Antenna configuration | 1T64R |
| Receiver | IRC |
| MCS for BLER evaluation | 12 |

Evaluation results

The Evaluation results are shown as Figure 4, where the performances are evaluated in terms of NMSE (left figure) and BLER (right figure).

| - NMSE | - BLER |
| --- | --- |

Figure 4 –Evaluation results for AI/ML channel estimation

For NR PUSCH DMRS Type 2, a high error floor appears for baseline since two non-orthogonal pseudo-random sequences are used which incurs strong interference to each other, while the AI/ML-based channel estimation can still achieve a good performance attributing to the inter-stream interference suppression.

For 24 orthogonal DMRS ports, by learning the correlations of channel among subcarriers, AI/ML-based channel estimation can achieve 1) more than 7dB gain in term of channel estimation NMSE, and 2) around 3.5dB gain in term of SNR @ 0.1 BLER than the baseline channel estimation.

Observation: AI/ML-based channel estimation enhancement outperforms legacy channel estimation algorithms on supporting large number of spatially multiplexed streams due to the inter-stream interference suppression.

- For Type 2 DMRS with 12 orthogonal ports and two pseudo-random sequences to support 24 spatially multiplexed streams, baseline channel estimation has high error floor, while AI/ML-based channel estimation can reach 0.1 BLER with 1dB SNR.

- For candidate 24 orthogonal DMRS ports to support 24 spatially multiplexed streams, AI/ML-based channel estimation can achieve 3.5dB gain in term of SNR @ 0.1 BLER than the baseline channel estimation.

Proposal: Study AI/ML-based channel estimation enhancement to improve the performance of large number of spatially multiplexed UEs.

Conclusions

In this contribution, we discussed AI/ML-based channel estimation enhancement. Based on the discussion, we have the following observation.

Observation: AI/ML-based channel estimation enhancement outperforms legacy channel estimation algorithms on supporting large number of spatially multiplexed streams due to the inter-stream interference suppression.

- For Type 2 DMRS with 12 orthogonal ports and two pseudo-random sequences to support 24 spatially multiplexed streams, baseline channel estimation has high error floor, while AI/ML-based channel estimation can reach 0.1 BLER with 1dB SNR.

- For candidate 24 orthogonal DMRS ports to support 24 spatially multiplexed streams, AI/ML-based channel estimation can achieve 3.5dB gain in term of SNR @ 0.1 BLER than the baseline channel estimation.

Proposal: Study AI/ML-based channel estimation enhancement to improve the performance of large number of spatially multiplexed UEs.

- References

RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#94e, December 2021.

RP-212668, “Moderator’s summary for discussion [RAN94e-R18Prep-08] AI/ML for Air Interface”, RAN#94e, December, 2021.

RP-213599, “New WID: MIMO Evolution for Downlink and Uplink”, RAN#94e, December, 2021.

Borgerding, Mark, Philip Schniter, and Sundeep Rangan. "AMP-inspired deep networks for sparse linear inverse problems." IEEE Transactions on Signal Processing 65, no. 16 (2017): 4293-4308.

- Appendix - Objectives in WID

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.