3GPP TSG-RAN WG1 Meeting #109-e 	R1-2204916

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.4

Source:	Huawei, HiSilicon, China Unicom

Title:	Considerations on the support of PRS/SRS bandwidth aggregation

Document for:	Discussion and Decision

Introduction

RAN#94e approved the SID [1] on expanded and improved NR positioning. The following objective is identified to support bandwidth aggregation of PRS and SRS.

- Study solutions for accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers considering e.g. timing errors, phase coherency, frequency errors, power imbalance, etc [RAN4]:

In this paper, we discuss RAN1 aspects to support PRS/SRS bandwidth aggregation.

Discussion

The terminology of CA positioning, which is no longer used in the formal WID document, actually corresponds to the myth that in order to support positioning CA, UE and gNB should also support communication CA. However, as the positioning interest grows as the benefit on accuracy improvement from a larger bandwidth shows, the desire to expand the communication bandwidth may not be the same in the same scenario.

Take UWB for example, the RF bandwidth is typically 500MHz (499.2MHz), but the data rate is far less than the bandwidth (approximately tens of Mbps). Positioning does not call for high data rate, but actually requires the bandwidth only for the sake of fine ToA estimate.

This calls for the decoupling of communication bandwidth (not only about the RF bandwidth, but also the corresponding baseband processing capability to cope with the peak data rate that occupies with corresponding RF bandwidth) and positioning bandwidth (only about the RF bandwidth, assuming reference signal processing including transmission preparation and receive processing can be done offline).

It is sensible to decouple PRS/SRS bandwidth aggregation and communication CA from network/feature deployment aspect as well. In typical deployments, radio hardware that fits operator spectrum holding (e.g. wideband hardware) is deployed from day 1, while communication CA is enabled depending on capacity and data rate requirement later on. Decoupling PRS/SRS bandwidth aggregation and communication CA can make better utilization of the already deployed wideband hardware to support improved positioning accuracy without the necessity to deploy communication CA.

Then it should be designed to allow both UE and gNB to only implement PRS/SRS bandwidth aggregation, but not CA for communication. For example, gNB/UE may support a single CC of 100 MHz for communication, but its IF/RF still have a high capability to process more than 100 MHz reference signal.

A full functionality baseband mapped to a PCC that can handle the overall communication services, aggregated with a lite baseband mapped to the entire IF/RF bandwidth that is only tasked to process the aggregated positioning reference signal transmission and reception, can effectively reduce the cost from both UE and gNB.

Target PCell

Positioning only

Full functionality BB (SSB+CSI-RS+PXCCH+PXSCH+SRS)

Pos functionality BB

(PRS/SRS-only)

Radio/RF capability

No data

With data

Target PCell

Positioning only

Full functionality BB (SSB+CSI-RS+PXCCH+PXSCH+SRS)

Pos functionality BB

(PRS/SRS-only)

Radio/RF capability

No data

With data

Figure 1 Illustration of decoupling PRS/SRS bandwidth aggregation and communication CA

- Observation 1: Decoupling support of PRS/SRS bandwidth aggregation and communication CA enables positioning accuracy improvement without the necessity to bear the full cost of communication CA for both UE and gNB.

Conclusion

In this contribution, we provided our view on the support of PRS/SRS bandwidth aggregation, and we have the following observation.

- Observation 1: Decoupling support of PRS/SRS bandwidth aggregation and communication CA enables positioning accuracy improvement without the necessity to bear the full cost of communication CA for both UE and gNB.

- Reference

- RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel, RAN#94-e, Electronic Meeting, 6th – 17th Dec, 2021.