3GPP TSG-RAN WG1 Meeting #109-e	R1-2204917

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.6.3

Source:	Huawei, HiSilicon

Title:	Overall considerations for Rel-18 RedCap

Document for:	Discussion and Decision

# Introduction

In RAN#94 meeting, a new SID on support of further NR RedCap UE complexity reduction has been approved [1]. The main goal is to further reduce UE complexity/cost under the framework of Rel-17 RedCap. The approved SID are as following:

- Study further UE complexity reduction techniques based on Rel-17 evaluation methodology in TR 38.875 [RAN1] - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact - Potential solutions, which may complement each other, for reducing device complexity are focusing on: - UE bandwidth reduction to 5MHz in FR1, - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - reduced UE peak data rate in FR1, - Possibly including restricted bandwidth for PDSCH and/or PUSCH - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - Notes: - Rel-15 SSB should be reused and L1 changes minimized. - Operation in BWP with/without SSB and without/with RF retuning should be considered. - It is not precluded that some solutions for FR1 can be applied to FR2 in WI stage. - Aim to define a single Rel-18 RedCap UE type for further UE complexity reduction.

In this contribution, we want to discuss some overall considerations for Rel-18 RedCap.

# Overall considerations for Rel-18 RedCap

Rel-18 RedCap intends to further reduce UE complexity based on Rel-17 RedCap. Before the detailed discussion, we want to provide some overall consideration for Rel-18 RedCap on coexistence, BWP operation, and efficiency, etc.

## Coexistence

As discussed in Rel-17 RedCap, some negative impact on non-RedCap UEs will be caused by RedCap UEs, thus the coexistence between Rel-17 RedCap UEs and non-RedCap UEs was considered, such as PUSCH fragmentation issue. For Rel-18 RedCap UEs, legacy UEs include not only non-RedCap UEs but also Rel-17 RedCap UEs. So, coexistence between Rel-17 and Rel-18 RedCap and non-RedCap UEs should be considered.

In Rel-17 RedCap, potential PUSCH fragmentation caused by common PUCCH of RedCap UEs has been identified and solved. This issue will reduce non-RedCap UEs’ peak data rate, especially for the non-RedCap UEs which do not support discontinuous uplink frequency resource allocation. The agreed solution is allowing network to deactivate frequency hopping for RedCap PUCCH resources. If the network intends to avoid PUSCH fragmentation, it can place the separate initial UL BWP at the end of carrier bandwidth and deactivate frequency hopping for RedCap PUCCH resources. In our view, this issue will still exist and be more complicated for Rel-18 RedCap UEs. Only reusing Rel-17 solution seems not enough. For example, the above solution may result in that both Rel-18 RedCap UEs and Rel-17 RedCap UEs are squeezed into the same limited bandwidth at the end of carrier bandwidth. Since the peak data rate is reduced, Rel-18 RedCap UEs occupy the limited bandwidth longer than Rel-17 RedCap UEs, costing further capacity reduction of Rel-17 RedCap UEs.

In addition to the issue of PUSCH fragmentation, there are also some other coexistence issues, such as PDCCH blockage caused by the RedCap UEs, which has been discussed and not resolved in Rel-17 RedCap.

In our view, the above potential coexistence issues should be considered in Rel-18 RedCap.

Proposal 1: In Rel-18 RedCap, potential issues, such as PUSCH fragmentation and PDCCH blockage etc., should be considered into coexistence based on the Rel-17 framework.

## BWP operation

In Rel-15, BWP functionality was introduced. A BWP is defined by a location (start PRB) and number of contiguous PRBs which is not larger than UE maximum bandwidth. A UE can be configured with multiple BWPs and can change the active BWP via BWP switching mechanism, which can be triggered by DCI, RRC signaling, or timer expiry.

If Rel-18 RedCap UEs reuse the current BWP operation, there may be some issues. For UEs in RRC_Idle/Inactive mode, reusing the current BWP operation will result in all Rel-18 RedCap UEs restricted within limited bandwidth of cell-specific BWP. It will much reduce access capacity and cause the loss of frequency diversity gain for Rel-18 RedCap UEs. To solve this issue, multiple initial DL/UL BWP can be considered. However, multiple initial DL/UL BWP will bring big challenge to the current initial BWP framework, and will lead to larger signaling overhead. In addition, multiple BWP configurations require larger UE memory cost, which is not good for UE complexity reduction. This issue is more serious for UE dedicated BWPs in RRC_Connected mode. To cover 100 MHz carrier bandwidth, more than 4 BWPs are needed, the signaling overhead and UE memory requirements are very large. So, this issue should be studied carefully.

On the other hand, during BWP switching, a UE can’t transmit or receive any data. And the current switch delay is slot-level for DCI triggered BWP switch. Hence, the current dynamic BWP switching is inefficient.

Thus, some enhancements on current BWP operation can be considered.

Proposal 2: In Rel-18 RedCap, enhancements to BWP operation can be considered.

## Efficiency

As discussed in contribution [3], for the common messages, e.g., SIB1, OSI, paging, RAR, etc., bandwidth of the messages may or may not exceed 5MHz. A natural solution is to introduce new common messages dedicated for Rel-18 RedCap UE which will be limited within 5MHz. However, it will cause additional resource overhead and seems not efficient. So how to transmit common messages to Rel-18 RedCap UE efficiently can be studied. The target should be less resource consumption and better performance.

In summary, in our view, during the study of further UE complexity reduction, efficiency should be an important aspect to be considered in Rel-18 RedCap.

Proposal 3: In Rel-18 RedCap, efficiency improvement (e.g., reduce signaling overhead of Rel-18 UEs) can be considered.

# Conclusions

Based on the analysis, we have the following observations and proposals:

Proposal 1: In Rel-18 RedCap, potential issues, such as PUSCH fragmentation and PDCCH blockage etc., should be considered into coexistence based on the Rel-17 framework.

Proposal 2: In Rel-18 RedCap, enhancements to BWP operation can be considered.

Proposal 3: In Rel-18 RedCap, efficiency improvement (e.g., reduce signaling overhead of Rel-18 UEs) can be considered.

# References

R1-213661, “New SID on Study on further NR RedCap UE complexity reduction” Ericsson, RAN#94-e.

3GPP TR 38.875, “Study on support of reduced capability NR devices”.

R1-2203169, “Discussion on potential solutions to further reduce UE complexity”, Huawei, HiSilicon, RAN1#109-e.