3GPP TSG-RAN WG1 Meeting #109-e	R1-2204918

e-Meeting, May 9– May 20, 2022

Agenda Item:	9.7.3

Source:	Huawei, HiSilicon

Title:	Discussion on information assistance for network energy saving

Document for:	Discussion and Decision

Introduction

The SID for NR network energy saving contains the following objectives [1]:

- Definition of a base station energy consumption model [RAN1] - Adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 to the base station side, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, base station load, etc), sleep states and the associated transition times, and one or more reference parameters/configurations. - Definition of an evaluation methodology and KPIs [RAN1] - The evaluation methodology should target for evaluating system-level network energy consumption and energy savings gains, as well as assessing/balancing impact to network and user performance (e.g. spectral efficiency, capacity, UPT, latency, handover performance, call drop rate, initial access performance, SLA assurance related KPIs), energy efficiency, and UE power consumption, complexity. The evaluation methodology should not focus on a single KPI, and should reuse existing KPIs whenever applicable; where existing KPIs are found to be insufficient new KPIs may be developed as needed. Note: WGs will decide KPIs to evaluate and how. - Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include: - How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2] - Information exchange/coordination over network interfaces [RAN3] Note: Other techniques are not precluded

One of the objectives is to study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception. In this contribution, we will discuss information exchanged over network interface and some potential UE assistance information. Other potential techniques for improving network energy saving will be discussed in another contribution [2]. The discussion of base station energy consumption model and evaluation methodology can be found in [3].

Discussion

Information exchanged/coordination over network interface

In Rel-16, it is supported to reduce energy consumption via information exchanging over Xn interface. For example, in a deployment, the cell with capacity boosters can be distinguished from cells providing basic coverage. Capacity booster cell can be switched off when its capacity is no longer needed and to be re-activated via on-demand basis, which is briefly shown in Figure 1.

Figure 1. Briefly introduction of R16 energy saving over network interface.

We illustrate this function with help of Figure 2 in details. The function builds upon the possibility for the NG-RAN node owning a capacity booster cell (e.g., NG-RAN node2) to autonomously decide to switch-off such cell to lower energy consumption (e.g., Step 1). The switch-off decision can be typically based on cell load information or can be taken by O&M. Then the NG-RAN node will initiate handover actions in order to offload the cell being switched off, and indicate the reason for handover with an appropriate cause value (e.g., Step 1-a) to support the target node taking subsequent actions.

All neighbor NG-RAN nodes are informed by the NG-RAN node owning the concerned cell (e.g., NG-RAN node2) about the switch-off actions over the Xn interface, by means of the NG-RAN node Configuration Update procedure (e.g., Step 2). During switch off time period of the boost cell, all informed nodes maintain the cell configuration data, e.g., neighbor relationship configuration and the NG-RAN node may prevent idle mode UEs from camping on this switch-off cell.

The NG-RAN node owning non-capacity boosting cells (e.g., NG-RAN node1) may decide to switch-on the cells previously being deactivated due to energy saving (e.g., Step 3) if cell load is heavy or decided by O&M, and then Cell Activation procedure will be performed over the Xn interface (e.g., Step 4).

All peer NG-RAN nodes will be informed by the NG-RAN node owning the concerned cell (e.g., NG-RAN node1) about the re-activation by an indication on the Xn interface (e.g., Step5).

Figure 2. Exchanged information over Xn interface

Based on the above analysis, it can be seen that Rel-16 already supports to use exchanged information over Xn interface to reduce energy consumption. Thus, the necessity and benefits of exchanging/coordinating information over network interfaces for network energy saving can be further studied based on the existing scheme in RAN2/RAN3 first while RAN1 awaits when there is any impact foreseen.

Proposal 1: Study in RAN3 first the necessity and benefits of exchanging/coordinating information over network interfaces for network energy saving based on the existing schemes.

Potential UE assistance information

Network energy saving should not be achieved by endlessly sacrificing data transmission performance. Thus, sleep and active modes are proposed for BS power consumption modeling [3]. For low/medium traffic load, network can enter energy saving modes, e.g., sleep modes, to reduce energy consumption. But when the traffic load becomes high, network should turn into non-energy saving mode, e.g., active mode, to assure the data transmission performance. Therefore, UE assistance information should be considered from two perspectives. One is for assisting network to make decisions whether/when to enter/resume from sleep modes to save energy or perform data transmission, the other is to help mitigate the potential performance impact due to implementation of energy saving techniques or even enhance the performance.

Proposal 2: Study UE assistance information for the purpose of

- assisting network to make decisions on the need/timing for entering/resume from sleep modes

- mitigating the performance impact when BS energy saving techniques are implemented.

Based on the above goals, the following existing UE information can be considered:

- Mobility information, e.g., location, velocity, for predicting the time of UE departure from the cell

- Traffic information: e.g., traffic distribution, volume change, expected volume, etc.

- Function wise information: e.g., CSI reports/feedback.

However, the specific discussion for the first purpose should involve RAN2 to jointly study whether benefits can be brought by the above UE assistance information for network energy saving. For physical layer function associated UE feedback information, they can be studied along with proposed energy saving techniques.

Proposal 3: UE assistance information in terms of mobility/traffic information can be studied jointly with RAN2 while UE feedback information with performance impact, e.g. CSI feedback is to be studied and evaluated in RAN1.

Conclusions

In this contribution, the following observation and proposals are made:

Proposal 1: Study in RAN3 first the necessity and benefits of exchanging/coordinating information over network interfaces for network energy saving based on the existing schemes.

Proposal 2: Study UE assistance information for the purpose of

- assisting network to make decisions on the need/timing for entering/resume from sleep modes

- mitigating the performance impact when BS energy saving techniques are implemented.

Proposal 3: UE assistance information in terms of mobility/traffic information can be studied jointly with RAN2 while UE feedback information with performance impact, e.g. CSI feedback is to be studied and evaluated in RAN1.

- References

RP-220297, Revised SI: Study on network energy savings for NR, March 17 – March 23, 2022

R1- 2203173, Discussion on network energy saving techniques, Huawei, Hisilicon, May 9 – May 20, 2022

R1- 2203172, Discussion on performance evaluation for network energy saving, Huawei, Hisilicon, May 9 – May 20, 2022