3GPP TSG-RAN WG1 Meeting #109-e 	R1-2204921

e-Meeting, May 9th – 20th, 2022

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on LS from RAN4 on FR2 RedCap UE

Document for:	Discussion and Decision

Introduction

In [1], RAN4 has sent an LS, which is mainly to discuss signaling design due to the new power class for FR2 RedCap UE introduced by RAN4: “design signalling for new power class 7 for FR2 RedCap UE and evaluate if there is any other signalling impact”. However, the description about the number of Rx branch and DL MIMO layer is not clear or misleading. We raise this issue in this paper.

Discussion

- During the study phase for Redcap UEs, the antenna configurations for RedCap UEs has been discussed, and the final conclusion is included in WID as below. RedCap UE can support 1Rx branch or 2Rx braches according to different use cases or implementation complexity for both FR1 and FR2, UE can report the supported Rx branch number by PDSCH MIMO layer number. And the RedCap UE feature definition in TS 38.306 for R17 RedCap UEs is also aligned with the description in the WID.

Objective of Core part WI [2]: - Maximum number of DL MIMO layers: - For a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported. - For a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported.

- TS 38.306 [3]: - 4.2.21 RedCap Parameters - 4.2.21.1 Definition of RedCap UE: … - 1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported. UE features and corresponding capabilities related to more than 2 UE Rx branches and more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 2 UE Tx branches and more than 2 UL MIMO layers are not supported by RedCap UEs;

- In the RAN4#101be meeting, the following agreement are made:

RAN4#101be Agreement [4] Issue 5-1: New power class for RedCap UE [Agreement:] - For power class for FWA device - Reuse the PC5 power class - For power class for wearable UE and additional industry sensor use case - FFS whether to define the new power class for wearable UE and additional industry sensor separately, or define one power class for both - To limit the number of additional RF requirements, focus on the following three requirements - Minimum EIRP - Minimum EIS - Spherical coverage - For all the devices above, 2-layer DL MIMO is not mandated and FFS whether to define 2-layer MIMO performance requirements for them. Issue 5-2-1-1: Reducing the# of Rx branch [Agreement:] Not reduce the number of Rx branch for FR2

It can be seen that the discussion and relevant agreements in RAN4 related to the number of Rx branches and the number of DL MIMO layers is to limit the additional RF requirements for test purpose for some specific use cases [5]. While the description of below in the LS only captures part of the background in RAN4 discussion context, and conflicts with WID requirement and RAN1/RAN2 specifications:

- Not reduce the number of Rx branches, i.e. 2 Rx branches assumed for FR2 RedCap UE

- 2-layer DL MIMO is not mandated

Therefore,

Proposal: RAN1 to clarify to RAN2/RAN4 that there should be no differentiation between FR1 and FR2 regarding the support of number of Rx branches as well as the corresponding number of DL MIMO layers, i.e. this should not be interpreted in a way that has any signaling impact on RAN1/RAN2 specifications, e.g. UE feature finalization.

Conclusions

In this contribution, we discuss the potential mismatch between RAN1&RAN2 and RAN4, and propose

Proposal: RAN1 to clarify to RAN2/RAN4 that there should be no differentiation between FR1 and FR2 regarding the support of number of Rx branches as well as the corresponding number of DL MIMO layers, i.e. this should not be interpreted in a way that has any signaling impact on RAN1/RAN2 specifications, e.g. UE feature finalization.

- References

- R4-2206545, “LS on FR2 RedCap UE”.

- RP-211574, “Revised WID on support of reduced capability NR devices”.

- 3GPP TS 38.306 V17.0.0, “User Equipment (UE) radio access capabilities (Release 17)”.

- R4-2206144, “DRAFT Meeting Report for TSG RAN WG4 meeting: 101-bis-e”

- R4-2202371, “WF on the RedCap RF”, RAN4 #101-bis-E.