3GPP TSG-RAN WG1 Meeting #109-e 	R1-2204922

e-Meeting, May 9th – 20th, 2022

Agenda Item:	7.2.8

Source:	Huawei, HiSilicon

Title:	PRS reception without TDD configuration

Document for:	Discussion and Decision

Introduction

LTE background

In LTE, the TDD configurations of neighboring cells provided to the UE are introduced in Rel-15 in R2-1817896 [1]. This is because PRS are not transmitted on special subframes and UL subframes, on which the PRS repetition should not be present, and in the meantime, UE should not perform reception on those subframes.

OTDOA-NeighbourCellInfoElement ::= SEQUENCE {

physCellId							INTEGER (0..503),

cellGlobalId						ECGI				OPTIONAL,		-- Need ON

earfcn								ARFCN-ValueEUTRA	OPTIONAL,		-- Cond NotSameAsRef0

cpLength							ENUMERATED {normal, extended, ...}

OPTIONAL,		-- Cond NotSameAsRef1

prsInfo								PRS-Info			OPTIONAL,		-- Cond NotSameAsRef2

antennaPortConfig					ENUMERATED {ports-1-or-2, ports-4, ...}

OPTIONAL, 		-- Cond NotsameAsRef3

slotNumberOffset					INTEGER (0..19)		OPTIONAL,		-- Cond NotSameAsRef4

prs-SubframeOffset					INTEGER (0..1279)	OPTIONAL,		-- Cond InterFreq

expectedRSTD						INTEGER (0..16383),

expectedRSTD-Uncertainty			INTEGER (0..1023),

...,

[[ earfcn-v9a0					ARFCN-ValueEUTRA-v9a0	OPTIONAL		-- Cond NotSameAsRef5

]],

[[	tpId-r14					INTEGER (0..4095)		OPTIONAL,		-- Need ON

prs-only-tp-r14				ENUMERATED { true }		OPTIONAL,		-- Cond TBS

cpLengthCRS-r14				ENUMERATED { normal, extended, ... }

OPTIONAL,		-- Cond CRS

sameMBSFNconfigNeighbour-r14	BOOLEAN				OPTIONAL,		-- Need ON

dlBandwidth-r14				ENUMERATED {n6, n15, n25, n50, n75, n100}

OPTIONAL,		-- Cond NotSameAsRef6

addPRSconfigNeighbour-r14	SEQUENCE (SIZE (1..maxAddPRSconfig-r14)) OF

Add-PRSconfigNeighbourElement-r14

OPTIONAL		-- Need ON

]],

[[

tdd-config-v1520			TDD-Config-v1520		OPTIONAL		-- Need ON

]]

}

It was also clarified that the TDD configuration should be the same on the same frequency band (LTE).

TDD-Config field descriptions

subframeAssignment This field specifies the TDD UL/DL subframe configuration where sa0 points to Configuration 0, sa1 to Configuration 1 etc. as specified in TS 36.211 [16], table 4.2-2. The target device assumes the same value for all assistance data cells residing on same frequency band.

Rel-16 NR background

In NR Rel-16, RAN1 already made the following agreements with respect to the slot format of the serving cell only, as documented in the following agreements.

Agreement made in RAN1#99.

Agreement: - Configured DL PRS are transmitted on DL symbols of a slot configured by higher layers - Configured DL PRS are transmitted on symbols of slot configured as flexible symbols by higher layers - If the UE is not provided with a measurement gap, the UE is not expected to process DL PRS Resources on serving or neighboring cells on symbols indicated as UL by the serving cell

Agreement made in RAN1#100-e.

Agreement: Proposals 1-7 in Section 3 of R1-2001365 including text proposals for TS38.213 and TS38.214 are agreed

With the enclosed TPs

Proposal 1: 	TS 38.213, clause 11.1 Slot configuration

[…] For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS in a set of symbols of a slot, the UE receives the PDCCH, the PDSCH, the CSI-RS, or the DL PRS if the UE does not detect a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3 that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot; otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS, or the DL PRS in the set of symbols of the slot. For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format 1_0, DCI format 1_1, or DCI format 0_1 indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - the UE does not expect to cancel the transmission in symbols from the set of symbols that occur, relative to a last symbol of a CORESET where the UE detects the DCI format 1_0 or the DCI format 1_1 or the DCI format 0_1, after a number of symbols that is smaller than the PUSCH preparation time for the corresponding UE processing capability [6, TS 38.214] assuming and corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 1_0, DCI format 1_1 or DCI format 0_1 and the SCS configuration of the SRS, PUCCH, PUSCH or r, where r corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise r=0 - the UE cancels the PUCCH, or PUSCH, or PRACH transmission in remaining symbols from the set of symbols and cancels the SRS transmission in remaining symbols from the subset of symbols For a set of symbols of a slot that are indicated to a UE as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not receive PDCCH, PDSCH, or CSI-RS when the PDCCH, PDSCH, or CSI-RS overlaps, even partially, with the set of symbols of the slot. For a set of symbols of a slot that are indicated to a UE as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not receive DL PRS in the set of symbols of the slot, if the UE is not provided with a measurement gap. For a set of symbols of a slot that are indicated to a UE as downlink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit PUSCH, PUCCH, PRACH, or SRS when the PUSCH, PUCCH, PRACH, or SRS overlaps, even partially, with the set of symbols of the slot. [..]

Proposal 2: TS 38.213, clause 11.1.1 UE procedure for determining slot format

For a set of symbols of a slot indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE detects a DCI format 2_0 providing a format for the slot using a slot format value other than 255 […] - if the UE is configured by higher layers to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE receives the PDSCH or the CSI-RS in the set of symbols of the slot only if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as downlink - if the UE is configured by higher layers to receive DL PRS in the set of symbols of the slot, the UE receives the DL PRS in the set of symbols of the slot only if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as downlink or flexible. - if the UE is configured by higher layers to transmit PUCCH, or PUSCH, or PRACH in the set of symbols of the slot, the UE transmits the PUCCH, or the PUSCH, or the PRACH in the slot only if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as uplink […]

Proposal 3: TS 38.213, clause 11.1.1 UE procedure for determining slot format

[…] If a UE is configured by higher layers to receive a CSI-RS or a PDSCH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as uplink or flexible, or the UE detects a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3 indicating to the UE to transmit PUSCH, PUCCH, SRS, or PRACH in at least one symbol in the set of the symbols, the UE cancels the CSI-RS reception in the set of symbols of the slot or cancels the PDSCH reception in the slot. If a UE is configured by higher layers to receive a DL PRS in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as uplink, or the UE detects a DCI format 0_0, DCI format 0_1, DCI format 0_2, DCI format 1_0, DCI format 1_1, DCI format 1_2, or DCI format 2_3 indicating to the UE to transmit PUSCH, PUCCH, SRS, or PRACH in at least one symbol in the set of the symbols, the UE cancels the DL PRS reception in the set of symbols of the slot. If a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as downlink or flexible, or the UE detects a DCI format 1_0, DCI format 1_1, or DCI format 0_1 indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then […]

Proposal 4: 	TS 38.213, clause 11.1.1 UE procedure for determining slot format

[…] For a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE does not detect a DCI format 2_0 providing a slot format for the slot - the UE receives PDSCH or CSI-RS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format 1_0, DCI format 1_1, or DCI format 0_1 - the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, DCI format 2_3, or a RAR UL grant - the UE receives PDCCH as described in Clause 10.1 - if the UE is configured by higher layers to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE does not receive the PDSCH or the CSI-RS in the set of symbols of the slot - if the UE is configured by higher layers to receive DL PRS in the set of symbols of the slot, the UE receives the DL PRS. - if the UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in the set of symbols of the slot and the UE is not provided EnableConfiguredUL-r16, the UE […]

Proposal 5: TS 38.214, clause 5.1.6.5 PRS reception procedure

[…] The UE is expected to measure the DL PRS resource outside the active DL BWP or with a numerology different from the numerology of the active DL BWP if the measurement is made during a configured measurement gap. When not configured with a measurement gap, the UE is only required to measure DL PRS within the active DL BWP and with the same numerology as the active DL BWP. When the UE is expected to measure the DL PRS resource outside the active DL BWP it may request a measurement gap in higher layer parameter [XYZ]. […]

Proposal 6:

UE can receive PRS on flexible symbols indicated by SFI.

Proposal 7:

If UE fails to detect an SFI, UE receives DL PRS on DL and flexible symbols of a slot configured by higher layers.

Discussion

The intention of the above agreements for NR is intended for the PRS reception that covers the serving cell, where the semi-static slot format and/or dynamic SFI can be received by the UE for the purpose of identifying the DL/UL/FL symbols in a slot and the symbols that can be used for PRS reception.

- Observation 1: The existing PRS reception versus the slot format is intended for the case when serving cell is concerned, in which case the semi-static slot format and/or the dynamic SFI can be available at the UE.

On the other hand, PRS measurement, like other RRM procedures, may also include the positioning frequency layers that does not overlap with the serving cell, and in some extreme cases, it can be possible that PRS that the UE is receiving is not coming from the serving cell at all.

Observation 2: It is possible that some positioning frequency layers does not contain any PRS from the serving cell.

Applying the serving cell TDD configuration to the positioning frequency layers that are independent from the serving cell may be problematic, since this would require multiple positioning frequency layers to share the same aligned TDD configuration which is too restrictive.

Observation 3: The serving cell slot format may not be applied to the positioning frequency layers that do not contain the PRS from the serving cell.

The problem can be simply formulated as:

What is the slot format assumption on the PRS symbol for the positioning frequency layers that do not contain PRS from any serving cell?

To resolve the problem, two alternatives can be considered.

Alt.1 UE may assume the symbol as DL/FL that is configured for PRS reception on the positioning frequency layers that do not contain the PRS from any serving cell.

Alt.2 Introduce the TDD configuration in the assistance data

  - The TDD configuration is provided per TRP or per positioning frequency layer, which is common for all TRPs within a positioning frequency layer.

  - For the TDD configuration of the non-serving cell, UE may receive the PRS on DL/FL symbols.

  - UE capability for supporting TDD configuration is introduced for backward compatibility.

  - Introduce the NRPPa signaling for LMF to obtain the TDD configuration from each TRP.

- Given the late stage of Rel-16, we suggest to make no ASN.1 change to Rel-16 specification, but RAN1 can further discuss whether Alt.2 can be introduced for Rel-17.

- Proposal 1: RAN1 to clarify that for Rel-16, on positioning frequency layers that do not contain the PRS from any serving cell, UE may assume the symbol as DL/FL that is configured for PRS reception.

- Proposal 2: RAN1 to discuss whether the following change is adopted in Rel-17:

- Introduce the TDD configuration in the assistance data The TDD configuration is provided per TRP or per positioning frequency layer, which is common for all TRPs within a positioning frequency layer. For the TDD configuration of the non-serving cell, UE may receive the PRS on DL/FL symbols. UE capability for TDD configuration is introduced for backward compatibility. Introduce the NRPPa signaling for LMF to obtain the TDD configuration from each TRP.

Conclusion

In this contribution, we have discussed the UE behaviour of PRS reception without TDD configuration for the non-serving cells. Based on the discussion, we have the following observations and proposals.

- Observation 1: The existing PRS reception versus the slot format is intended for the case when serving cell is concerned, in which case the semi-static slot format and/or the dynamic SFI can be available at the UE.

Observation 2: It is possible that some positioning frequency layers does not contain any PRS from the serving cell.

Observation 3: The serving cell slot format may not be applied to the positioning frequency layers that do not contain the PRS from the serving cell.

- Proposal 1: RAN1 to clarify that for Rel-16, on positioning frequency layers that do not contain the PRS from any serving cell, UE may assume the symbol as DL/FL that is configured for PRS reception.

- Proposal 2: RAN1 to discuss whether the following change is adopted in Rel-17:

- Introduce the TDD configuration in the assistance data The TDD configuration is provided per TRP or per positioning frequency layer, which is common for all TRPs within a positioning frequency layer. For the TDD configuration of the non-serving cell, UE may receive the PRS on DL/FL symbols. UE capability for TDD configuration is introduced for backward compatibility. Introduce the NRPPa signaling for LMF to obtain the TDD configuration from each TRP.

- Reference

- R2-1817896, Addition of TDD UL/DL configuration to OTDOA assistance data, Qualcomm, RAN2#104, Spokane, US, 12 – 16 Nov. 2018.