3GPP TSG-RAN WG1 Meeting #109-e	R1-2204931

e-Meeting, May 9th – 20th, 2022

Agenda Item:	7.2.6

Source:	Huawei, HiSilicon

Title:	Discussion on triggering offset of aperiodic CMR and IMR set for L1-SINR

Document for:	Discussion and Decision

Introduction

In Rel-15, it was agreed that aperiodic CMR set and the associated aperiodic IMR (CSI-IM or NZP CSI-RS) set should be in the same slot for CSI report in RAN1#92bis [1] and 93 meeting [2].

Agreement 1 (RAN1#92bis)

Aperiodic triggering offset of CSI-IM follows triggering offset value of the associated NZP CSI-RS for channel measurement.

Agreement 2 (RAN1#93)

If interference measurement is performed on aperiodic NZP CSI-RS, UE is not expected to be configured with a different aperiodic triggering offset of the NZP CSI-RS for interference measurement from the associated NZP CSI-RS for channel measurement.

In Rel-16, it was agreed that L1-SINR measurement is based on CMR set and IMR set in RAN1#97 [3].

Agreement 3 (RAN1#97)

- When dedicated IMR is not configured,

  - If CMR is based on CSI-RS, when L1-SINR is configured, and interference measurement is performed using CMR with CSI-RS only with density 3 REs/RB for 1-port CSI-RS is used

    - Spec does not require UE to use SSB for interference measurement

  - Note: CSI-RS above is CSI-RS for BM

- When dedicated IMR is configured,

  - NW can configure interference measurement for L1-SINR with either of the following options

    - ZP-IMR only

    - NZP-IMR only

    - (WA) ZP-IMR and NZP IMR (interference measurement is taken on both)

      - Maximum Number of ZP IMR is 1

  - If IMR is configured based on NZP IMR only, when L1-SINR is configured, interference measurement is performed only with density 3 REs/RB CSI-RS

  - If IMR is configured based on ZP IMR only, when L1-SINR is configured, interference measurement is performed using ZP IMR

    - FFS: interference measurement is performed using CMR additionally

  - Support of L1-SINR is optional

  - FFS: Support of NZP IMR and ZP IMR are separate UE capabilities

  - Note: CSI-RS above is CSI-RS for BM

However, there was no discussion in Rel-16 for L1-SINR whether the configured IMR should be in the same slot as CMR. In this contribution, we propose to clarify that the restriction above is also applied to aperiodic CMR set and the associated aperiodic IMR (CSI-IM or NZP CSI-RS) set for L1-SINR.

Discussion

In Rel-15, the agreements 1 & 2 in section 1 were agreed to reduce UE complexity and CSI reporting latency. In current spec, they are captured as below:

Section 5.2.1.5.1 TS 38.214

“When aperiodic CSI-RS is used with aperiodic reporting, the CSI-RS offset is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16. The CSI-RS triggering offset has the values of {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} slots. If the UE is not configured with minimumSchedulingOffsetK0 for any DL BWP and minimumSchedulingOffsetK2 for any UL BWP and if all the associated trigger states do not have the higher layer parameter qcl-Type set to 'typeD' in the corresponding TCI states, the CSI-RS triggering offset is fixed to zero. The aperiodic triggering offset of the CSI-IM follows offset of the associated NZP CSI-RS for channel measurement.

…

If interference measurement is performed on aperiodic NZP CSI-RS, a UE is not expected to be configured with a different aperiodic triggering offset of the NZP CSI-RS for interference measurement from the associated NZP CSI-RS for channel measurement.”

L1-SINR based measurement and report was specified in Rel-16, where aperiodic CMR set and the associated aperiodic IMR (CSI-IM or NZP CSI-RS) set are supported, as shown in agreement 3 in section 1. The same benefits of UE complexity and CSI reporting latency can be achieved for L1-SINR when CMR and IMR are within the same slot. However, this discussion was missed for L1-SINR in Rel-16.

Therefore, we propose to clarify that the aperiodic CMR set and the associated aperiodic IMR (CSI-IM or NZP CSI-RS) set for L1-SINR should be in the same slot.

Proposed conclusion: The following in TS 38.214 also applies to L1-SINR.

- The aperiodic triggering offset of the CSI-IM follows offset of the associated NZP CSI-RS for channel measurement.

- If interference measurement is performed on aperiodic NZP CSI-RS, a UE is not expected to be configured with a different aperiodic triggering offset of the NZP CSI-RS for interference measurement from the associated NZP CSI-RS for channel measurement.

Conclusion

In this contribution, we discussed the trigger offset for aperiodic CMR and IMR for L1-SINR, and have the following proposal:

Proposed conclusion: The following in TS 38.214 also applies to L1-SINR.

- The aperiodic triggering offset of the CSI-IM follows offset of the associated NZP CSI-RS for channel measurement.

- If interference measurement is performed on aperiodic NZP CSI-RS, a UE is not expected to be configured with a different aperiodic triggering offset of the NZP CSI-RS for interference measurement from the associated NZP CSI-RS for channel measurement.

References

[1] 3GPP Final Minutes report RAN1#92 v100, RAN1 Chairman

[2] 3GPP Final Minutes report RAN1#92b v100, RAN1 Chairman

[3] 3GPP Final Minutes report RAN1#97 v100, RAN1 Chairman