3GPP TSG RAN WG1 #109-e			                                                                      R1-2204935

e-Meeting, May 9th –20th, 2022

Source:		Mavenir

Title:			On disabling HARQ feedback for IOT-NTN

Agenda Item:		9.12.2

Document for:		Discussion

# Introduction

In previous RAN1 meetings, the following agreements were reached on Enhancements on HARQ for IOT-NTN:

Agreement:[1] Increasing the number of HARQ processes for NB-IoT and for eMTC in NTN is recommended not to be supported in Rel-17. Agreement:[2] Capture the following in the TR: RAN1 discussed reporting of additional information by a UE (such as timing information to inform the network that a sufficient number of repetitions has been transmitted, requested number of repetition, BLER-based triggering or bundling of feedback, buffer status, enabling/disabling HARQ feedback, etc.) Conclusion:[2] RAN1 concluded that reporting of additional feedback is not an essential feature for NTN IoT in Rel-17.

# Discussions

During Rel-17 time frame, there were discussions on the potentially large throughput impact of HARQ stalling, originating from the large RTT delay. Arguably, this was the reason to introduce specification-based HARQ disabling.

However, there was a view that the impact of HARQ stalling is minor. According to [4], the RAN1 specifications allows eNB to send a next PDSCH transmission on a same HARQ process (either initial or re-transmission) after UE completes the PDSCH processing. Here, the PDSCH processing time is specified as k0 msec (or 4 msec for LTE FDD), after which UE transmits HARQ-ACK feedback. This implies that network does not need to wait until UE to receive PDSCH before sending the next PDSCH on a same HARQ processed, even when we have a large propagation delay from eNB to UE. In this case, eNB can schedule next PDSCH in only k0 msec after sending a PDSCH. It should be noted that this “next” PDSCH needs to be scheduled as a new transmission only (with NDI bit toggling), as HARQ-ACK has not been received at the eNB yet.

The PDSCH throughput reduction owing to the PDSCH stalling is 1/k0. The amount of impact seems to be minor, given the typical use cases of NTN IOT.

Subsequently transmitting PDSCH after k0 msec with NDI toggling can be regarded as “standard transparent” HARQ disabling, and allows the network to achieve reasonable DL throughput.

One drawback of this standard transparent implementation is that the HARQ feedback UE transmits are not going to be used for determining HARQ retransmissions. However, the HARQ feedback is still useful for the network for link adaptation perspective.

Given this standards-transparent method is available, further studies are needed if any extra specification is necessary for HARQ disabling.

# Conclusions

In this contribution, we discussed the impact of disabling HARQ feedback for IoT-NTN. Our observations are summarized as follows:

Observations:

- Subsequently transmitting PDSCH after k0 msec with NDI toggling can be regarded as “standard transparent” HARQ disabling, and allows the network to achieve reasonable DL throughput.

  - Although HARQ feedback is not used for determining HARQ retransmission decision, the ACK/NACK is still useful for network’s link adaptation.

- Further studies are needed if any extra specification is necessary for HARQ disabling.

# References

- Chairmain’s Notes, 3GPP RAN1 #104b-e, 2021

- Chairmain’s Notes, 3GPP RAN1 #105-e, 2021

- TS 36.213, version 17.1.0 Release 17 - section 16: UE Procedures related to narrow band IoT

- R1-2104817, On HARQ enhancements for IoT NTN, Ericsson