3GPP TSG RAN WG1 #109-e			                                                                      R1-2204936

e-Meeting, May 9th – 20th, 2022

Source:		Mavenir

Title:			General aspects of AI/ML framework

Agenda Item:		9.2.1

Document for:		Discussion

# Introduction

This tdoc presents Mavenir’s views on general aspects of AI/ML framework, per the following objective captured in the AI/ML SID [1].

AI/ML model, terminology, and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate

# Discussions

During the past several years, AI/ML-powered inference machine has been studied in academia and industry for improving wireless network / UE KPIs.

While training of an AI/ML machine typically requires lots of efforts and data, the inference operation requires manageable computational efforts and data at either at the UE or at the network side. Hence, if pre-trained models with good KPI advantages can be incorporated in the 3GPP signaling framework, the use of AI/ML is likely to be practical and well justified. The potential use cases are especially for those problems where no optimal solutions exist, or they are too much computationally complicated.

AI/ML machine can be incorporated in 3GPP compliant networks in different levels of transparency.

- In a first category (cat 1), AI/ML machine is designed relying only on legacy 3GPP specifications, with aiming to improve network or UE KPIs. Legacy 3GPP specifications already provide good amount of useful data for AI/ML training. Network vendors / operators can collect training data at the CUs/DUs or by means of MDT. For example, for AI/ML use cases like SSB optimizations which aims to find optimal set of beams for the cells, existing 3GPP protocols and UE feedback for beam management and mobility can be used for collecting training data and KPIs.

- In a second category (cat 2), new 3GPP signalling is introduced to facilitate AI machine inference.

  - Cat 2-1: The 3GPP specification provides extra signalling to be used as extra inputs to the AI machine constructed independently by either gNB or UE.

  - Cat 2-2: The 3GPP specification provides a framework along with parameters for the framework, so that UE can construct AI/ML inference machine according to the signalling and the framework. In this case, extra UE feedback mechanism and signalling may need to be defined to facilitate UE feedback of the inference results. The framework is likely to have RAN1 specification impact because UE’s L1 AI/ML architecture needs to be standardized. The parameters for the framework, on the other hand, can be semi-statically configured, e.g., by RRC signalling. Some further comments are available below:

    - The framework should be general enough to support network-initiated upgrade or tuning of the machine, to be able to improve the machine over the time, and/or to adapt to various wireless channels / environments. On the other hand, the framework should also be rigid, so that the machine implemented at the UE can transparently operate even after the network-initiated upgrade and tuning.

    - This category requires large spec efforts. Also, this method imposes restriction on AI/ML designs because the AI/ML designs need to comply with the framework, so the algorithm design freedom can be limited. Therefore, we will need to see significant performance gain to justify spec changes based on this category, and we will need to find a flexible framework that allows for various AI/ML designs within the framework.

- In a third category (cat 3), new 3GPP signalling is introduced to facilitate training or tuning of the AI/ML machine. In this case, the specification may introduce extra UE feedback signalling to be used for the training purpose, which can be used for training of various AI/ML machine at the gNB. To help training L1/L2 AI/ML algorithms, RAN1 specified UE feedback may be necessary. However, more discussion is needed if RAN1 UE feedback can be justified solely for the purpose of network-side AI/ML training, considering the extra overhead, high impact on UE implementations and power consumption consequences.

# Conclusions

This contribution has discussed three main categories of AI/ML incorporation in 3GPP compliant framework.

Proposal: The following 3 categories of AI/ML adoption to 3GPP specifications should be studied:

- Cat 1: AI machine is designed relying only on legacy 3GPP specifications.

- Cat 2: New 3GPP signaling is introduced to facilitate AI machine inference

  - Cat 2-1: The new signaling messages are introduced so that they can be used extra inputs for the AI machine, independently constructed either at gNB or at UE side

  - Cat 2-2: The new signaling messages indicate parameter values for define an AI machine at the UE. The AI machine is constructed according to RAN1 specification defined framework along with those signaled parameters.

- Cat 3: New 3GPP signaling is introduced to facilitate training of tuning of AI machine.

Furthermore:

- RAN1 spec enhancement based on Cat 2 and Cat 3 should only be considered only when there is significant gain over Cat 1.

# References

[1] RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm