3GPP TSG RAN WG1 #109-e			                                                                      R1-2204937

e-Meeting, May 9th – 20th, 2022

Source:		Mavenir

Title:			AI/ML for CSI feedback enhancement

Agenda Item:		9.2.2.2

Document for:		Discussion

# Introduction

This tdoc presents Mavenir’s views on AI/ML for CSI feedback enhancement, per the following objective captured in the AI/ML SID [1].

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project.

# Discussions

For massive MIMO systems, MU-MIMO scheduling algorithm, precoding, and available CSI largely determines network throughput performance. In TDD systems where channel reciprocity holds, gNB can use SRS to obtain the CSI for cell center users. For FDD systems, and for extending the MU-MIMO coverage of TDD systems, CSI-RS and corresponding CSI feedback are needed. Towards this, 3GPP RAN1 has introduced type-1 and type-2 feedback methods and their enhancements in NR specifications of R15 – R17. While impressive performance gains were observed with newer release CSI feedback methods during the study, any new enhancements require new extra implementations and validation efforts from both UE and gNB sides, which might have been delaying wide market adoption of these newer and more advanced CSI feedback methods.

Recently, quite a few literatures have discussed AI/ML based CSI feedback enhancement (e.g., [2], [3]). In particular, auto-encoder based architecture seems to be quite popular for CSI feedback compression purpose. In its end use case in the inference stage, the encoder portion of an auto-encoder resides at UE, and the decoder portion at gNB. The UE encoder takes the CSI-RS based CSI measurement results, and generates an output corresponding to a compressed version of CSI. gNB uses the decoder portion of the auto-encoder to de-compress the feedback CSI and get the full band CSI back. Perhaps owing to the academic nature of these studies, the network-level spectral efficiency gain and overhead reduction of these methods are not yet clearly shown there. Nevertheless, the performance gain looks promising, and it is proposed to study the performance gains of auto-encoder based AI/ML architecture in system-level simulations in this SI.

Proposal 1: RAN1 should study auto-encoder based architecture for AI/ML powered CSI feedback enhancements.

- System-level simulations should be used for characterizing performance advantages of these approaches.

If significant gains in overhead and performance are demonstrated, RAN1 should consider adoption of CSI feedback based on AI/ML auto-encoders. The specification impacts of auto encoder may include (1) AI/ML framework design; (2) parameter signaling for the framework; and (3) CSI feedback design.

The framework of (1) should be flexible enough to allow for various AI/ML encoder implementations. The framework should also be such that one UE implementation can support all the parameter combinations offered by (2), without requiring UE implementation changes. If these requirements are fulfilled for the framework and signaling, CSI feedback performance enhancement can be done with just revising the AI/ML CSI encoder parameters. Therefore, much less UE IOT (inter-operability tests) efforts are anticipated for adopting better performing CSI feedback method. The flexible AI/ML CSI encoder framework is likely to give greater flexibility and advantages for operators.

The CSI encoder is likely to be by and large a pre-trained neural network. There are existing methods to specify a neural network in computer science community (e.g., by means of JSON), and naturally these can be good starting point for 3GPP to specify the protocol to (1) design a framework and (2) design of parameter signaling.

CSI feedback design (3) is likely to be RAN1 leading topic. As 5G NR CSI feedback framework is quite flexible, it is expected that RAN1 can leverage a lot from the existing framework. The details need to be further studied.

Proposal 2: The following 3 areas should be further discussed for specifying the auto-encoder in 3GPP:

- AI/ML framework design

- Parameter signaling for the framework

- CSI feedback design

Proposal 3: Study feasibility of a flexible AI/ML framework and corresponding higher-layer signaling that allows for online upgrade of CSI encoder without requiring UE implementation changes.

# Conclusions

This contribution has discussed AI/ML incorporation for CSI feedback enhancement.

Proposal 1: RAN1 should study auto-encoder based architecture for AI/ML powered CSI feedback enhancements.

- System-level simulations should be used for characterizing performance advantages of these approaches.

Proposal 2: The following 3 areas should be further discussed for specifying the auto-encoder in 3GPP:

- AI/ML framework design

- Parameter signaling for the framework

- CSI feedback design

Proposal 3: Study feasibility of a flexible AI/ML framework and corresponding higher-layer signaling that allows for online upgrade of CSI encoder without requiring UE implementation changes.

# References

[1] RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

[2] Chao Lu, et. al., MIMO Channel Information Feedback Using Deep Recurrent Network, IEEE Commun. Letters, 2019

[3] Qi Li, et. al., A Novel CSI Feedback Approach for Massive MIMO Using LSTM-Attention CNN, IEEE Access, 2020