3GPP TSG RAN WG1 #108-e							                      R1-2204938

e-Meeting, May 9th – 20th, 2022

Source:   	Mavenir Incorporated

Title: 	              Beam Management Enhancement using AI/ML

Agenda item:	9.2.3.2

Document for:	Discussion & Decision

# 1	Background

This tdoc presents Mavenir’s views on beam management enhancement using AI/ML, per the following objective captured in the AI/ML SID [1].

Use cases to focus on:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project.

# 2	Potential Use Cases to Study

We find the following topics in beam management can be enhanced using AI/ML-based algorithms.

2.1 Intra/Inter TRxP Transmit Beam-Sweeping in P-1

P-1 is defined in TR 38.912 as “…to enable UE measurement on different TRxP Tx beams to support selection of TRxP Tx beams/UE Rx beam(s) - For beamforming at TRxP, it typically includes a intra/inter-TRxP Tx beam sweep from a set of different beams” and the beam sweeping procedure is defined as “operation of covering a spatial area, with beams transmitted and/or received during a time interval in a predetermined way”.

However, when a TRxP (e.g., gNB) needs to cover its entire angular domain (i.e., an azimuth span of 360 degree and an elevation span of 180 degree), it needs to transmit a large number of beams in different directions which causes high overhead and interferences to neighbour cells.

In this context, we propose that instead of sweeping beams in all the directions, an AI/ML-based algorithm should determine the optimal directions and periodicity for beam-sweeping based on the user equipment (UE) distributions and channel conditions.

For example, if the UE distribution history indicates that the UEs are geographically clustered in a sector of a gNB and each cluster can be covered by a single beam then, the gNB can transmit beams towards the centres of UE clusters with smaller time interval (e.g., in every 5 ms) and transmit beams in all the directions with higher time intervals (e.g., in every 160 ms). In that way, the gNB covers the UEs that are frequently appearing inside the clusters as well as the UEs that are seldom appearing outside of the clusters (i.e., not considered in the UE distribution history). This reduces the overall number of beam-sweeping directions compare to the case where the gNB transmits beams in all the directions with a constant time interval (e.g., in every 10 ms). However, the coverage remains same in both the cases. In this way, the AI/ML-based algorithm can reduce the P-1 overhead and thus, improve the throughput by reducing the beam-sweeping directions and increasing the beam-sweeping periodicity.

In order to train the AI/ML models, the minimum set of data need to be collected from the network (e.g, UE directions/UE-specific beam directions history, channel history, measurement reports etc.) need to be further studied along with their impact on the existing specifications.

2.2  TRxP Transmit Beam Selections in P-2

P-2 is defined in TR 38.912 as “… to enable UE measurement on different TRxP Tx beams to possibly change inter/intra-TRxP Tx beam(s) - From a possibly smaller set of beams for beam refinement than in P-1”. Specifically, in P-1, TRxP obtains coarse directions of transmit beams using wide beams and in P-2, TRxP obtains finer beam directions by transmitting narrower NZP-CSI-RS beams in different directions within the angular range obtained in P-1. Then, the UE measures all these beams and the best P-2 transmit beam is selected based on the RSRP (i.e., L1-RSRP) of the beams.

However, the frequent communications between TRxP and UE over the CSI-RS and the SRS resources to determine the best P-2 beam causes a certain overhead.

In this context, we propose that instead of relying on instantaneous CSI-RS and SRS measurements, an AI/ML-based algorithm should predict the best P-2 beam or the best set of P-2 beams based on the history of P-1 and P-2 beam measurement reports and the instantaneous P-1 beam measurement report. For example, an AI/ML-based algorithm can map an instantaneous P-1 measurement report to a set of the P-1 measurement reports from the history and pull up the corresponding P-2 transmit beams. Then, select the beam with the highest probability. This would significantly reduce the P-2 overhead and thus, improve the throughput.

In order to train the AI/ML models, the minimum set of data need to be collected from the network (e.g, UE directions/UE-specific beam directions history, channel history, measurement reports etc.) need to be further studied along with their impact on the existing specifications.

2.3 Beam-switching

UEs in connected mode can switch between one beam to another beam based on L1/L2 algorithm. However, due to the temporal and spatial dynamics of channel between UE and TRxP, the beam-switching procedure often suffers in too early switches, too late switches, ping-pong effects and switching failures.

In this context, we propose that an AI/ML based algorithm can optimize the beam-specific cell individual offset (CFO) and Time-to-Trigger (TTT) parameters in real time or, near-real time. This would reduce the cases of too early, too late switches, ping-pong effects and switching failures and thus, reduce the latency and the outage rate.

In order to train AI/ML models, the minimum set of data need to be collected from the network (e.g., CFO, TTT and the corresponding number of too early switches, too late switches, ping-pong cases and switches failures) need to be further studied along with their impact on the existing specifications.

To summarize, we have the following proposal for RAN WG1:

Proposal 1: RAN WG1 should study the application of AI/ML-based algorithms on the following Use Cases to understand if any specification improvement is required:

- Reducing the P-1 overhead by optimizing the intra/inter TRxP (e.g., gNB) transmit beam-sweeping directions and periodicity.

- Reducing the P-2 overhead by predicting the best set of P-2 beams.

- Improving the beam-based mobility by determining beam-specific cell individual offset (CFO) and Time-to-Trigger (TTT).

# 3	Potential KPIs

We believe the following KPIs need to be considered while studying the application of AI/ML-based algorithms in beam management.

3.1 Throughput

One of the key motivations of applying AI/ML-based algorithms in beam management is to reduce the overhead (e.g., P-1, P-2 overhead) without significantly degrading the performances (e.g., overall RSRP coverage). Therefore, network throughput (which reflects coverage as well as overhead) is an important KPI to consider in this study.

3.2 Beam-based Mobility related KPIs

Another motivation of applying AI/ML-based algorithms in beam management is to improve the beam-based mobility. In that case, the set of KPIs regarding beam-switching (e.g., number of too early switches, too late switches, ping-pong cases and switching failures) should be considered.

Proposal 2: RAN WG1 should consider the following KPIs while studying the application of AI/ML-based algorithms in beam management:

- Throughput

- Beam-switching success rate

- Number of too early beam-switches

- Number of too late beam-switches

- Number of ping-pong cases

- Link failure rate

- Outage rate

# 4	Conclusion

In this contribution, we presented our views on beam management enhancements using AI/ML. We summarize our proposals below.

Proposal 1: RAN WG1 should study the application of AI/ML-based algorithms on the following Use Cases to understand if any specification improvement is required:

- Reducing the P-1 overhead by optimizing the intra/inter TRxP (e.g., gNB) transmit beam-sweeping directions and periodicity.

- Reducing the P-2 overhead by predicting the best set of P-2 beams.

- Improving the beam-based mobility by determining beam-specific cell individual offset (CFO) and Time-to-Trigger (TTT).

Proposal 2: RAN WG1 should consider the following KPIs while studying the application of AI/ML-based algorithms in beam management:

- Throughput

- Beam-switching success rate

- Number of too early beam-switches

- Number of too late beam-switches

- Number of ping-pong cases

- Link failure rate

- Outage rate

References:

[1] RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm