3GPP TSG RAN WG1 #109-e			R1-2204940

e-Meeting, May 9th – May 20th, 2022

Source: 	Intel Corporation

Title: 	Views on potential solutions for SL positioning

Agenda item:	9.5.1.3

Document for: 	Discussion and Decision

# Introduction

The study item on expanded and improved NR positioning was approved in [1]. One of the study objectives is introduction of positioning functionality over Sidelink (SL):

Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required]

In this contribution, we express our views on potential enhancements that should be considered to support SL positioning in NR systems.

Bandwidth considerations for SL positioning

Considering that (1) several SL positioning use-cases and their requirements involving 1~3m accuracy with 95+% confidence, and (2) need to support OOC/partial coverage scenarios, the support of SL BW at least up to 100 MHz (FR1)/400 MHz (FR2) would be necessary from the perspective of specification support.

On the other hand, the bandwidths assumed for Rel-16 V2X/SL and Rel-16 positioning studies that include at least 20 MHz (e.g., ITS band) should also be considered. As can be expected, it may be challenging to support high accuracy positioning performance with limited BW configurations unless multiple positioning methods, including non-NR-based methods (e.g., GNSS-RTK assistance data) or enhanced methods like bandwidth aggregation, etc. are considered.

    - Specification support of up to 100 MHz/400 MHz BW for FR1/FR2 (licensed bands) should be considered for SL positioning.

    - In addition, RAN1 should study the performance for SL positioning in limited bandwidth scenarios, e.g., up to 20 MHz.

Positioning methods for SL

Most of the methods specified currently for Uu interface should be considered for support for SL. This includes methods like Time Difference of Arrival (TDOA), Round Trip Time (RTT), and Angle of Arrival(AoA) and Angle of Departure (AoD)-based positioning methods. The same set of methods should be considered for support for SL positioning over the PC5 interface as well.

For scenarios involving positioning using SL only, Road Side Units (RSUs) with known coordinates can serve similar roles as base stations or Transmission Reception Points (TRPs) in DL and/or UL positioning.

One or more of TDoA, AoA/AoD, and RTT based positioning methods may be adapted for positioning utilizing combinations of Uu and PC5 links involving:

- multiple UEs and one or more TRPs,

- one or more UEs, and one or more RSUs, and one or more TRPs, or

- one or more UEs and one or more RSUs.

Adaptations of multi RTT mechanisms wherein Rx-Tx time difference at multiple nodes (UEs, RSUs, or TRPs) can be especially attractive as robust schemes considering typically worse time-sync between UEs and RSUs as against between TRPs.

    - One or more of TDoA, AoA/AoD, and RTT based positioning methods may be adapted for positioning utilizing combinations of Uu and PC5 links involving:

      - multiple UEs and one or more TRPs,

      - one or more UEs, and one or more RSUs, and one or more TRPs, or

      - one or more UEs and one or more RSUs.

Positioning reference signals

Currently, NR-Positioning Reference Signal (NR-PRS) in the Downlink (DL) and Sounding Reference Signals (SRS) for positioning in the Uplink (UL) are supported for positioning over the Uu interface.

Sidelink Positioning Reference Signals (SL-PRS) may be defined based on either NR-PRS or SRS-for-positioning or both. Considering OFDM waveform specified for SL, both reference signal options are feasible for support over SL. While NR-PRS offers reuse of the DL PRS receiver at UEs, it would require new capability at UEs to transmit NR-PRS over SL. On the other hand, with SRS-for-positioning, existing implementations at network side for reception of SRS-for-positioning may be utilized.

In terms of ability to support multiplexing between SL-PRS resources and resource sets from same (e.g., for Tx beam sweeping) or different UEs, both NR-PRS and SRS-for-positioning offer good flexibility with flexibility allocations in time and frequency domain with support of flexible numbers of symbols in a slot, time offsets for starting symbol within a slot, frequency offsets with a carrier as well as RE offsets within a PRB, and numerous comb options: 2, 4, 6, 12 for NR-PRS and 2, 4, and 8 for SRS-for-positioning.

For either case, existing Rel-16 NR designs should be maximally reused.

    - For SL-PRS design, both NR-PRS and SRS-for-positioning should be studied for potential support.

Resource allocation for SL-PRS

In the rest of the contribution, unless mentioned explicitly, “SL-PRS” is used to imply either SL-PRS based on NR-PRS or based on SRS-for-positioning.

In general, resource pool(s) may be defined for SL-PRS in a dedicated manner. Such an approach may be suitable for efficient multiplexing of transmissions of SL-PRS from one or more UEs when such transmissions are not accompanied with PSSCH transmissions.

On the other hand, SL-PRS transmissions may also be multiplexed with PSSCH in a slot, e.g., as specified for CSI-RS transmissions in the SL. In such a case, SL-PRS may be mapped to one or more resource pool(s) configured by a serving gNB for PSSCH.

As for PSCCH or PSSCH resource pools, UEs may be pre-configured with appropriate resource pools for SL-PRS transmissions for use when in partial or out-of-coverage (OOC) situations.

Depending on the exact design, in addition to the above, the option of transmitting SL-PRS following (pre-)configured configurations and mapped directly to SL BWP without use of resource pools as for S-SS/PBCH block transmissions may be considered as well.

    - For SL-PRS resource allocation, resource pool(s) may be (pre-)configured either dedicated for SL-PRS or shared with resource pool(s) (pre-)configured for PSSCH.

    - Both cases wherein SL-PRS may or may not be multiplexed with PSSCH in a slot from a transmitting UE should be considered for further study.

Similar to SL communications, resources for SL-PRS transmission/reception may be either configured by the gNB or autonomously selected from resource pools. Next, we consider some potential design choices for either option.

In terms of identifying destination (receiving) UEs for SL-PRS, both unicast and groupcast communication framework should be considered for support. Further, for a given Tx UE that may be configured for groupcast for both communication and SL-PRS transmission, the grouping of destination UEs for groupcast for SL-PRS may be separate from that for communication.

    - For SL-PRS transmissions, both unicast and groupcast-based identification of destination UEs should be considered.

    - RAN1 to investigate potential solutions for both gNB-controlled resource allocation as well as UE-autonomous resource selection for SL-PRS.

## gNodeB-controlled resource allocation for SL-PRS

In general, a UE may be provided with configurations for transmission or reception of SL-PRS respectively by the Location Management Function (LMF) via the serving gNB for in-coverage UEs and may be pre-configured for use in Out-of-Coverage (OoC) situations. The corresponding configurations defined for NR-PRS and SRS-for-positioning may be considered as a starting point for SL-PRS configurations depending on the choice of the positioning reference signal.

For NR-PRS-based SL-PRS, different PRS resource sets may be assigned to different UEs while each UE could be provided with multiple PRS resources, e.g., for Tx beam sweeping. Similar to NR-PRS, each SL-PRS resource set and resources within each set can be indexed and identifiable, for eventual reporting back along with measurements as defined for the Uu interface.

Further, as part of SL-NRS configuration, muting patterns may also be configured by a serving gNB to coordinate transmissions of SL-NRS from a number of UEs and/or RSUs in a geographical area.

Although resources for SL-PRS may be provided by a serving gNB, in order to support inter-cell or partial coverage scenarios, it would also be necessary for a transmitting UE to be able to announce the SL-PRS configuration and scheduling information for receiving UEs in other cells or those that may be out of coverage.

Accordingly, some further options may be considered to enable such indication over the SL. These are elaborated next.

- Opt. 1: SL-PRS configuration may be provided in a PSSCH that triggers periodic resources for SL-PRS with the first occasion/slot defined via an offset with respect to the slot in which the PSSCH is received. Further, the following sub-options may be considerable for Opt. 1:

  - Opt. 1A: For the slots with SL-PRS, PSCCH may not be transmitted. This option can be considered for cases when SL-PRS is not multiplexed with PSSCH in a slot.

  - Opt. 1B: For the slots with SL-PRS, PSCCH may be transmitted that schedule a PSSCH and may also convey part of the scheduling information for SL-PRS in the slot.

- Opt. 2: SL-PRS configuration may be provided in a PSCCH (as SCI) that either (i) schedules a single instance of SL-PRS transmission in the slot, or (ii) triggers periodic resources for SL-PRS with the first occasion/slot defined via an offset with respect to the slot in which the PSSCH is received. Note that, for (i), SL-PRS configuration in a PSCCH scheduling a single instance of SL-PRS transmission can include multiple slots of SL-PRS transmissions. Further, the following sub-options may be considerable for Opt. 2:

  - Opt 2A: Each resource for SL-PRS in a slot may be associated with a single PSCCH.

  - Opt 2B: Alternatively, each resource set for SL-PRS may be associated with a single PSCCH with a single SCI indicating potentially multiple resources, e.g., for Tx beam sweeping.

In addition to the above options, some of which aim to offer mechanisms to control the signalling overhead (OH) over SL, other options to minimize the associated OH from transmission of PSCCH and PSSCH should be considered. These may include consideration of use of only one of the two stages of SCI carried in a PSCCH.

    - RAN1 to investigate potential solutions for conveying SL-PRS configuration and scheduling information from a transmitting UE over SL.

      - For different requirements and use-cases (in-coverage, inter-cell, partial coverage), different options that enable trade-off between flexibility, overhead, and reliability should be studied carefully.

In general, it would be desirable not to increase SCI format sizes for PSCCH monitoring. Thus, if the currently defined formats for first and second stages of SCI are to be re-used for SL-PRS-related scheduling information, solutions to distinguish such SCI as against those for SL communications (PSSCH scheduling) should be explored. These could include application of different scrambling for the CRC for one or both stages of SCI.

    - If the currently defined formats for first and second stages of SCI are to be re-used for SL-PRS-related scheduling information, solutions to distinguish such SCI as against those for SL communications (PSSCH scheduling) should be explored.

## UE-autonomous resource selection for SL-PRS

As a baseline option, Mode 2 resource allocation for PSSCH, using resource reservation indication via PSCCH, should be considered for transmission of SL-PRS. In this case, SL-PRS may be transmitted within the symbols indicated for PSSCH with PSSCH being rate-matched around the SL-PRS REs.

Towards this, for indication of SL-PRS resource set and resource configurations, including periodicity, frequency location within resource pool or SL BWP, etc. may be provided to the receiving UE(s) by one or more of: (1) the transmitting UE via PSSCH or PSCCH, or (2) as gNB assistance information via higher layers mapping source IDs to PRS resource set/resources, or (3) a suitable combination of (1) and (2).

    - As a baseline mechanism, Mode 2 resource allocation for PSSCH, using resource reservation indication via PSCCH, should be considered for transmission of SL-PRS.

      - FFS: Detailed options for indication of SL-PRS configuration and scheduling information, including potential availability of assistance information from serving gNB for in-coverage scenarios.

Further, considering potentially different BW requirements for a PSSCH and SL-PRS, options that allow for appropriate decoupling of the transmission bandwidth over a number of subchannels may be studied further.

Solutions that enable trade-off against signalling OH should be explored further for UE-autonomous resource selection methods as well. These could include potential enhancements to resource reservation mechanisms currently defined for SL communications.

Also, it would be necessary to enable coordination of muting patterns between a number of transmitting UEs. Towards this, potential adaptations to the currently defined pre-emption mechanism for UE-autonomous resource selection could be considered.

For SL-PRS configurations, including resource indication and scheduling for SL-PRS over SL, UE-autonomous resource selection for SL-PRS may be defined using one or combinations of the options listed under gNB-controlled resource allocation schemes.

    - RAN1 to study suitable options for efficient signalling of SL-PRS configurations and scheduling information based on potential enhancements to resource reservation mechanism currently specified for PSSCH transmissions.

    - RAN1 to study potential ways to realize coordination and muting patterns across a number of transmitting UEs in a distributed manner.

Measurements, measurement reporting, and coordinate estimation

Most of the currently defined measurements for positioning should be studied for SL positioning. These include RSTD, PRS-RSRP/SRS-RSRP, AoD, and Rx-Tx measurements at receiving UE. Further, AoA/ZoA measurements can be optionally supported for some cases, e.g., at RSUs.

In terms of measurement reporting, they may be reported to gNB or to another UE(/RSU) and include references for PRS resource set and resource IDs.

In general, the measurement reports can be routed to the LMF via a serving gNB. Similar to Uu positioning, GNSS-RTK assistance data or other assistance information may be broadcasted by serving cell when in coverage.

In addition, broadcast of assistance information by RSUs could be helpful for various V2X use-cases.

    - For different positioning methods, consider support of the currently defined measurements that may include RSTD, SL-PRS-RSRP, AoD, and Rx-Tx measurements.

    - AoA/ZoA measurements may also be considered for optional support, e.g., by RSUs.

Conclusion

In this contribution, we provided our views on potential enhancements for support of SL positioning in NR systems. In summary, we have following proposals:

    - Specification support of up to 100 MHz/400 MHz BW for FR1/FR2 (licensed bands) should be considered for SL positioning.

    - In addition, RAN1 should study the performance for SL positioning in limited bandwidth scenarios, e.g., up to 20 MHz.

    - One or more of TDoA, AoA/AoD, and RTT based positioning methods may be adapted for positioning utilizing combinations of Uu and PC5 links involving:

      - multiple UEs and one or more TRPs,

      - one or more UEs, and one or more RSUs, and one or more TRPs, or

      - one or more UEs and one or more RSUs.

    - For SL-PRS design, both NR-PRS and SRS-for-positioning should be studied for potential support.

    - For SL-PRS resource allocation, resource pool(s) may be (pre-)configured either dedicated for SL-PRS or shared with resource pool(s) (pre-)configured for PSSCH.

    - Both cases wherein SL-PRS may or may not be multiplexed with PSSCH in a slot from a transmitting UE should be considered for further study.

    - For SL-PRS transmissions, both unicast and groupcast-based identification of destination UEs should be considered.

    - RAN1 to investigate potential solutions for both gNB-controlled resource allocation as well as UE-autonomous resource selection for SL-PRS.

    - RAN1 to investigate potential solutions for conveying SL-PRS configuration and scheduling information from a transmitting UE over SL.

      - For different requirements and use-cases (in-coverage, inter-cell, partial coverage), different options that enable trade-off between flexibility, overhead, and reliability should be studied carefully.

    - If the currently defined formats for first and second stages of SCI are to be re-used for SL-PRS-related scheduling information, solutions to distinguish such SCI as against those for SL communications (PSSCH scheduling) should be explored.

    - As a baseline mechanism, Mode 2 resource allocation for PSSCH, using resource reservation indication via PSCCH, should be considered for transmission of SL-PRS.

      - FFS: Detailed options for indication of SL-PRS configuration and scheduling information, including potential availability of assistance information from serving gNB for in-coverage scenarios.

    - RAN1 to study suitable options for efficient signalling of SL-PRS configurations and scheduling information based on potential enhancements to resource reservation mechanism currently specified for PSSCH transmissions.

    - RAN1 to study potential ways to realize coordination and muting patterns across a number of transmitting UEs in a distributed manner.

    - For different positioning methods, consider support of the currently defined measurements that may include RSTD, SL-PRS-RSRP, AoD, and Rx-Tx measurements.

    - AoA/ZoA measurements may also be considered for optional support, e.g., by RSUs.

References

- RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel, December 2021.