3GPP TSG-RAN WG1 Meeting #109-e	R1-2204944

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.16.5

Source:	Ericsson

Title:	Views on NR positioning Enhancements UE features

Document for:	Discussion, Decision

- Introduction

This contribution provides our view on the list of UE features for NR positioning in release17, based on the UE feature discussion outcome captured in R1-2202929.

- UE Features for NR positioning enhancements

## (27-3-1) M-sample measurements

The agreed feature 27-3-1 indicates whether the UE supports measuring M=1 samples.  However, in the UE feature table, there is some text regarding candidate values and FFS for other values.  Given that this feature only indicates support for M=1, we propose to remove the text related to candidate values as follows:

| 27. NR_pos_enh | 27-3-1 | M-sample measurements | The capability to support reporting a measurement based on measuring M=1 samples (instances) of a DL PRS resource set | 13-1 | No |  | If the UE does not provide the capability, the UE is assumed to support M=4 only | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported Note: The sample number M=1 does not account for the potential AGC sample Note: this feature is supported for both UE-assisted and UE based positioning | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## (27-10a) Low latency MG activation request for PRS measurements

| 27. NR_pos_enh | 27-10a | Low latency MG activation request for PRS measurements | support of low latency MG activation request for PRS measurements | [27-10] |
| --- | --- | --- | --- | --- |

FG 27-10a informs the LMF that the UE supports the use of low latency activation of MG configurations.  Based on the information, the LMF can request the gNB to reconfigure the MG in UE. the UE should not have to report this capability if it is not capable to receive fast DL MAC CE reconfigurations.  Therefore we think that 27-11 should be a prerequisite, instead of 27-10, which signals the UL MAC CE capability.

The following change is proposed:

| 27. NR_pos_enh | 27-10a | Low latency MG activation request for PRS measurements | support of low latency MG activation request for PRS measurements | [27-11] |
| --- | --- | --- | --- | --- |

## (27-15b) Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP

During RAN1#108e, two components were left in yellow for FG 27-15b.  both component 5 and 6 may impact the required retuning time when the SRS needs to be transmitted outside of the initial BWP.  if the numerology is the same and the SRS overlap with the initial BWP, retuning time could be smaller compared to when the UE needs to readjust numerology and move to the initial BWP.  a separate FG for the case of different numerology and with SRS BW outside of coreset0 / SSB could thus be considered.

| 27. NR_pos_enh | 27-15b | Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP | - Maximum SRS bandwidth supported for each SCS that UE supports within a single CC - Max number of SRS Resource Sets for positioning supported by UE - Max number of periodic SRS Resources for positioning - Max number of periodic SRS Resources for positioning per slot - [Different numerology between the SRS and the initial UL BWP is supported] - [SRS operation without restriction on the BW: BW of the SRS may not include BW of the CORESET#0 and SSB] - Max number of P/SP SRS Resources for positioning - Max number of P/SP SRS Resources for positioning per slot - FFS: center frequenecy | 27-15 |
| --- | --- | --- | --- | --- |

Component 5 and 6 of FG 27-15b can be moved to a new, separate FG with 27-15b as a prerequisite.

## (27-20) PRS subset association for UE assisted DL-AoD

| 27. NR_pos_enh | 27-20 | PRS subset association for UE assisted DL-AoD | 1. Support of assistance data enhancement to indicate a subset of PRS resources for each PRS resource for the purpose of prioritization of DL-AoD reporting. 2. Supported resource set relationship for the target PRS resource and the associated subset [3. Support associated subset measurement reporting] |
| --- | --- | --- | --- |

The support of subset measurement reports is not a necessary component of the FG. The report of the subset is no different from reports from the main PRS. therefore we think the component #3 can be removed from 27-20.

Component 3 of FG 27-20 can be removed.

# Conclusion

We have proposed to modify/append 4 UE features for NR positioning enhancements.

- References

- R1-2202929,   Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e including remaining RAN1 issues, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#108e