3GPP TSG-RAN WG1 Meeting #109-e	R1-2204947

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.16.12

Source:	Ericsson

Title:	Views on UE features for NR MBS

Document for:	Discussion, Decision

- Introduction

This contribution provides our view on the list of UE features for NR MBS in release 17,  based on the  updated RAN1 UE features list for Rel-17 NR captured in in R1-2202929[1].

- UE Features for NR MBS

## (33-1)  Broadcast feature

During RAN1#108-e, the use of DCI indicated slot level repetition was agreed to be moved to another FG, with FG 33-3-1 as a candidate for merging.

| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. - support of higher layer configured slot-level repetition up to 8 for MTCH DCI indicated slot-level repetition up to 16 for MTCH is defined as another FG, FFS whether to merge with FG 33-3-1 |
| --- | --- | --- | --- |

FG 33-3-1 is a feature group that includes dynamic slot level repetition for G-PDSCH for multicast, as it is depended on FG 33-2, the basic FG for multicast. In order for broadcast UEs to be able to support dynamic repetition witout having to support multicast, we propose to separate the broadcast support from multicast support by either adding 33-1 as a prerequisite to 33-3-1, or creating a separate FG .

Support Dynamic Slot-level repetition for group-common PDSCH for broadcast as part of 33-3-1. At least one of FG 33-1 or FG 33-2 should be supported by the UE as prerequisite.

Proposed changes:

| 33. NR_MBS | 33-3-1 | Dynamic Slot-level repetition for group-common PDSCH | - Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast. | 33-2 Or 33-1 Note: 33-1 prerequisite to support the feature for broadcast 33-2 prerequisite to support the feature for multicast |
| --- | --- | --- | --- | --- |

## (33-2a) Support of ACK-NACK based feedback.

There is a remaining FFS for FG 33-2a regarding whether to define a separate FG for separate PUCCH configuration from unicast.

| 33. NR_MBS | 33-2a | Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling 2) Support of PTM retransmission for multicast 3) support of Type-1 and Type-2 HARQ-ACK CB for multicast feedback only 4) Support of shared PUCCH resource configurations with unicast FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a |
| --- | --- | --- | --- |

In our view, it would be beneficial for feature uptake to have a separate FG for support of separate PUCCH configured for multicast. It is possible to support multicast and reuse unicast PUCCH, especially if NACK only is not supported/not in use. Therefore, a separate feature group can be signalled.

Define support for a separate PUCCH resource configuration  from unicast as separate FG from FG 33-2a

## (33-5-1h) SPS for multicast

For SPS for multicast, the number of G-CS-RNTI for SPS multicast is defined (in bracket so far) per slot per CC:

| 33. NR_MBS | 33-5-1h | Multiple G-CS-RNTIs for SPS group-common PDSCHs | Max number of G-CS-RNTIs for SPS multicast [per slot per CC] | 33-5-1 | Yes |  |  | [Per UE] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

We do not think the clarification “per CC” is needed. The multicast main feature group 33-2 is already define as per FSPC.  Similarly, we do not see the use of “per slot”. A single multicast PDSCH per slot is supported by rel17, with the possibility to also have TDM or FDM of unicast in the slot if supported.

Remove “per slot per CC” from the description of FG 33-5-1h

# Conclusion

We have proposed to modify/append 4 UE features for NR MBS.

- References

[1] R1-2202929,   Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e including remaining RAN1 issues, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#108e