3GPP TSG-RAN WG1 #109	R1-2204949

e-Meeting, May 9th-20th, 2022

Agenda Item:	9.5.1.2

Source:	Ericsson

Title:	Simulation assumptions and evaluations for  NR SL positioning

Document for:	Discussion, Decision

# Introduction

The study on expanded and improved NR positioning SID [1] states that the study should include an analysis of the scenario and requirements for sidelink positioning:

Scenario/requirements

  - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage

  - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104

  - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104)

  - Spectrum: ITS, licensed

In this contribution we provide our view on simulation assumptions for the different coverage scenarios applicable to the agreed use cases.

# Discussion

## General considerations

For in-coverage scenarios and partial coverage (communication), it can be assumed that either LPP-based measurement reports can be sent to the LMF, or that UE base positioning can be assisted by the LMF.  For out of coverage scenarios, the reports and configuration exchanges are between UEs.  The evaluation methodology should however be independent on how the UE or TRP acquires the relevant information for positioning (e.g directly or via a relay).

Evaluation methodology does not include how the UE/TRP is provided with the positioning configuration

Similar to rel17, we propose not to define any baseline reference signals in the evaluation methodology. Configurations of DL PRS and SRS supported by Rel-16 and rel-17 specifications are used for evaluation of the achievable performance based on Rel-17 positioning technologies.

Do not   define any baseline reference signals in the evaluation methodology

In the following sections, we describe the required gNB (TRP) and UE parameters as well as channel models the gNB and UE models which should apply to all coverage cases. Channel models and deployment setup (including dropping procedure and TRP deployments) depends on the deployment and coverage scenarios.

UE and gNB parameters are common for all use cases, with FR1/FR2 parameter differentiations

## Channel models and deployment models for SL positioning

Channel models for SL communication has been agreed and used during 3GPP standardization work, and are presented in TS 38.885. In our view, these channel models as well as the parametrizations should be reused as much as possible. Additionally, we acknowledge that suitable 3GPP SL channel models are missing in some scenarios, and we believe that some pragmatism is needed for these cases. We propose that for the use cases where models are missing, suitable Uu models can be used, with suitable parametrization. Our choice and motivations are provided below.

### Common parameters for SL positioning evaluations

the studies for NR positioning support in Rel-16 and NR-positioning enhancements in Rel-17 already developed models for indoor and outdoor gNBs, including urban and macro deployment. In our view, these models should be sufficient to be re-used as is by the study in Rel-18 as the common parameters.

Reuse Table 6.1  from 38.857 for common parameters for evaluations in Rel-18

### Parameters and channel models for indoor use cases:

Indoor use cases include IIOT, public safety and commercial use cases.  We note that for indoor use cases, we have established that coverage was normally not an issue and as little as 1 symbol was needed to obtain good positioning performance. Therefore, we propose not to evaluate different UE speed for these cases, as the positioning solution may not need to rely on long coherence time indoor.

For indoor evaluation, a single UE speed of 3m/s is used.

The IIOT use case can re-use the rel17 IIOT scenario. For UE to UE evaluations (relative positioning / ranging), the UE model can be used at both ends of the link.

Reuse Table 6.1-1  from 38.857 for parameters for evaluations of IIOT use indoor use cases in Rel-18. For UE to UE evaluations (relative positioning / ranging), the UE model can be used at both ends of the link.

For the TRP-UE link in indoor only commercial use cases, we propose to re-use rel16/rel17 indoor open office evaluation settings.

Reuse Table 6.1-1-3 from 38.855 for parameters for evaluations of commercial use cases in Rel-18. For UE to UE evaluations (relative positioning / ranging), the UE model can be used at both ends of the link.

For the public safety indoor use case,  a model including UEs in different floors should be used.  As there is no current model ready for evaluation of the channel between UEs across floors, RAN1 should discuss what approach to take.

The table below shows a summary of scenarios assumption for indoor evaluation:

Table 1 Summary of indoor  scenarios simulation assumptions

| Scenario | Use case | Simulation assumption Note: Table 6.1 from 38.857 is re-used for common parameters across scenarios |
| --- | --- | --- |
| Scenario 1 | Indoor, IIOT | Table 6.1-1 from 38.857 |
| Scenario 2 | Indoor, commercial | Table 6.1-1-3 from 38.855 |
| Scenario 3 | Indoor, public safety | FFS for device to device Table 6.1-1-3 from 38.855 for TRP to UE. |
| Note: for UE-UE links in the scenarios, the UE models for the evaluated scenario is used at both ends of the link. |  |  |

### Parameters and channel models for outdoor use cases:

For all outdoor use cases (public safety, commercial, industrial),  device to device evaluations should re-use v2v simulation assumptions from 38.885. the channel model details should be discussed, among which:

- LOS/NLOS for v2v

- CDL model for fast fading (urban, highway)

For the commercial  use case,  all use cases described 22.261 have a LOS channel for ranging. Thus for outdoor device to device links, v2v channel model can be reused, while the rel16 and rel17 Umi model can be reused for outdoor TRP-UE links.  For public safety and industrial IOT use cases, the situation is less clear and should be discussed.

Re-use v2v channel models for UE to UE links in outdoor scenarios

- FFS: CDL channel model for fast fading

For outdoor scenario commercial use cases, use the LOS assumption for path loss between UEs .

  - FFS: LOS assumption between UEs for outdoor scenarios of public safety and IIOT.

For TRP-to-UE links, the rel16 and rel17 Umi model can be reused for outdoor TRP-UE links.

Reuse Table 6.1-1-4 from 38.855 for parameters for evaluations of TRP to UE  links in  outdoor use cases in Rel-18.

The table below  shows a summary of scenarios assumption for outdoor evaluation:

Table 2 Summary of outdoor scenarios simulation assumptions

| Scenario | Use case | Simulation assumption Note: Table 6.1 from 38.857 is re-used for common parameters across scenarios |
| --- | --- | --- |
| Scenario 4 | outdoor, IIOT | Table 6.1-1-4 from 38.855 for outdoor TRP-UE links v2v channel models for UE to UE links FFS: LOS condition |
| Scenario 5 | outdoor, commercial | Table 6.1-1-4 from 38.855 for outdoor TRP-UE links v2v channel models for UE to UE links in LOS conditions |
| Scenario 6 | outdoor, public safety | Table 6.1-1-4 from 38.855 for outdoor TRP-UE links v2v channel models for UE to UE links FFS: LOS condition |
| Note: for UE-UE links in the scenarios, the UE models for the evaluated scenario is used at both ends of the link. |  |  |

# Conclusions

In the previous sections we made the following observations:

No table of figures entries found.Based on the discussion in the previous sections we propose the following:

Proposal 1	Evaluation methodology does not include how the UE/TRP is provided with the positioning configuration

Proposal 2	Do not   define any baseline reference signals in the evaluation methodology

Proposal 3	UE and gNB parameters are common for all use cases, with FR1/FR2 parameter differentiations

Proposal 4	Reuse Table 6.1  from 38.857 for common parameters for evaluations in Rel-18

Proposal 5	For indoor evaluation, a single UE speed of 3m/s is used.

Proposal 6	Reuse Table 6.1-1  from 38.857 for parameters for evaluations of IIOT use indoor use cases in Rel-18. For UE to UE evaluations (relative positioning / ranging), the UE model can be used at both ends of the link.

Proposal 7	Reuse Table 6.1-1-3 from 38.855 for parameters for evaluations of commercial use cases in Rel-18. For UE to UE evaluations (relative positioning / ranging), the UE model can be used at both ends of the link.

Proposal 8	Re-use v2v channel models for UE to UE links in outdoor scenarios

-	FFS: CDL channel model for fast fading

Proposal 9	For outdoor scenario commercial use cases, use the LOS assumption for path loss between UEs .

a.	FFS: LOS assumption between UEs for outdoor scenarios of public safety and IIOT.

Proposal 10	Reuse Table 6.1-1-4 from 38.855 for parameters for evaluations of TRP to UE  links in  outdoor use cases in Rel-18.

# References

RP-213588, Study on expanded and improved NR positioning,  RAN#94e, 2021

# Appendix 1: Rel16 Scenarios and system parameters for positioning evaluations

Table 6.1.1-1: Common scenario parameters applicable for all scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 2GHz, 4GHz – Note 1 | 30 GHz – Note 1 |
| Bandwidth, MHz | 5MHz, 50MHz for 2GHz 100MHz for 4GHz | 100MHz, 400MHz |
| Subcarrier spacing, kHz | 15kHz for 5MHz and 50MHz 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU - Optional: Provided by company |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Table 6.1.1-2: UE antenna radiation pattern model 1 (FR2)

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in dim (dB) |  |
| Antenna element radiation pattern in dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |
|  | Note: are in local coordinate system. |

Table 6.1.1-3: Scenario 1. Indoor office scenario parameters

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Layout | Indoor floor: (12BSs per 120m x 50m), TRP number per floor:12, Inter-gNB distance = 20m - Note 1 |  |
| Total gNB TX power, dBm | 24dBm | 24dBm EIRP should not exceed 58 dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |
| gNB antenna radiation pattern | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |
| Channel model | Indoor open office – Note 2 |  |
| Peneteration loss | 0dB |  |
| Number of floors | 1 |  |
| UE drop procedure | 100% indoor, uniformly distributed over the horizontal area |  |
| UE mobility | 3km/h |  |
| UE antenna height | 1.5m |  |
| Min gNB-UE distance (2D), m | 0m |  |
| gNB antenna height | 3m |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Table 6.1.1-4: Scenario 2: Urban micro (UMi) scenario parameters

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Layout | Hexagonal grid, 19 or 7 macro sites, 3 sectors per site, ISD = 200m, Note 1 Wrap-around is applied, Note 2 |  |
| Total gNB TX power, dBm | 44dBm | 37dBm per panel EIRP should not exceed 73dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1), (dH, dV) = (0.5, 0.8)λ, - Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2), (dH, dV) = (0.5, 0.5)λ, (dg,H,dg,V) = (4.0, 2.0)λ, - Note 1 |
| gNB antenna radiation pattern | Directional, 8dBi – Note 1 Table 6.1.1-5 | Directional, 8dBi – Note 1 Table 6.1.1-5 |
| Channel model | UMi Street Canyon – Note 3 |  |
| Penetration loss | For outdoor UEs: 0dB |  |
| Number of floors | All UEs are on the ground. |  |
| UE drop procedure | 100% outdoor uniformly distributed over the horizontal area |  |
| UE mobility (for modeling Doppler effects) | Outdoor: 3km/h |  |
| UE height, m | 1.5m |  |
| Min. gNB-UE distance (2D), m | 10m |  |
| gNB antenna height | 10 m by default. Companies can bring results with uniform distribution [5-20]m​ |  |
| Note 1: According to 3GPP TR 38.802 Note 2: In case if interference considerations are not properly taken into account for 7 sites companies are encouraged to provide results for 19 sites. Note 3: According to 3GPP TR 38.901 |  |  |

Table 6.1.1-5: gNB antenna patterns for UMi and UMa

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8dBi |

Table 6.1.1-6: Scenario 3: Outdoor macro (UMa) scenario parameters

|  | FR1 Specific Values |
| --- | --- |
| Layout | Hexagonal grid, 3 sectors per site, 7 or 19 macro sites, ISD = 500m – Note 1 Wrap-around is applied. Note 2 |
| Total gNB TX power, dBm | 49dBm |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1), (dH, dV) = (0.5, 0.8)λ – Note 1 Applicable for 2GHz and 4 GHz carrier frequency. |
| gNB antenna radiation pattern | Directional, 8dBi – Note 1, Table 6.1.1-5 |
| Channel model | UMa scenario – Note 3 |
| Penetration loss | For outdoor UEs: 0dB For indoor UEs: 20dB+0.5d2D-in – Note 3 |
| Number of floors, (floor height) | 8, (3m) |
| Antenna Height: | Uniformly distributed [20-50] m – Note 4 25m + α, where α~uniform[-5, 25] |
| UE Height | 3(nfl – 1) + 1.5 m – Note 4 where, nfl ~ uniform(1,Nfl) and Nfl = 8 |
| UE dropping procedure | 50% indoor and 50% outdoor uniformly distributed over the horizontal area (separate statistic) |
| Min. gNB-UE distance (2D), m | 35m |
| UE mobility (for modeling Doppler effects) | For indoor UEs: 3km/h For outdoor UEs: 60km/h |
| Note 1: According to 3GPP TR 38.802 Note 2: In case if interference considerations are not properly taken into account for 7 sites companies are encouraged to provide results for 19 sites. Note 3: According to 3GPP TR 38.901 Note 4: According to 3GPP TR 37.857 |  |

# Appendix 2: Rel-17 Additional scenarios and channel models for NR positioning enhancements

Table 6-1: Common scenario parameters applicable for all scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 3.5GHz | 28GHz |
| Bandwidth, MHz | 100MHz | 400MHz |
| Subcarrier spacing, kHz | 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Baseline: Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU Optional: 4-panels UE: - The antenna elements of the same polarization of the same panel is virtualized into one TXRU |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns (Optional) |  |
| UE/gNB RX and TX timing error | (Optional) The UE/gNB RX and TX timing error, in FR1/FR2, can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of T1 ns, with truncation of the distribution to the [-T2, T2] range, and with T2=2*T1: - T1: X ns for gNB and Y ns for UE - X and Y are up to sources - Note: RX and TX timing errors are generated per panel independently Apply the timing errors as follows: - For each UE drop, - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*Y,2*Y] and another random sample for the Rx error according to the same [-2*Y,2*Y] distribution. - For each gNB - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*X,2*X] and another random sample for the Rx error according to the same [-2*X,2*X] distribution. - Any additional Time varying aspects of the timing errors, if simulated, can be left up to each company to report. - For UE evaluation assumptions in FR2, it is assumed that the UE can receive or transmit at most from one panel at a time with a panel activation delay of 0ms. |  |
| Note 1: According to TR 38.802 Note 2: According to TR 38.901 |  |  |

Table 6.1-1: Parameters common to InF scenarios

|  |  | FR1 Specific Values |  | FR2 Specific Values |
| --- | --- | --- | --- | --- |
| Channel model |  | InF-SH, InF-DH |  | InF-SH, InF-DH |
| Layout | Hall size | InF-SH: (baseline) 300x150 m (optional) 120x60 m InF-DH: (baseline) 120x60 m (optional) 300x150 m |  |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |  |
|  | Room height | 10m |  |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |  |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |  |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |  |
| Penetration loss |  | 0dB |  |  |
| Number of floors |  | 1 |  |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be - (baseline) at least the convex hull of the horizontal BS deployment. - (optional) It can also be the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. |  |  |
| UE antenna height |  | Baseline: 1.5m (Optional): uniformly distributed within [0.5, X2]m, where X2 = 2m for scenario 1(InF-SH) and X2= for scenario 2 (InF-DH) |  |  |
| UE mobility |  | 3km/h |  |  |
| Min gNB-UE distance (2D), m |  | 0m |  |  |
| gNB antenna height |  | Baseline: 8m (Optional): two fixed heights, either {4, 8} m, or {max(4,), 8}. |  |  |
| Clutter parameters: {density , height ,size } |  | Low clutter density: {20%, 2m, 10m} High clutter density: - Baseline): {40%, 2m, 2m} for fixed UE antenna height and gNB antenna height - (Optional): {40%, 3m, 5m} - (Optional): {60%, 6m, 2m} |  |  |
| Note 1: According to Table A.2.1-7 in TR 38.802 |  |  |  |  |