3GPP TSG-RAN WG1 #109	R1-2204950

e-Meeting, May 9th-20th, 2022

Agenda Item:	9.5.1.3

Source:	Ericsson

Title:	Methods for NR SL ranging and positioning

Document for:	Discussion, Decision

# Introduction

The study on expanded and improved NR positioning  SID [1] states that the study should include an analysis of the scenario and requirements for sidelink positioning:

- Scenario/requirements

  - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage

  - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104

  - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104)

  - Spectrum: ITS, licensed

In this contribution we provide our view on different solutions for ranging and positioning in the different coverage scenarios.

# Discussion

## Ranging

There are many wireless ranging solutions available commercially, and there is a surge of such solutions of late. Mostly these solutions are ultra-wideband (UWB) based, and 3GPP based ranging solutions can provide complementary and competing ranging features to these existing systems. One benefit the 3GPP-based ranging solutions can leverage is the existence of fixed and differently placed (than UWB devices) radio infrastructure, such as radio base stations having fixed coordinates and bigger radio horizon than UWB devices. In 3GPP solutions we should leverage such complementary features and make the sidelink ranging more accurate and versatile.

3GPP based sidelink ranging  will have unique advantage over similar competing technologies in terms of fixed infrastructure of base stations.

3GPP should prioritize exploiting complementary benefits of fixed wireless network infrastructure to benefit over competing non-3GPP ranging solutions.

Ranging between devices can be made in different ways, depending on device synchronization and network support. For out-of-coverage (OoC) scenarios, or when there is no assistance from the network, the uncertainty in time synchronization will require the use of two-way ranging (TWR) for RTT measurements.

Sidelink ranging between devices should be possible in different scenarios. In-coverage, partial coverage and out-of-coverage scenarios. A few things should be prioritized during the study item for sidelink ranging

- Usage of existing signals for ranging among UEs.

- Usage of spectrum while doing ranging among UEs.

A sidelink ranging solution should reuse existing signals

### Hybrid ranging

As discussed above, one big salient feature of 3GPP based ranging will be presence of fixed infrastructure. The Fig.1 below illustrates the necessity of involving gNB in the sidelink ranging.

Figure 1 An example of network constructing synthetic range when UEs are in NLOS. In the above picture, the synthetic range computed by the network using the position estimates of UEs would be more correct than the range estimates between UEs. In such a case UEs can improve their range estimates by using synthetic range from the network.

For in-coverage scenarios where the UE is connected to the network, resource efficient one-way-ranging (OWR) can be performed over PC5, assuming additional measurements are made over Uu. One such measurement to use would be the timing advance, which can be used to calculate the timing offset between the transmitting and receiving device when preforming OWR over PC5.

The gNBs and the UEs will have complementary information useful for estimating range between UEs. Fusing information from gNBs and UEs for estimating range between UEs will increase accuracy of range estimation between UEs

Hybrid ranging involving gNBs and UEs should be studied.

Absolute position of two devices or UEs can be used for estimating the distance or range between the devices. Ranging between devices would be redundant if very accurate device positions are available. However, range accuracy can also be high for nearby devices by exchange of signals among themselves and doing appropriate measurements.

Very accurate absolute positioning of devices also provides ranging between devices. However, as very accurate positioning is not always guaranteed, device-to-device ranging can help to deliver the required accuracy.

## Positioning

For in-coverage and partial-coverage scenarios, the 3GPP Rel. 17 uplink and downlink positioning methods can have reduced accuracy due to for instance NLOS conditions or long inter-site distance. In such situations, hybrid positioning with uplink/downlink positioning measurements and additional sidelink range measurements between UEs can improve the positioning accuracy of all UEs. The possibility for LMF to request such additional sidelink measurements should be supported, which has already been implicitly agreed as a system requirement in [2].

It should be possible for LMF to request sidelink positioning measurements between UEs.

In out-of-coverage scenarios, the design of the sidelink positioning solution need to be made in such a way that it is versatile and can cater for mobile devises forming ad-hoc positioning networks. It makes sense for such solution do be decentralized in the sense that no UE need to coordinate other UEs, and each UE initiate and collects relevant positioning measurements independently.  A target UE that needs to be positioned can discover nearby UEs with the ProSe Direct Discovery procedure, and then initiate unicast ranging with a set of supporting UEs (UEs with estimated positions) to determine its own position by trilateration. Once it has computed it's own location it may serve as anchor point to other UEs.

In out-of-coverage, UE-based positioning solution should between pairs of UEs, where UEs discover other UEs capable of supporting in positioning, and initiate unicast ranging measurements towards these nodes. A UE acting as a location server for another UE, i.e. centralizing assisdance data from other UEs,  is not supported.

# Conclusions

In the previous sections we made the following observations:

Observation 1	3GPP based sidelink ranging and positioning will have unique advantage over similar competing technologies in terms of fixed infrastructure of base stations.

Observation 2	Very accurate absolute positioning of devices alleviates the need of ranging between devices. However, as very accurate positioning is not always guaranteed, ranging should be performed between the devices to improve the range estimation.

Observation 3

Observation 4	Absolute or relative positioning can be done based on (i) pair-vice range measurements between UEs and (ii) that UEs can share their location with each other.

Observation 5	Absolute or relative positioning based on pair-vice range measurements can be decentralized in the sense that no UE has to coordinate other UEs.

Observation 6	Bla bla

Observation 7

Based on the discussion in the previous sections we propose the following:

Proposal 1	3GPP should prioritize exploiting complementary benefits of fixed wireless network infrastructure to benefit over competing solutions.

Proposal 2	It should be possible for LMF to request sidelink positioning measurements between UEs.

# References

RP-213588, Study on expanded and improved NR positioning, RAN#94e, 2021

3GPP TS 22.261, "Service requirements for the 5G system".