3GPP TSG-RAN WG1 #109	R1-2204953

e-Meeting, May 9th-20th, 2022

Agenda Item:	9.5.2.3

Source:	Ericsson

Title:	On the requirements, evaluations and potential enhancements for Low Power High Accuracy Positioning

Document for:	Discussion, Decision

# Introduction

The SID for the for Rel-18[1] includes the following objective for low power high accuracy positioning (LPHAP)

- Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1] - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study. - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

In this contribution, we present our view as to which positioning  methods to be considered for the use case defined in the SID, the requirements for the use case and the need for potential enhancements.

# Discussion

Case 6 is defined as follow in TS 22.104:

Table A.7.2-1: Low power high accuracy positioning use cases Use Case #; Horizontal accuracy; Corresponding service level (22.261); Positioning interval/ duty cycle; battery life time/ minimum operation time. 1; 10 m; Service Level 1; on request; 24 months. 2; 2 m to 3 m; Service Level 2; < 4 seconds; > 6 months. 3; < 1 m; Service Level 3; no indication; 1 work shift - 8 hours (up to 3 days, 1 month for inventory purposes). 4; < 1 m; Service Level 3; 1 second; 6 - 8 years. 5; < 1 m; Service Level 3; 5 seconds - 15 minutes; 18 months. 6; < 1 m; Service Level 3; 15 s to 30 s; 6 - 12 months. 7; 30 cm; Service Level 5; 250 ms; 18 months. 8; 30 cm; Service Level 5; 1 second; 6 - 8 years (no strong limitation in battery size). 9; 10 m; Service Level 1; 20 minutes; 12 years (@20mJ/position fix). Use case six Flexible modular assembly area: Tracking of workpiece (in- and outdoor) in assembly area and warehouse.

The accuracy target of 1m has been demonstrated in rel16 and 17 and a variety of UL, DL, UL+DL methods can achieve it already for both RRC_CONNECTED and RRC_INACTIVE state. In our view, the main objective of Rel-18 for the LPHAP objective is thus to deliver the long battery life target of the use case.

NR positioning has already demonstrated sub-meter positioning accuracy in previous releases.

RAN1 to focus on the power saving improvement for LPHAP

Considering power saving constraint, we propose to only consider downlink methods for LPHAP, and focus on measurements in RRC_IDLE to minimize power usage.  The motivation is that UL RS transmission is  comparatively more power hungry that DL RS reception. Moreover, for power saving reasons, it is also better to support methods that support UE based positioning.

LPHAP use case 6 can be delivered in Rl18  with the exisiting methods in terms of accuracy. Power comsumption target  can be reached with DL measurements performed in RRC_IDLE.

Support DL positioning measurements in RRC_IDLE in 38.215

RAN1 can discuss specify DL measurements to specify for RRC_IDLE during the normative phase of the WI.

# Conclusion

In the previous sections we made the following observations:

Observation 1	NR positioning has already demonstrated sub-meter positioning accuracy in previous releases.

Observation 2	LPHAP use case 6 can be delivered in Rl18  with the exisiting methods in terms of accuracy. Power comsumption target  can be reached with DL measurements performed in RRC_IDLE.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN1 to focus on the power saving improvement for LPHAP

Proposal 2	Support DL positioning measurements in RRC_IDLE in 38.215

Proposal 3	LPHAP use case 6 can be delivered in Rl18  with the exisiting methods in terms of accuracy. Power comsumption target  can be reached with DL measurements performed in RRC_IDLE.  RAN1 can discuss specify DL measurements to specify for RRC_IDLE during the normative phase of the WI.

# 4	References

RP-213588, “Study on expanded and improved NR positioning”, Intel, December 2021.