3GPP TSG-RAN WG1 #109	R1-2204954

e-Meeting, May 9th-20th, 2022

Agenda Item:	9.5.3

Source:	Ericsson

Title:	Consideration for RedCap Positioning

Document for:	Discussion, Decision

# 1	Introduction

According to the Rel-18 study item description on study on expanded and improved new radio (NR) positioning [1], the objective on reduced capability (RedCap) positioning is as follows:

- Positioning support for RedCap UEs, considering the following: - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs [RAN1] - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with for RedCap UEs [RAN1, RAN2]

In this contribution, we share our consideration about RedCap positioning in terms of simulation assumptions for performance evaluation.

# 2 	Discussion

RedCap has been introduced in Rel-17 to support the IoT scenarios, such as industrial wireless sensors, video surveillance and wearables which have the requirements of reduced UE cost, smaller device form factor and/or longer battery life than NR while higher data rates and/or better latency than long term evolution - machine type communications (LTE-M) and narrowband – internet of things (NB-IoT). Therefore, the application scenarios of RedCap positioning include indoor factory (InF), indoor office (IOO), urban micro (UMi), urban macro (UMa) and rural macrocell (RMa).

Evaluation scenarios for RedCap positioning include InF, IOO, UMi, UMa and RMa.

The existing RAT-dependent solutions in NR positioning Rel-17 which can support RedCap positioning include DL-TDOA, UL-TDOA, multi-RTT, DL-AoD and UL-AoA, where downlink-based solutions such as DL-TDOA may be more suitable for IoT application scenarios, and multi-RTT is also a good candidate especially in asynchronized network.

Evaluate DL-TDOA and multi-RTT for RedCap positioning, with high priority.

As summarized in Table 1, the limitations of RedCap UEs which may have impacts on positioning include reduced maximum UE bandwidth, reduced minimum UE antenna configuration, reduced maximum UE antenna configuration, reduced minimum EIRP in FR2 and supported HD-FDD in FR1 [2][3]. Hence, we have the proposals below for these limitations of RedCap UEs.

Table 1 Limitations of RedCap UEs which may have impacts on positioning

|  | FR1 |  | FR2 |  |
| --- | --- | --- | --- | --- |
|  | NR | RedCap | NR | RedCap |
| Maximum UE bandwidth | 100MHz | 20MHz | 400MHz | 100MHz |
| Minimum antenna configuration | 2 Rx branches (FDD) 4 Rx branches (TDD) 1 Tx branch | 1 Rx branch 1 Tx branch | (Mg, Ng, M, N, P) = (1, 1, 2, 2, 2) with PC3 [see 38.101-2] | (Mg, Ng, M, N, P) = (1, 1, 1, 2, 2) with PC7 [see 38.101-2] |
| Maximum antenna configuration | 8 Rx branches | 2 Rx branches 2 Tx branches | - | (Mg, Ng, M, N, P) = (1, 1, 2, 2, 2) with PC3 [see 38.101-2] |
| Minimum EIRP |  |  | 22.4dBm (PC3 for band n257, n258, n261) | 17dBm (PC7 for band n257, n258, n261), 22.4dBm (PC3 for band n257, n258, n261) |
| Duplex operation | FDD, TDD | HD-FDD, FDD, TDD | TDD | TDD |

For performance evaluation of RedCap positioning,

- maximum UE bandwidth is 20MHz in FR1 and 100MHz in FR2,

- minimum antenna configurations are one Rx branch and one Tx branch in FR1 and (Mg, Ng, M, N, P) = (1, 1, 1, 2, 2) with PC7 in FR2,

- maximum antenna configurations are two Rx branches and two Tx branches in FR1 and (Mg, Ng, M, N, P) = (1, 1, 2, 2, 2) with PC3 in FR2, and

- minimum EIRP in FR2 is 17dBm for RedCap UEs with PC7 and 22.4dBm for RedCap UEs with PC3.

- Note: performance evaluation for RedCap UEs with PC3 in FR2 is optional.

The carrier frequencies used for the performance evaluation during the study item of NR positioning Rel-17 are 3.5GHz in FR1 and 28GHz in FR2, thus we prefer to use same configurations for RedCap positioning in InF, IOO, UMi and UMa. The carrier frequency in RMa can be 700MHz as RedCap study item in Rel-17. Regarding subcarrier space (SCS), it is recommended to configure 15kHz for RMa and 30kHz for other scenarios in FR1, and 120kHz in FR2.

Carrier frequencies are 700MHz for RMa and 3.5GHz for other scenarios in FR1, and 28GHz in FR2.

SCS can be 15kHz for RMa and 30kHz for other scenarios in FR1, and 120kHz in FR2.

In TR 38.875, the UE distribution for RedCap performance evaluation in urban is 80% indoor with the speed of 3km/h and 20% outdoor in cars with the speed of 30km/h. In TR 38.802 the UE distribution for NR performance evaluation in RMa is 50% indoor with the speed of 3km/h and 50% outdoor in cars with the speed of 120km/h. So we would like to use similar assumptions for the performance evaluation of RedCap positioning.

UE distribution in UMi and UMa is 80% indoor with the speed of 3km/h and 20% outdoor in cars with the speed of 30km/h.

UE distribution in RMa is 50% indoor with the speed of 3km/h and 50% outdoor in cars with the speed of 120km/h.

More detailed simulation assumptions are provided in Table 2 of Annex.

Use the simulation assumptions listed in Table 2 for performance evaluation in RMa.

# 3	Conclusion

In this contribution, we have the proposals below for RedCap positioning.

Proposal 1	Evaluation scenarios for RedCap positioning include InF, IOO, UMi, UMa and RMa.

Proposal 2	Evaluate DL-TDOA and multi-RTT for RedCap positioning, with high priority.

Proposal 3	For performance evaluation of RedCap positioning,

	maximum UE bandwidth is 20MHz in FR1 and 100MHz in FR2,

	minimum antenna configurations are one Rx branch and one Tx branch in FR1 and (Mg, Ng, M, N, P) = (1, 1, 1, 2, 2) with PC7 in FR2,

	maximum antenna configurations are two Rx branches and two Tx branches in FR1 and (Mg, Ng, M, N, P) = (1, 1, 2, 2, 2) with PC3 in FR2, and

	minimum EIRP in FR2 is 17dBm for RedCap UEs with PC7 and 22.4dBm for RedCap UEs with PC3.

	Note: performance evaluation for RedCap UEs with PC3 in FR2 is optional.

Proposal 4	Carrier frequencies are 700MHz for RMa and 3.5GHz for other scenarios in FR1, and 28GHz in FR2.

Proposal 5	SCS can be 15kHz for RMa and 30kHz for other scenarios in FR1, and 120kHz in FR2.

Proposal 6	UE distribution in UMi and UMa is 80% indoor with the speed of 3km/h and 20% outdoor in cars with the speed of 30km/h.

Proposal 7	UE distribution in RMa is 50% indoor with the speed of 3km/h and 50% outdoor in cars with the speed of 120km/h.

Proposal 8	Use the simulation assumptions listed in Table 2 for performance evaluation in RMa.

# References

RP-213588, “SID on study on expanded and improved NR positioning”, RAN#94e, December 2021.

3GPP TR 38.875, “Study on support of reduced capability NR devices”, March 2021.

R4-2206545, “LS on FR2 RedCap UE”, 12st Feb – 3rd March 2022.

3GPP TR 38.857, “Study on NR positioning enhancements”, March 2021.

3GPP TR 38.855, “Study on NR positioning support”, March 2019.

3GPP TR 38.802, “Study on new radio access technology, physical layer aspects”, September 2017.

# Annex: Simulation Assumptions in RMa

Table 2 Simulation assumptions in RMa

| Parameters | Values |
| --- | --- |
| Layout | Hexagonal grid, 19 or 7 macro sites, 3 sectors per site, ISD = 1732m or 5000m, Note 1 Wrap-around is applied, Note 2 |
| Carrier frequency | 700MHz |
| Bandwidth | 10MHz for DL and 10MHz for UL – Note 1 |
| Subcarrier spacing | 15kHz SCS |
| gNB noise figure | 5 dB |
| gNB antenna configuration | See Table A.2.1-4 in TR 38.802 |
| gNB antenna element gain + connector loss | See Table A.2.1-4 in TR 38.802 |
| Total gNB TX power | 49 dBm |
| UE max. Tx power | 23 dBm – Note 1 |
| UE noise figure | 9 dB – Note 1 |
| UE antenna configuration | The number of antenna: Rx={1,2}, Tx={1,2}, (Mg, Ng, M, N, P)={(1,1,1,1,1), (1,1,1,1,2)} with 0.5λ spacing with omni-directional antenna element |
| UE antenna radiation pattern | Omni, 0dBi |
| Channel model | ITU Rural with LOS and NLOS |
| Number of floor | All UEs are on the ground |
| UE distribution | 50% outdoor in cars (120km/h) and 50% indoor (3km/h), uniformly distributed over the horizontal area (separate statistic) |
| UE height | 1.5 m |
| Min. gNB-UE distance (2D) | 35 m |
| gNB antenna height | 35 m |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns |
| Note 1: According to 3GPP TR 38.802 Note 2: In case if interference considerations are not properly taken into account for 7 sites companies are encouraged to provide results for 19 sites. Note 3: According to 3GPP TR 38.901 |  |