3GPP TSG-RAN WG1 #109	R1-2204955

e-Meeting, May 9th-20th, 2022

Agenda Item:	9.5.4

Source:	Ericsson

Title:	Considerations for PRS/SRS bandwidth aggregation

Document for:	Discussion, Decision

# Introduction

The SID for the for Rel-18[1] includes the following objective for Improved accuracy, integrity, and power efficiency:

Study solutions for accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers considering e.g. timing errors, phase coherency, frequency errors, power imbalance, etc

In this contribution, we present our current understanding of the potential performance gains that can be achieved by this method under different assumptions.

# Time of arrival estimation accuracy

Consider the following single-path channel model in the frequency domain:

clj=PTXGnlexpi∅l-2πifSCSj-flδ+Nlj

Where j=-nl2,…,nl2-1 are subcarrier indices, l=1,2,…,m are carrier indices (each with different unknown phase offset ∅l), fSCS is the subcarrier spacing, fl is the carrier frequency of carrier l, PTX is the TX power, G is the (unknown) path gain, δ is the (unknown) delay, and Nlj is i.i.d. complex Gaussian noise with zero mean and variance σ2. We assume that for each carrier index l we have measured on nl consecutive subcarriers, e.g., by combining k consecutive symbols from a staggered comb-k PRS. The bandwidth of carrier l is then nlfSCS.

From calculating the Cramér-Rao bound it turns out that any unbiased estimator δ of the delay has a bounded error variance according to

varδ-δ≥3σ22π2PTXGfSCS2l=1mnl2-2

We see that the Cramér-Rao lower bound on the TOA error variance (over all unbiased TOA estimators) scales to the inverse square of the bandwidth (assuming fixed TX power). This means that if the bandwidth is doubled for all carriers l=1,2,…,m (while at the same time keeping the TX power constant), the TOA error variance is decreased to about ¼th of the original value. On the other hand, the TOA error variance is inversely proportional to the number of consecutive PRS resources (which is seen if we interpret the carriers l=1,2,…,m as different PRS resources with the same bandwidth nl=n). If the number of PRS resources is doubled (i.e., from a 1-slot staggered comb-6 PRS to a 2-slot staggered comb-6 PRS at the same bandwidth), the TOA error variance is halved. The TOA error variance therefore decreases faster with increasing bandwidth than increasing PRS resources in the time (symbol) dimension.

These benefits from increasing the bandwidth will not hold if some bandwidth parts are non-coherent in phase, i.e., if there is an unknown phase offset between the bandwidth parts. This is also seen from the Cramér-Rao bound, since it does not depend on the carrier frequencies fl. Let's say, for example, that we have a full 100MHz PRS, where there is an unknown phase offset ∅1 for the upper 50MHz part and another unknown phase offset ∅2 for the lower 50MHz bandwidth part. This would correspond to the same TOA error variance as a 50MHz carrier with double resources in time, and not a 100MHz carrier.

Another way to put it is that the total bandwidth does not impact the TOA accuracy if the carriers (bandwidth parts) are non-coherent in phase. Instead, the TOA accuracy is a function of the individual bandwidths for each carrier. Let us say that we have two non-coherent but contiguous bandwidth parts of nfSCS each. The total bandwidth is then 2nfSCS, but the TOA error variance bound is only downscaled by about a factor of 2n2fSCS2, and not by 4n2fSCS2 which would have been the case if both bandwidth parts were coherent in phase. Furthermore, if we move one of the bandwidth parts so that the total bandwidth increases, then the TOA error variance is completely unaffected.

For AWGN with Gaussian noise, the Cramér-Rao TOA error variance decreases twice as fast with respect to the bandwidth compared to the time-domain PRS resources, under the assumption that the subcarriers within the bandwidth region have the same (unknown) phase offset.

If the bandwidth regions are non-coherent in phase, then the CRLB TOA accuracy is a function of the individual bandwidths for each carrier and does not depend on the total bandwidth.

If two PRS resources on different carriers are too far apart (either in time or in frequency), they may have completely different phase offsets. This would defeat the potential accuracy gained from high bandwidth. If the PRS resources are close, chances are that the difference in phase offsets are small enough so that the increased accuracy is anyways significant.

Increased bandwidth has the potential for improving even the multipath situation, although this is less straightforward. High bandwidth will single out individual peaks in the power-delay profile, which would have been smeared together for lower bandwidth. These effects also rely on having a single phase offset for each channel path, over the entire bandwidth.

As a highest priority, study a multipath scenario where the PRS is distributed over two contiguous 100MHz carriers (which together forms a 200MHz bandwidth scenario). Compare this by putting all PRS resources on a single 100MHz carrier.

# Conclusion

In the previous sections we made the following observations:

Observation 1	For AWGN with Gaussian noise, the Cramér-Rao TOA error variance decreases twice as fast with respect to the bandwidth compared to the time-domain PRS resources, under the assumption that the subcarriers within the bandwidth region have the same (unknown) phase offset.

Observation 2	If the bandwidth regions are non-coherent in phase, then the CRLB TOA accuracy is a function of the individual bandwidths for each carrier and does not depend on the total bandwidth.

Based on the discussion in the previous sections we propose the following:

Proposal 1	As a highest priority, study a multipath scenario where the PRS is distributed over two contiguous 100MHz carriers (which together forms a 200MHz bandwidth scenario). Compare this by putting all PRS resources on a single 100MHz carrier.

# 4	References

RP-213588, “Study on expanded and improved NR positioning”, Intel, December 2021.