3GPP TSG RAN WG1 #109-e		                          			   R1-2204957

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.8.3

Source:	InterDigital, Inc.

Title:	Remaining issues for PUCCH coverage enhancements

Document for:	Discussion and Decision

# Introduction

This contribution proposes that RAN1 concludes earlier discussions on the application of dynamic PUCCH repetition factor indication mechanism to SPS PDSCH, considering R15/R16 maintenance agreements made at RAN1#108-e.

# Dynamic PUCCH repetition factor indication

RAN1 discussed dynamic PUCCH repetition factor indication for SPS PDSCH for several meetings without reaching a conclusion. During these discussions, it was realized that there was no common understanding on how the PUCCH resource is indicated for the (first) PDSCH associated with an activation DCI even for R15. In RAN1#108-e, RAN1 discussed this issue within R15 maintenance discussion [108-e-NR-CRs-02] and reached the following agreement:

Agreement The TP for SPS PDSCH activation and PUCCH resource selection for the 1st SPS PDSCH is endorsed in R1-2202898 (TS38.213, Rel-16, CR#0286, Cat. F), R1-2202899 (TS38.213, Rel-17, CR#0287, Cat. A). - Note: Qualcomm, LG, DOCOMO, and Apple have concern that the above text proposal removes the functionality of PRI and DAI of DL SPS activation DCI. But these companies can accept the proposal for the sake of RAN1 progress, to avoid the consequence of losing the functionality of DL SPS in NR, given this proposal is the only solution due to other company's objection to other proposal(s).

As described in the cover page of the CR in R1-2202899 [1], the agreement means that the PUCCH resource determination for the first SPS PDSCH transmitted in response of the SPS PDSCH activation follows the RRC-configured n1PUCCH-AN or SPS-PUCCH-AN-List like all subsequent SPS-PDSCHs. Since the PUCCH is not dynamically indicated, it follows that dynamic PUCCH repetition factor indication introduced in R17 does not apply to the PUCCH in response to this first SPS PDSCH.

Proposal 1: RAN1 concludes that the dynamic PUCCH repetition factor indication mechanism of NR R17 is not applicable to HARQ-ACK for the first SPS PDSCH associated with the activation DCI.

As part of coverage enhancement AI, RAN1 also discussed applicability to HARQ-ACK of SPS release DCI but the conclusion on this was held up by the issue for the first SPS PDSCH. In the case of SPS release, it is clear that the PUCCH resource is provided dynamically so that the following conclusion can be made:

Proposal 2: RAN1 concludes that the dynamic PUCCH repetition factor indication mechanism of NR R17 is applicable to HARQ-ACK corresponding to the SPS release DCI.

No change to latest version of TS 38.213 is required to reflect the above conclusions, since it already states in section 9.2.6 that the UE uses the repetition factor tied to the indicated PUCCH resource (PUCCH-nrofSlots) if the PUCCH resource is indicated by a DCI format.

# Conclusion.

In this contribution, the following proposals are made.

Proposal 1: RAN1 concludes that the dynamic PUCCH repetition factor indication mechanism of NR R17 is not applicable to HARQ-ACK for the first SPS PDSCH associated with the activation DCI.

Proposal 2: RAN1 concludes that the dynamic PUCCH repetition factor indication mechanism of NR R17 is applicable to HARQ-ACK corresponding to the SPS release DCI.

# Reference

[1] R1-2202898, “Clarification of the SPS PDSCH activation and HARQ-ACK transmission for the 1st SPS PDSCH”, Moderator (Nokia).