3GPP TSG RAN WG1 #109-e	Tdoc R1-2204961

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	8.7.3

Source:	Ericsson

Title:	Related aspects of Rel-17 UE PS maintenance

Document for:	Discussion and Decision

# 1		Introduction

In this paper, following maintenance aspect is considered.

- PDCCH monitoring adaptation along with BWP-switching.

# 2	Discussion

As the PDCCH monitoring adaptation is configured per-BWP, 2 BWPs may be configured with different PDCCH monitoring adaptation configuration. Therefore, when the BWP is switched, the state which the UE should follow in the new active BWP needs to be agreed.

For the case of BWP-switching is triggered by the BWP inactivity timer, the behavior can be straightforward. I.e., the newly active BWP can start with its default behavior, e.g., start with SSSG0 (if configured) and no skipping (since skipping is via DCI only anyways). The other possible option is to follow the latest state of the source BWP. However, this may bring an inconsistency as the PDCCH monitoring adaptation configuration of the source and the target BWP configurations can be different. As in this case the NW does not have a direct DCI indication, following the default behavior of the target BWP is a more intuitive solution. Similarly, when the target BWP is configured with PDCCH monitoring adaptation and the source BWP is not configured with PDCCH monitoring adaptation bitfield, the default behavior of the target BWP can be used when the UE starts using the target BWP.

When the BWP inactivity timer expires, the UE starts the target BWP with the default state (e.g., SSSG0).

When the source BWP is not configured with PDCCH monitoring adaptation and the target BWP is configured with PDCCH monitoring adaptation, the target BWP starts with the default state (e.g., SSSG0).

# Conclusion

In section 2, the following proposals were made:

Proposal 1	When the BWP inactivity timer expires, the UE starts the target BWP with the default state (e.g., SSSG0).

Proposal 2	When the source BWP is not configured with PDCCH monitoring adaptation and the target BWP is configured with PDCCH monitoring adaptation, the target BWP starts with the default state (e.g., SSSG0).

# References

RAN1#108-e chair notes

# Annex

Working Assumption (Excerpted from RAN#107bis-e)

- The bit length of the candidate skipping values and SSSG switching initial timer values in slots for 480kHz and 960kHz SCS are assumed to be the same as that for 120KHz SCS

Agreement

-             The 480kHz and 960kHz SCS are also supported for Rel-17 PDCCH monitoring adaptation.

Agreement

- It is up to B52.6GHz to deicide the values for candidate skipping values and SSSG switching initial timer values.

Agreement

For DCI format 0_1, DCI format 0_2, DCI format 1_1 and DCI format 1_2,

- 1 or 2 bits, if PDCCHSkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured

-     1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-     2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2;

Note: Same as in existing specifications, specifications allow gNB to configure a search-space set with zero PDCCH candidates.

Agreement

Answer 1 for R1-2200884 (R2-2201960):

Yes, PDCCH skipping can be applied to C-RNTI(s) and MCS-C-RNTI(s) in at least Type3 PDCCH CSS and USS monitored during RAR/MsgB window for RAR/MsgB reception based on RAN1 agreements till RAN1#107bis-e.

Agreement

Answer 2 for R1-2200884 (R2-2201960):

RAN1 would discuss and conclude how to capture the above RAN2 agreements in RAN1 specification.

o    It is RAN1 understanding  PDCCH skipping  is not applied to perform PDCCH monitoring during RAR window/MsgB window/contention resolution timer or when SR is pending.

- FFS: If the UE is indicated skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE can stops PDCCH skipping and monitor PDCCH for UL grant (i.e., PDCCH skipping is not activated ).

Agreement

Answer 4 for R1-2200884 (R2-2201960):

- It is RAN1’s understanding that transmission of CSI/SRS is not affected by PDCCH skipping i.e. CSI/SRS is still transmitted during the skipping duration.

Agreement

- It is RAN1 understanding that for DRX operation,

  - As DCP is monitored only outside active time, it cannot be missed due to PDCCH skipping as skipping applies only in active time.

Note: RAN1 is discussing whether PDCCH skipping duration can apply to outside active time.

Agreement

If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE

- Resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by

- Working Assumption: C-RNTI/CS-RNTI/MCS-C-RNTI

- Otherwise, decrease the timer value by one after each slot.

- Note: There is no timer for SSSG switching is running for UE in SSSG#0

Agreement

[Draft] LS reply on PDCCH skipping, R1-2202896 is endorsed in principle.

Agreement

Final LS reply on PDCCH skipping, R1-2202905 is endorsed

Working Assumption

- Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B),

  - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception

    - FFS: a minimum applicable scheduling offset is configured in the BWP

  - Note: Pswitch is defined in Table 10.4-1 in TS38.213

Agreement

- Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH skipping (i.e., Beh 1A), select one of the following schemes

  - Alt 1a: the UE applies Beh 1A on an active BWP of the serving cell at the first slot after the last OFDM symbol of the PDCCH reception

    - FFS: a minimum applicable scheduling offset is configured in the BWP

    - FFS: whether the UE will monitor PDCCH when DRX Retransmission timer is running

Agreement

-     When the timer expires, the UE monitors PDCCH on the serving cell according to search space sets with group index 0 in a slot

  - no earlier than [application delay] after the slot where timer expires and

  - if PDCCH skipping is being applied, no earlier than in slot where PDCCH skipping duration expires.

Working assumptions:

- When the UE receives DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication' field in slot n, the UE does not expect to receive DCI format  0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

- If SSSG timer expires in slot n, the UE does not expect to receive DCI format  0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.