3GPP TSG RAN WG1 #109-e			Tdoc R1-2204963

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	8.16.7

Source:	Ericsson

Title:	UE features for UEPS

Document for:	Discussion and Decision

# 1		Introduction

This document discusses remaining issues of UE features for UE power savings.

# 2	Proposals

Our proposed changes to the Rel-17 UE power savings FGs are as follows.

- FG 29-2 :

  - Add as component 2) “Support receiving L1 indication for TRS availability”.

  - Add in Note column “Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7”.

- FG 29-3b :

  - Add in Note column “UE supports search space set group switching capability-1 according to Table 10.4-1 of 38.213.” to clarify that SSSGS is based on capability 1 switching time according to Table 10.4-1 of 38.213. (See Annex A for detailed agreement from RAN1#108-e).

- (New) FG 29-3e

  - Introduce new FG (29-3e) with below description to indicate support of SSSG switching according to capability 2 switching time, analogous to 10-9d that was introduced for Rel-16 NR-U (see Annex B).

| 29. NR_UE_pow_sav_enh | 29-3e | Support Search space set group switching capability 2 | Search space set group switching Capability-2 according to Table 10.4-1 of 38.213 for SSSG switching. | 29-3b | Yes |  | n/a | Per band |  | N/A | NA |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# References

RAN1#108-e chair notes

R1-2202929          Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e including remaining RAN1 issues        Moderators (AT&T, NTT DOCOMO, INC.)

# Annex A (Agreement from RAN1#108-e)

Below is the working assumption made in RAN1#08-e, and the corresponding Pswitch table from 38.213.

Working Assumption

- Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B),

  - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception

    - FFS: a minimum applicable scheduling offset is configured in the BWP

  - Note: Pswitch is defined in Table 10.4-1 in TS38.213

<table 10.4-1 from 38.213>

Table 10.4-1: Minimum value of Pswitch [symbols]

| μ | Minimum Pswitch value for UE processing capability 1 [symbols] | Minimum Pswitch value for UE processing capability 2 [symbols] |
| --- | --- | --- |
| 0 | 25 | 10 |
| 1 | 25 | 12 |
| 2 | 25 | 22 |
| 3 | 40 | - |
| 5 | 160 | - |
| 6 | 320 | - |

# Annex B (Rel-16 NR-U capability for SSSG switching capability #2)

| 10-9d | Support Search space set group switching capability 2 | Search space set group switching Capability-2: P=10/12/22 symbols for µ = 0/1/2 SCS | one of {10-9, 10-9b} | searchSpaceSetGroupSwitchingcapability2-r16 | SharedSpectrumChAccessParamsPerBand-r16 | n/a | n/a | Without this capability, the UE supports search space set group switching capability-1: P=25/25/25 symbols for µ=0/1/2 the signaling is per band but is only expected for a band where shared spectrum channel access must be used | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |