3GPP TSG RAN WG1 Meeting #109-e		R1-2204967

# E-meeting, May 9th – 20th, 2022

Title:	Draft reply LS on configuration of p-MaxEUTRA and p-NR-FR1

Response to:	R1-2200873, R1-2203038

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	Qualcomm [RAN1]

To:	RAN5, RAN4

Cc:	RAN2

Contact person:	Fred Takeda

ktakeda (at) qti (dot) qualcomm (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

At the RAN1#108-e meeting, RAN1 responded to RAN5 LS (R1-2200873) regarding configuration of p-MaxEUTRA and p-NR-FR1 in R1-2202769 as follows.

There is no specified UE behavior in existing RAN1 specifications for the case where FR1-FR1 EN-DC is configured but p-MaxEUTRA or p-NR-FR1 is not configured. RAN1 may discuss potential action, if any, after RAN2/4 responses are received.

In parallel, RAN4 reply LS (R1-2203038) was agreed at the RAN4#102-e that includes following description.

For UEs supporting dynamic power sharing, RAN4 understanding is there is no specified UE behavior when the network does not configure p-MaxEUTRA or p-NR-FR1. It is up to RAN1 to confirm if this is a valid configuration. RAN4 will further discuss whether ‘infinity’ could be used as default value if these two parameters are not configured, and whether and how to capture this in the specification. For UEs not supporting dynamic power sharing, RAN4 understanding is the UE’s transmitted power is not fully specified by RAN4. It is up to RAN1 to decide if p-MaxEUTRA or p-NR-FR1 should be configured by the network or if default values are needed.

To conclude the discussions, RAN1 can confirm the following:

- p-MaxEUTRA and p-NR-FR1 are not mandatory parameters

  - RAN1’s understanding is that maximum transmission power of each cell-group is defined in TS 38.101-3, and p-MaxEUTRA and p-NR-FR1 are the parameters that may cap the maximum transmission power for the corresponding cell-group, if configured

  - The default value when p-MaxEUTRA or p-NR-FR1 is not configured is up to RAN4

# 2	Actions

To 3GPP RAN5, RAN4

ACTION: To take into account the RAN1’s feedback on configuration of p-MaxEUTRA and p-NR-FR1 for EN-DC operation for the specification work.

# 3	Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #110		22 – 26 August 2022, Toulouse, France