3GPP TSG RAN WG1 #109-e			R1-2204973

e-Meeting, May 9th – 20th, 2022

Agenda item:	7.1

Source: 	Qualcomm Incorporated

Title: 	On parallel transmission of SRS and other channels

Document for:	Discussion and Decision

# Background

Since Rel-15, the parallel transmission of SRS and other channels for inter-band CA is governed by the following UE capabilities in TS 38.306 (per band combination)

| parallelTxSRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of SRS and PUCCH/ PUSCH across CCs in an inter-band CA band combination. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| parallelTxPRACH-SRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of PRACH and SRS/PUCCH/PUSCH across CCs in an inter-band CA band combination. | BC | No | N/A | N/A |

The RAN1 specifications cover this condition in the following statements of TS 38.214:

In case of intra-band carrier aggregation or in inter-band CA band combination if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by UE, the UE is not expected to be configured with SRS from a carrier and PUSCH/UL DM-RS/UL PT-RS/PUCCH formats from a different carrier in the same symbol.

In case of intra-band carrier aggregation or in inter-band CA band combination if simultaneous SRS and PRACH transmissions are not supported by UE, the UE shall not transmit simultaneously SRS resource(s) from a carrier and PRACH from a different carrier.

In summary, in the case of intra-band CA, the specification does not allow the UE to simultaneously transmit SRS and other channels, while for the case of inter-band CA the transmission of SRS and other channels is given by a UE capability.

- Discussion

The case of simultaneous transmission for intra-band CA is not allowed by RAN1 specifications due to the assumption of using a single PA, which may incur in phase discontinuity. However, in the case of non-contiguous intra-band CA, the UE may use separate PAs for the multiple carriers, thus not having this issue.

Observation 1: Current rules for parallel transmission of SRS and other channels do not consider the case of non-contiguous intra-band CA, where the UE may operate with two separate PAs.

We also note that, in terms of signaling, the case of non-contiguous intra-band CA is very similar to the case of inter-band CA. For non-contiguous case, a band is repeated multiple times in a band combination, thus allowing a similar signaling flexibility as the inter-band case.

Therefore, it is our view that the case of non-contiguous intra-band CA should be treated similarly to the inter-band CA in terms of capability for parallel SRS transmission with other channels.

Proposal 1: Clarify that, for non-contiguous intra-band CA, the UE can optionally indicate the capability to perform parallel transmission of SRS and other channels (as given by the capabilities parallelTxSRS-PUCCH-PUSCH and parallelTxPRACH-SRS-PUCCH-PUSCH)

Necessary specification changes to TS 36.214 and TS 36.306 are shown below.

=====================================TP1 38.306 ======================================

| parallelTxSRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of SRS and PUCCH/ PUSCH across CCs in an inter-band or intra-band non-contiguous CA band combination. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| parallelTxPRACH-SRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of PRACH and SRS/PUCCH/PUSCH across CCs in an inter-band or intra-band non-contiguous CA band combination. | BC | No | N/A | N/A |

====================================================================================

=====================================TP2 38.214 ======================================

In case of intra-band contiguous carrier aggregation, or in inter-band or intra-band non-contiguous CA band combination if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by UE, the UE is not expected to be configured with SRS from a carrier and PUSCH/UL DM-RS/UL PT-RS/PUCCH formats from a different carrier in the same symbol.

In case of intra-band contiguous carrier aggregation, or in inter-band or intra-band non-contiguous CA band combination if simultaneous SRS and PRACH transmissions are not supported by UE, the UE shall not transmit simultaneously SRS resource(s) from a carrier and PRACH from a different carrier.

====================================================================================

Proposal 2: Endorse TP1 and TP2 from RAN1 perspective, and send an LS to RAN2 with TP1 asking them to implement these changes.

- Summary

In this contribution we presented one issue regarding the lack of clarity of specifications on simultaneous transmission of SRS and other channels for intra-band non-contiguous CA. We made the following observations and proposals:

Observation 1: Current rules for parallel transmission of SRS and other channels do not consider the case of non-contiguous intra-band CA, where the UE may operate with two separate PAs.

Proposal 1: Clarify that, for non-contiguous intra-band CA, the UE can optionally indicate the capability to perform parallel transmission of SRS and other channels (as given by the capabilities parallelTxSRS-PUCCH-PUSCH and parallelTxPRACH-SRS-PUCCH-PUSCH)

Proposal 2: Endorse TP1 and TP2 from RAN1 perspective, and send an LS to RAN2 with TP1 asking them to implement these changes.