3GPP TSG-RAN WG1 Meeting #109-e	R1-2204977

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.1.2

Source: 	Qualcomm Incorporated

Title: 	Remaining Details for Multi-TRP Operation

Document for:	Discussion/Decision

# Inter-Cell mTRP

## Issue 1: Implementing the existing agreement on rate matching

The agreement below regarding PDSCH rate matching in the case of inter-cell mTRP, is not fully captured by the specifications yet:

Agreement

Agree on scheme1

- Scheme1: PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI

- FFS: whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- FFS: whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB

Basically, the following aspects from the agreement should be reflected in 38.214:

- The “SSB time domain position” for SSB with PCI different from the serving cell is configured by a new parameter “ssb-PositionsInBurst” which is different than the legacy “ssb-PositionsInBurst” associated with the serving cell

- Whether we rate match around PDSCH or not also depends on association of PDSCH with PCI

Note that the PDCCH part is already implemented in 38.213 as shown below while the PDSCH rate matching part is missing from 38.214.

For monitoring of a PDCCH candidate by a UE, if the UE

-	has received ssb-PositionsInBurst in SSB-MTCAdditionalPCI for a serving cell, and

-	at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SSB-MTCAdditionalPCI with same physical cell identity as the one associated with a RS having same quasi-collocation properties as a CORESET for the PDCCH candidate,

the UE is not required to monitor the PDCCH candidate.

Proposal 1: Adopt the following TP (38.214) to capture the existing agreement:

Reason for change: To capture an existing agreement that is not implemented yet in 38.214.

Summary of change: PDSCH rate matching behaviour when overlaps with SSBs associated with additional PCIs.

Consequences if not approved: Ambiguous PDSCH rate matching when overlaps with SSBs associated with additional PCIs.

============TP for 38.214 Section 5.1.4 ====================================

--Unchanged part omitted------------------------

When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, and the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. If PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources according to ssb-PositionsInBurst in SSB-MTCAdditionalPCI with same physical cell identity as the one associated with a RS having same quasi-collocation properties as the PDSCH, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.

A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH.

===============================================================

## Issue 2: Clarifications on SSB overlap with UL Signals/Channels

In RAN1 #108-e, the following proposal was agreed:

Agreement

For inter-cell mTRP , UE does not transmit PUCCH/PUSCH/PRACH in a slot or SRS in the symbols if in time domain the PUCCH/PUSCH/PRACH/SRS overlaps with an SSB of a serving cell PCI or an SSB associated with the active additional PCI.

The above agreement is captured in 38.213 for the following procedures:

- When SSB overlaps with UL channel/RS, UE does not transmit the UL channels/RS [38.213, Section 11.1].

- UE does not expect the set of SSB symbols to indicated as uplink symbols either semi-statically or dynamically (by SFI) [38.213, Section 11.1 and Section 11.1.1].

- For available slot counting to determine NPUCCHRepeat slots in the case of PUCCH repetition, i.e., a slot is not counted toward the NPUCCHRepeat slots if the PUCCH resource in that slot overlaps with a SSB [38.213, Section 9.2.6].

The same principle should be followed for the PUSCH repetition Type B (SSB symbols should be assumed to be invalid symbols in a nominal repetition for PUSCH repetition Type B) in 38.214 following the agreement above.

Additionally, in 38.213, only the PCI associated with the active TCI states is considered. However, this is problematic as UE still measures SSBs from the PCIs not associated with the active TCI states. To address the issue, we can either define “SSBs measured by UE” or remove the condition “associated to physical cell ID with active TCI states”. The former may not be very straightforward as multiple configurations may need to be checked. Furthermore, in practice, when additional PCIs are configured, irrespective of whether they are associated with the active PCIs or not, network will configure measurements for the UE. Otherwise, network cannot know which beam should be activated for the UE. Hence, our preference is the latter option, i.e., remove the condition “associated to physical cell ID with active TCI states”.

Based on the two points in the discussions above, we propose the following:

Proposal 2: Adopt the following TPs (38.213 and 38.214) regarding overlap with UL signals/channels:

Reason for change: In 38.213, the case that additional PCIs are not associated with active TCI states but still used by the UE for measurements is not addressed with respect to overlap between SSB and UL signals/channels. In 38.214, SSBs associated with additional PCIs are not considered as invalid symbols for PUSCH repetition Type B.

Summary of change: In 38.213, remove the condition “associated to physical cell ID with active TCI states” in relevant sections. In 38.214, add SSBs indicated by ssb-PositionsInBurst in SSB-MTCAdditionalPCI as invalid symbols for PUSCH repetition Type B.

Consequences if not approved: UE behaviour remains ambiguous when SSB used by the UE for measurements overlaps with UL signals/channels. PUSCH repetition Type B cannot be scheduled when overlap with SSBs associated with additional PCIs.

============TP for 38.213 Section 9.2.6 ====================================

--Unchanged part omitted------------------------

A SS/PBCH block symbol is a symbol of an SS/PBCH block with candidate SS/PBCH block index corresponding to the SS/PBCH block index indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon or, if the UE is not provided DLorJoint-TCIState or followUnifiedTCIstate, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI.

--Unchanged part omitted------------------------

===============================================================

============TP for 38.213 Section 11.1 ====================================

--Unchanged part omitted------------------------

For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE for reception of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or by ssb-PositionsInBurst in ServingCellConfigCommon or, if the UE is not provided DLorJoint-TCIState or followUnifiedTCIstate, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE.

If a UE

-	is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the multiple serving cells, and

-	indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and

-	is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells,

for a set of symbols of a slot that are indicated to the UE for reception of SS/PBCH blocks in a first cell of the multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon or, if the UE is not provided DLorJoint-TCIState or followUnifiedTCIstate, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI, the UE does not transmit PUSCH, PUCCH, or PRACH in the slot if a transmission would overlap with any symbol from the set of symbols, and the UE does not transmit SRS in the set of symbols of the slot in

-	any of the multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and

-	any one of the cells corresponding to the same band as the first cell, irrespective of any capability indicated by simultaneousRxTxInterBandCA.

--Unchanged part omitted------------------------

===============================================================

============TP for 38.213 Section 11.1.1 ====================================

--Unchanged part omitted------------------------

For a set of symbols of a slot corresponding to SS/PBCH blocks with candidate SS/PBCH block indices corresponding to the SS/PBCH block indexes indicated to a UE by ssb-PositionsInBurst in SIB1, or by ssb-PositionsInBurst in ServingCellConfigCommon, as described in clause 4.1 or, if the UE is not provided DLorJoint-TCIState or followUnifiedTCIstate, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI, the UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols of the slot as uplink.

--Unchanged part omitted------------------------

===============================================================

============TP for 38.214 Section 6.1.2.1 ====================================

--Unchanged part omitted------------------------

For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows:

-	A symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated is considered as an invalid symbol for PUSCH repetition Type B transmission.

-	For operation in unpaired spectrum, symbols indicated by ssb-PositionsInBurst in SIB1, or by ssb-PositionsInBurst in ServingCellConfigCommon, or by ssb-PositionsInBurst in SSB-MTCAdditionalPCI for reception of SS/PBCH blocks are considered as invalid symbols for PUSCH repetition Type B transmission.

-	For a reduced capability half-duplex UE in paired spectrum and for PUSCH repetition Type B transmission, symbols indicated by ssb-PositionsInBurst in SIB1, or by ssb-PositionsInBurst in ServingCellConfigCommon, or by ssb-PositionsInBurst in SSB-MTCAdditionalPCI for reception of SS/PBCH blocks are considered as invalid symbols for PUSCH repetition Type B transmission.

--Unchanged part omitted------------------------

-	If the UE

-	is configured with multiple serving cells within a cell group and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the multiple serving cells, and

-	indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and

-	is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells,

-	a symbol indicated to the UE for reception of SS/PBCH blocks in a first cell of the multiple serving cells by ssb-PositionsInBurst in SIB1, or by ssb-PositionsInBurst in ServingCellConfigCommon, or by ssb-PositionsInBurst in SSB-MTCAdditionalPCI is considered as an invalid symbol for PUSCH repetition Type B transmission in

-	any of the multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and

-	any one of the cells corresponding to the same band as the first cell, irrespective of any capability indicated by simultaneousRxTxInterBandCA

--Unchanged part omitted------------------------

===============================================================

# HST-SFN

## Issue 1: Editorial TP

In clause 5.2.1.5.1, the text description for specifying the TCI state for the reception of the aperiodic CSI-RS when the CORESET with lowest ID is indicated with two TCSI states has extra “if” wording that should be deleted.

Proposal 3:  Adopt the following editorial TP (38.214).

TP for 38.214 Section 5.2.1.5.1 ----------------Unchanged part omitted------------------------ - else if a UE is configured with sfnSchemePdcch set to 'sfnSchemeA', it is not configured with enableTwoDefaultTCI-States, and the two TCI states are activated for the CORESET by the activation command as described in clause 6.1.3.x of [10, TS 38.321] - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with an offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], periodic CSI-RS, semi-persistent CSI-RS, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48} and when enableBeamSwitchTiming is not provided or the NZP-CSI-RS-ResourceSet is configured with the higher layer parameter trs-Info , aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'off' or configured without the higher layer parameters repetition and trs-Info scheduled with offset larger than or equal to 48 when the UE provides beamSwitchTiming-r16 and enableBeamSwitchTiming is provided, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on' scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming-r16 and enableBeamSwitchTiming is provided; - else if, the UE applies the first one of two TCI states indicated for the CORESET with the lowest CORESET ID in the latest slot within the active BWP of the cell in which the CSI-RS is to be received when receiving the aperiodic CSI-RS, ----------------Unchanged part omitted------------------------

## Issue 2: Supported combination of SFN schemes in spec

RAN1 has agreed to support the following combination of SFN and single TRP transmission schemes for PDCCH and PDSCH as summarised in Table 1. However, this is not captured at RAN1 specifications.

Table 1: PDSCH and PDCCH transmission mode in SFN mode

|  |  | PDSCH |  |  |
| --- | --- | --- | --- | --- |
|  |  | Rel-15 | SFN scheme A | SFN scheme B |
| PDCCH | Rel-15 | N/A | Supported | Supported |
|  | SFN scheme A | Supported (URLLC) | Supported | No supported |
|  | SFN scheme B | Not support | Not supported | Supported |

In the last RAN1 meeting #108e, the following TP was discussed to describe UE behaviour when it doesn’t support the scheme of SFN-PDCCH and single TRP PDSCH.

If a UE is configured with sfnSchemePdcch set to 'sfnSchemeA' for a DL BWP and activated with two TCI states by MAC CE, and the UE does not report its capability of [nonSfnPdsch-sfnPdcch], the UE does not expect to be indicated with one TCI state in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2. If a UE is configured with sfnSchemePdcch set to 'sfnSchemeB' for a DL BWP and activated with two TCI states by MAC CE, the UE does not expect to be indicated with one TCI state in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2.

There are few issues related to the proposed. Regardless that there is no RAN1 agreement with such language, the TP is not technically accurate. First, it needs to be clarified the two TCI states activation is meant for one of the CORESET. Second, the description of UE behaviour that UE doesn’t expect single TCI state codepoint is not accurate since for UE supporting dynamic switching between SFN PDSCH and single-TRP PDSCH, a single TCI state codepoint is possible.

Proposal 4:  Adopt the following (38.214) TP for the description of PDCCH/PDSCH configuration of sfnSchemA and sfnSchemeB.

TP for 38.214 Section 5.1 ----------------Unchanged part omitted------------------------ If a UE is configured with sfnSchemePdcch set to 'sfnSchemeA' for a DL BWP and CORESET is activated with two TCI states by MAC CE, and the UE does not report its capability of [nonSfnPdsch-sfnPdcch], the UE shall be configured with sfnSchemePdsch set to 'sfnSchemeA'. If a UE is configured with sfnSchemePdcch set to 'sfnSchemeB' for a DL BWP and CORESET is activated with two TCI states by MAC CE, the UE shall be configured with sfnSchemePdsch set to 'sfnSchemeB'. ----------------Unchanged part omitted------------------------

## Issue 4: CSS Type 0/0A/1/2

PDCCH candidates in CSS type 0/0A/1/2 are used for scheduling PDSCH carrying broadcast messages (SIB1, OSI, paging, MSG2/4 or paging) while PDCCH candidates in CSS type 3 are used for UE group common signalling (GC-DCI) or scheduling UE specific PDSCH using fallback DCI format.

For TPR-based pre-compensation scheme, it is not expected that network can pre-compensate the PDCCH as different UEs may experience different Doppler frequency shifts. In addition, the gNB needs to serve a mixture of legacy UEs, Rel-17 UE supporting SFN PDCCH and Rel-17 UE that don’t support SFN PDCCH reception. Therefore, legacy procedure should be maintained where PDCCH candidates in CSS type 0/0A/1/2 should not be associated with SFN CORESET.

Observation 1: SFN scheme 1 and TRP-based pre-compensation scheme are not suitable for PDCCH candidates associated with CSS type 0/0A/1/2.

Proposal 5: UE doesn’t expect PDCCH candidates in CSS to be associated with CORESET activated with two TCI states.

## Issue 4: SFN CORESET before reception of MAC-CE

In Rel-15, a list of TCI states IDs is RRC configured as part of the CORESET configuration with a maximum of 64 TCI states. A single TCI state is activated through the MAC-CE if the list has more than one TCI state ID.  If only a single TCI state is listed with the “ControlResourceSet” parameter structure and the UE did not receive MAC-CE, then the UE assume QCL between the PDCCH and SSB/CSI-RS reference signal specified by the TCI state. For the scheme-1 SFN PDCCH, the MAC CE activation is needed when the SFN CORESET is RRC configured with more than two TCI states. However, if only two TCI states are RRC configured for the SFN CORESET, there is no need for MAC-CE activation command.

Observation 2: Rel-17 MAC-CE activation of two TCI states for SFN PDCCH is required only when more than two TCI states are RRC configured in the CORESET.

Proposal 6: When SFN PDCCH is configured by RRC and a CORESET is configured with only two TCI states and UE doesn’t receive MAC-CE, the UE assumes that the DM-RS antenna port associated with PDCCH receptions in the CORESET are QCLed with the DL RSs in the two TCI states.

# Conclusion

In this contribution, we proposed:

Proposal 1: Adopt the following TP (38.214) to capture the existing agreement:

Reason for change: To capture an existing agreement that is not implemented yet in 38.214.

Summary of change: PDSCH rate matching behaviour when overlaps with SSBs associated with additional PCIs.

Consequences if not approved: Ambiguous PDSCH rate matching when overlaps with SSBs associated with additional PCIs.

============TP for 38.214 Section 5.1.4 ====================================

--Unchanged part omitted------------------------

When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, and the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. If PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources according to ssb-PositionsInBurst in SSB-MTCAdditionalPCI with same physical cell identity as the one associated with a RS having same quasi-collocation properties as the PDSCH, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.

A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH.

===============================================================

Proposal 2: Adopt the following TPs (38.213 and 38.214) regarding overlap with UL signals/channels:

Reason for change: In 38.213, the case that additional PCIs are not associated with active TCI states but still used by the UE for measurements is not addressed with respect to overlap between SSB and UL signals/channels. In 38.214, SSBs associated with additional PCIs are not considered as invalid symbols for PUSCH repetition Type B.

Summary of change: In 38.213, remove the condition “associated to physical cell ID with active TCI states” in relevant sections. In 38.214, add SSBs indicated by ssb-PositionsInBurst in SSB-MTCAdditionalPCI as invalid symbols for PUSCH repetition Type B.

Consequences if not approved: UE behaviour remains ambiguous when SSB used by the UE for measurements overlaps with UL signals/channels. PUSCH repetition Type B cannot be scheduled when overlap with SSBs associated with additional PCIs.

============TP for 38.213 Section 9.2.6 ====================================

--Unchanged part omitted------------------------

A SS/PBCH block symbol is a symbol of an SS/PBCH block with candidate SS/PBCH block index corresponding to the SS/PBCH block index indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon or, if the UE is not provided DLorJoint-TCIState or followUnifiedTCIstate, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI.

--Unchanged part omitted------------------------

===============================================================

============TP for 38.213 Section 11.1 ====================================

--Unchanged part omitted------------------------

For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE for reception of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or by ssb-PositionsInBurst in ServingCellConfigCommon or, if the UE is not provided DLorJoint-TCIState or followUnifiedTCIstate, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE.

If a UE

-	is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the multiple serving cells, and

-	indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and

-	is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells,

for a set of symbols of a slot that are indicated to the UE for reception of SS/PBCH blocks in a first cell of the multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon or, if the UE is not provided DLorJoint-TCIState or followUnifiedTCIstate, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI, the UE does not transmit PUSCH, PUCCH, or PRACH in the slot if a transmission would overlap with any symbol from the set of symbols, and the UE does not transmit SRS in the set of symbols of the slot in

-	any of the multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and

-	any one of the cells corresponding to the same band as the first cell, irrespective of any capability indicated by simultaneousRxTxInterBandCA.

--Unchanged part omitted------------------------

===============================================================

============TP for 38.213 Section 11.1.1 ====================================

--Unchanged part omitted------------------------

For a set of symbols of a slot corresponding to SS/PBCH blocks with candidate SS/PBCH block indices corresponding to the SS/PBCH block indexes indicated to a UE by ssb-PositionsInBurst in SIB1, or by ssb-PositionsInBurst in ServingCellConfigCommon, as described in clause 4.1 or, if the UE is not provided DLorJoint-TCIState or followUnifiedTCIstate, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI, the UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols of the slot as uplink.

--Unchanged part omitted------------------------

===============================================================

============TP for 38.214 Section 6.1.2.1 ====================================

--Unchanged part omitted------------------------

For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows:

-	A symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated is considered as an invalid symbol for PUSCH repetition Type B transmission.

-	For operation in unpaired spectrum, symbols indicated by ssb-PositionsInBurst in SIB1, or by ssb-PositionsInBurst in ServingCellConfigCommon, or by ssb-PositionsInBurst in SSB-MTCAdditionalPCI for reception of SS/PBCH blocks are considered as invalid symbols for PUSCH repetition Type B transmission.

-	For a reduced capability half-duplex UE in paired spectrum and for PUSCH repetition Type B transmission, symbols indicated by ssb-PositionsInBurst in SIB1, or by ssb-PositionsInBurst in ServingCellConfigCommon, or by ssb-PositionsInBurst in SSB-MTCAdditionalPCI for reception of SS/PBCH blocks are considered as invalid symbols for PUSCH repetition Type B transmission.

--Unchanged part omitted------------------------

-	If the UE

-	is configured with multiple serving cells within a cell group and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the multiple serving cells, and

-	indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and

-	is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells,

-	a symbol indicated to the UE for reception of SS/PBCH blocks in a first cell of the multiple serving cells by ssb-PositionsInBurst in SIB1, or by ssb-PositionsInBurst in ServingCellConfigCommon, or by ssb-PositionsInBurst in SSB-MTCAdditionalPCI is considered as an invalid symbol for PUSCH repetition Type B transmission in

-	any of the multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and

-	any one of the cells corresponding to the same band as the first cell, irrespective of any capability indicated by simultaneousRxTxInterBandCA

--Unchanged part omitted------------------------

===============================================================

Proposal 3:  Adopt the following editorial TP (38.214).

TP for 38.214 Section 5.2.1.5.1 ----------------Unchanged part omitted------------------------ - else if a UE is configured with sfnSchemePdcch set to 'sfnSchemeA', it is not configured with enableTwoDefaultTCI-States, and the two TCI states are activated for the CORESET by the activation command as described in clause 6.1.3.x of [10, TS 38.321] - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with an offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], periodic CSI-RS, semi-persistent CSI-RS, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48} and when enableBeamSwitchTiming is not provided or the NZP-CSI-RS-ResourceSet is configured with the higher layer parameter trs-Info , aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'off' or configured without the higher layer parameters repetition and trs-Info scheduled with offset larger than or equal to 48 when the UE provides beamSwitchTiming-r16 and enableBeamSwitchTiming is provided, aperiodic CSI-RS in a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on' scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming-r16 and enableBeamSwitchTiming is provided; - else if, the UE applies the first one of two TCI states indicated for the CORESET with the lowest CORESET ID in the latest slot within the active BWP of the cell in which the CSI-RS is to be received when receiving the aperiodic CSI-RS, ----------------Unchanged part omitted------------------------

Proposal 4:  Adopt the following (38.214) TP for the description of PDCCH/PDSCH configuration of sfnSchemA and sfnSchemeB.

TP for 38.214 Section 5.1 ----------------Unchanged part omitted------------------------ If a UE is configured with sfnSchemePdcch set to 'sfnSchemeA' for a DL BWP and CORESET is activated with two TCI states by MAC CE, and the UE does not report its capability of [nonSfnPdsch-sfnPdcch], the UE shall be configured with sfnSchemePdsch set to 'sfnSchemeA'. If a UE is configured with sfnSchemePdcch set to 'sfnSchemeB' for a DL BWP and CORESET is activated with two TCI states by MAC CE, the UE shall be configured with sfnSchemePdsch set to 'sfnSchemeB'. ----------------Unchanged part omitted------------------------

Proposal 5: UE doesn’t expect PDCCH candidates in CSS to be associated with CORESET activated with two TCI states.

Proposal 6: When SFN PDCCH is configured by RRC and a CORESET is configured with only two TCI states and UE doesn’t receive MAC-CE, the UE assumes that the DM-RS antenna port associated with PDCCH receptions in the CORESET are QCLed with the DL RSs in the two TCI states.