| 3GPP TSG-RAN WG1 Meeting #109-e e-Meeting, May 9th – May 20th , 2022 | R1-2204978 |
| --- | --- |

Agenda item:	8.1.3

Source: 	Qualcomm Incorporated

Title: 	Remaining issues on SRS enhancement

Document for:	Discussion/Decision

# Remaining issues on SRS enhancements

## Issue #1: Maximum value of available slot offset ‘t’

The RRC parameter ‘AvailableSlotOffset’ is a list of up-to 4 values where each entry in the list can take values from the list {0,1,2,3 …,7}. In some slot format configuration that is DL heavy, e.g., DDDDDDDDDU, when network configures and triggers SRS set with indicated available slot offset of t=7, the UE needs to keep the bookkeeping up to 80 slots which exceeds legacy UE behaviour where maximum of value of slot offset is 32 physical slots from the triggering DCI.

Observation 1: The maximum value of available slot offset ‘t=7’ in some slot format pattern maps into large number of physical slots offsets from triggering DCI which increase UE overhead for bookkeeping.

To resolve this issue, UE should drop the triggered A-SRS set after a certain span of time if the indicated available slot is not met.

Proposal 1: UE drops the triggered A-SRS if the available slot offset is not reached within a specific span of time.

## Issue #2: Inter-set guard period

In last RAN1 meeting, it was agreed that UE does not transmit any other signal on any symbols of the interval if the interval between SRS resource sets is Y symbols. For the case when time interval between the two SRS resource sets is larger than minimum guard period Y, RAN4 agreed that UE can transmit other signals in between SRS sets.

Agreement UE is able to transmit other signals in-between SRS resource sets if the interval in-between SRS resource sets is larger than Y - FFS on which symbols can be available for transmission

However, it is not clear which symbol(s) are used as guard symbols and which symbols can be used for other signals transmission. In addition, this agreement needs to be reflected into RAN1 specification.

Proposal 2: For inter-set guard period, when the interval between SRS resource sets is larger than the minimum Y guard period, UE can transmit other signals in the interval except the guard symbol(s).

Proposal 3: RAN1 to clarify where the guard symbol(s) are located within the interval between the two SRS resource sets when the interval is larger than the minimum Y guard period.

## Issue #3: Maximum number of configured SRS sets

For SRS antenna switching 1T2R, 2T4R and 1T4R, Rel-17 introduced two new features for extending the maximum number of SRS sets based on UE capability indication. However, there is some confusion regarding the maximum number of configured P/SP/AP SRS sets for antenna switching sets when either one of the two capabilities is reported and should be clarified [2].  When the UE doesn’t support either capability, then Rel-15 specification is followed where maximum of two sets with different values of resource type (P, SP, AP) could be configured. When the UE supports both capability of extending number of SRS sets, then up to five sets (2 SP sets plus 1 P SRS set plus 2 AP sets) for 1T2R/2T4 and up to seven sets (2 SP sets plus 1 P set plus 4 AP sets) for 1T4R. However, the confusion comes when UE supports only of the extension’s features. Taking SRS antenna switching of 1T2R/2T4R as an example, then when the UE supports only extension of AP sets, it needs to be clarified whether the UE can be configured with maximum of 3 SRS sets (e.g., 1 P set + 2 AP set) or not. In addition, when the UE supports only extension of SP/S sets, it needs to be clarified whether Rel-15 restrictions are completely relaxed, or it means increasing the maximum number of sets to three.

Proposal 4: For 1T2R, 2T4R and 1T4R, further clarify the maximum number of configured SRS sets for antenna switching when the UE supports only the extension of maximum number of AP sets.

Proposal 5: For 1T2R, 2T4R and 1T4R, further clarify the maximum number of configured SRS sets for antenna switching when the UE supports only the extension of maximum number of SP/P sets.

# Conclusion

In this contribution, we presented our views on the SRS enhancements techniques. Below is the summary of our conclusion:

Observation 1: The maximum value of available slot offset ‘t=7’ in some slot format pattern could results into large number of physical slots from triggering DCI which increase UE overhead for bookkeeping.

Proposal 1: UE drops the triggered A-SRS if the available slot offset is not reached within a specific span of time.

Proposal 2: For inter-set guard period, when the interval between SRS resource sets is larger than the minimum Y guard period, UE can transmit other signals in the interval except the guard symbol(s).

Proposal 3: RAN1 to clarify where the guard symbol(s) are located within the interval between the two SRS resource sets when the interval is larger than the minimum Y guard period.

Proposal 4: For 1T2R, 2T4R and 1T4R, further clarify the maximum number of configured SRS sets for antenna switching when the UE supports only the extension of maximum number of AP sets.

Proposal 5: For 1T2R, 2T4R and 1T4R, further clarify the maximum number of configured SRS sets for antenna switching when the UE supports only the extension of maximum number of SP/P sets.

# Reference

- RP-193133, “WID proposal FeMIMO”, Samsung.,

- R1-2112949, “Introduction of further enhancements on MIMO for NR”, Nokia.