3GPP TSG-RAN WG1 #109-e	R1- 2204979

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.2.2

Source: 	Qualcomm Incorporated

Title: 	PDCCH monitoring enhancements

Document for:	Discussion/Decision

# Introduction

For PDCCH monitoring enhancements for FR2-2 in Rel-17, the following WI scope has been specified [1]:

- Support enhancement to PDCCH monitoring, including blind detection/CCE budget, and multi-slot span monitoring, potential limitation to UE PDCCH configuration and capability related to PDCCH monitoring.

In this contribution, we present our views on the remaining issues of multi-slot PDCCH monitoring.

# Discussion

## Monitoring occasions of Group (2) SS sets in a slot group

In RAN1 #108-e, as a part of the agreement on the SS set configuration, the following has been captured:

- Introduce new parameter monitoringSlotsWithinSlotGroup-r17 - The size is 8 bitsTwo sizes L are supported for this parameter: L=4 bits and L=8 bits - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group of L slots - The parameter monitoringSlotsWithinSlotGroup-r17 is applied in each of the L slots as determined by monitoringSlotPeriodicityAndOffset-r17 and duration-r17. - A slot in the slot each group of L slots is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group monitoringSlotsWithinSlotGroup-r17 is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - For Group (1) SSs - The slots indicated in the bitmap should be consecutive per group of L slots - The number of slots configured for multi-slot PDCCH monitoring per slot group of Xs slots should be no larger than Ys according to at least one of the Xs,Ys supported by a UE - Working assumption: For Group (2) SSs - For Type0/0A/2 CSS - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 can be up to L - For Type1 CSS without dedicated RRC - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 per slot group of Xs slots should be no larger than M, where M is FFS

For Group (1) SS sets, the agreement restricts that the slots indicated in the bitmap, monitoringSlotWithinSlotGroup-r17, should be consecutive. For example, based on the agreement, for the capability Xs,Ys=(8,4), the PDCCH monitoring slots within a slot group can have one of the configurations shown in Figure 1 (a), with different values of the bitmap. However, to comply with a combination of (Xs,Ys), where Ys>1, the consecutiveness restriction is not always necessary and may limit the flexibility of configuration. For instance, the configurations of PDCCH monitoring slots shown in Figure 1 (b) still comply with the capability Xs,Ys=(8,4), while the indicated slots in the bitmap are not consecutive. Therefore, for enhanced flexibility, the following modification of the agreement can be considered:

- For Group (1) SSs - The slots indicated in the bitmap should be consecutive per group of L slots - The number of slots configured for multi-slot PDCCH monitoring per slot group of Xs slots should be no larger than Ys according to at least one of the Xs,Ys supported by a UE - In the bitmap, the separation between the first indicated slot (i.e., the fist ‘1’ in the bitmap) and the last indicated slot (i.e., the last ‘1’ in the bitmap) should be less than Ys

Proposal 1: For enhanced flexibility, the following revision for the former agreement on Group (1) SS set configuration is considered:

- For Group (1) SSs

  - The number of slots configured for multi-slot PDCCH monitoring per slot group of Xs slots should be no larger than Ys according to at least one of the Xs,Ys supported by a UE

  - In the bitmap, the separation between the first indicated slot (i.e., the fist ‘1’ in the bitmap) and the last indicated slot (i.e., the last ‘1’ in the bitmap) should be less than Ys

In the Appendix, a text proposal for Proposal 1 is provided. As will also be discussed in Section 2.3, relaxing the consecutiveness restriction of monitoring slots can improve the flexibility of intra-slot-group PDCCH repetition configuration.

| (a) | (b) |
| --- | --- |

Figure 1: Configuration of PDCCH monitoring slots for Xs,Ys=(8,4); (a) with consecutive slot restriction, and (b) without consecutive slot restriction.

For Type1 CSS without dedicated RRC in Group (2) SS sets, the number of monitoring slot, M, is still FFS. Type1 CSS without dedicated RRC can be configured with the searchSpace IE in SIB1 (i.e., ra-SearchSpace) and used for initial access. Based on another agreement in RAN1 #108-e, when the higher layer parameter monitoringCapabilityConfigConfig is not provided (e.g., during the initial access), the combinations Xs,Ys=(4,1) and Xs,Ys=(8,1) are regarded as the default values for 480 kHz and 960 kHz SCSs, respectively. As such, the configuration of Type1 CSS without dedicated RRC should also be compliant with the agreement, and it is fair to assume M=1.

Proposal 2: For Type1 CSS without dedicated RRC, the number of slots configured for multi-slot PDCCH monitoring in monitoringSlotWithinGroup-r17 per slot group of Xs slots should be no larger than one.

- Xs=4 for 480 kHz SCS and Xs=8 for 960 kHz SCS

For Type0/0A/2 CSS, the current agreement does not pose any restrictions to the configuration of monitoring slots within a slot group. For example, all the Xs slots within a slot group can be configured for Type0/0A/2 CSS monitoring. However, based on the legacy rule for determining Type0/0A/2 CSS MOs, even though the configuration of monitoring slot is not restricted, the UE only picks one Type0/0A/2 CSS MO within a SI window or a paging frame according to the association with SSBs. Therefore, the number of actually monitored slots for Type0/0A/2 CSS within a slot group is implicitly restricted to at most one in most practical deployments. However, in some deployment scenario where the number of transmitted SSBs, N, is smaller than Xs, (i.e., N<Xs), the UE may be required to monitor more than one CSS MO within a slot group. To avoid such a situation, in RAN #108-e, some alternatives for a further restriction of Group (2) SS sets were discussed:

[FL NOTE: The following two aspects could be seen as alternative solutions, therefore labbelled as [Alt 1] and [Alt 2]] - [Alt 1] If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and - a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot group of Xs slots. - [Alt 2] Among the all configured Group (2) MOs, a UE is not required to monitor more than Z Group (2) MOs within a slot group of Xs slots - Z < Xs, FFS detailed values, e.g., Z = 1

In effect, both of the above alternatives can restrict the number of actually monitored Group (2) MOs. The difference is that Alt 1 is an implicit restriction and Alt 2 is an explicit restriction. For Alt 1, although some clarification may be required, in our understanding, the phrase “process information” includes monitoring (blind decoding) of PDCCH candidates with the corresponding RNTIs. As such, Alt 1 implies that, irrespective of the configured MOs of Type0/0A/2 CSS within a slot group, the UE does not expect to actually monitor more than one MO. The actually monitored MO would be determined by the legacy SSB-MO association rule. For the aforementioned case that the number of transmitted SSBs, N, is less than Xs, it is up to the network to configure Type0/0A/2 CSS appropriately so that the UE is not required to monitor more than one MO per slot group. For example, the network may use a SS set periodicity Xp for Group (2) SS sets such that NXp>Xs.

Since Alt 1 is a direct extension of Rel-15 behavior to multi-slot PDCCH monitoring, the standardization impact would be smaller compared to Alt 2. Therefore, Alt 1 is preferred to Alt 2.

Proposal 3: If a UE is provided

- one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and

- a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI

then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot group of Xs slots, where Xs=4 for 480 kHz SCS and Xs=8 for 960 kHz SCS.

In the Appendix, a text proposal for Proposal 3 is provided.

## Search space set group switching

When a UE reports more than one Xs,Ys combinations for its multi-slot PDCCH monitoring capability, two SSSGs may be associated with different Xs,Ys values for better flexibility. In principle, this implies that the BD and CCE budgets is determined per SSSG, and also the overbooking and dropping rule is applied per SSSG. For example, when a UE with 960 kHz SCS supports both (4,1) and (8,1), SSSG#0 may be configured according to (4,1), and SSSG#1 may be configured according to (8,1). SSSG#0 may provide more frequent PDCCH MOs, once in 4 slots, although the BD and CCE budgets are limited (i.e., 10 BD and 16 CCEs). On the other hand, SSSG#1 may be configured with PDCCH MOs with a periodicity larger than or equal to 8 slots and provide improved power efficiency and larger BD and CCE budgets (i.e., 20 BD and 32 CCEs). In the same situation, on the contrary, if the two SSSGs are restricted to have the same Xs,Ys value, (4,1) should be applied for both SSSGs. This will limit the BD and CCE budgets of both SSSGs to 10 BD and 16 CCEs and, as a results, may harm the scheduling flexibility of SSSG#1. Thus, it is beneficial to allow different Xs,Ys values, if supported, for different SSSGs.

As a consequence of the different Xs,Ys values, the location of Ys slots within a group of Xs slots may change across SSSGs, particularly when the values of Xs are different across SSSGs. However, as agreed earlier, the position of Ys slots should be maintained across slot groups of the same SSSG. Also, in RAN1 #108-e, it was agreed that the boundary of SSSG switching is always aligned with the boundary of the slot group. If the two SSSGs are configured with different values of Xs, the slot group boundaries of the SSSG with a smaller Xs value are nested within those of the SSSG with a larger Xs value. In such a case, it would be desirable to align the SSSG switching boundary with the common slot group boundary of the two SSSG, i.e., the boundary associated with the larger XS value.

Proposal 4: When a UE supports more than one Xs,Ys combinations for multi-slot PDCCH monitoring, the applied Xs,Ys value is determined per SSSG.

- Maximum BD/CCE budgets and overbooking/dropping is applied per SSSG according to the corresponding Xs,Ys value.

- The location of Ys slots within a group of Xs slots is maintained across slot groups of a SSSG, but may be different across SSSGs.

- When two SSSGs are associated with different slot group sizes Xs, the boundary of SSSG switching is aligned with the boundary of the slot group associated with the larger Xs value between the two SSSGs.

Further related to the joint configuration of multi-slot PDCCH monitoring and SSSG switching, a situation shown in Figure 2 may be considered. That is, SSSG switching occurs from the first SSSG to the second SSSG. Then, at the boundary of SSSG switching, the locations of the two Ys consecutive slots in the slot groups before and after the switching boundary may be different. In another case, the two SSSGs may be associated with different Xs,Ys values according to Proposal 4. Thus, in such cases, the separation between the two Ys consecutive slots may be less than Xs slots.

To avoid such a situation in Figure 2, a rule for dropping PDCCH MOs may be considered. For example, when the separation between the two Ys consecutive slots before and after the switching boundary is less than Xs slots, where Xs is according to the first (source) SSSG, some or all of the MOs in the Ys consecutive slots in the second (target) SSSG after the switching boundary may be dropped.

Proposal 5: A dropping rule for PDCCH MOs may be applied for the first Ys consecutive slots after SSSG switching, if the separation between the two Ys consecutive slots before and after the SSSG switching boundary is less than Xs slots.

Figure 2: An example of SSSG switching for multi-slot PDCCH monitoring.

## PDCCH repetition for Group (1) SS sets

Intra-slot PDCCH repetition, which is specified in Rel-17 as a part of multi-TRP enhancement, can be applied for slot-based and span-based PDCCH monitoring. Although the application of PDCCH repetition for multi-slot PDCCH monitoring has not been actively discussed yet, it would be fair to say that it should also be applicable for multi-slot PDCCH monitoring. For the Group (1) SS set configuration, the searchSpace IE may also provide searchSpaceLinking, which indicates another SS linked for PDCCH repetition. For SS sets linked for PDCCH repetition, some restrictions are posed for the values of some configuration parameters. That is, the two linked SS sets should be configured with the same number of PDCCH candidates for each aggregation level, the same number of MOs per slot, the same periodicity, offset, and duration. Likewise, when the PDCCH repetition is to be applied for multi-slot PDCCH monitoring, similar restrictions for the new parameters, which are related to the configuration of multi-slot PDCCH monitoring, should be considered.

Case 1) Ys=1

With YS=1, the UE supports FG3-5b-like configuration for Group (1) SS sets, and there can be up to two monitoring spans in a slot. Therefore, when two MOs within a slot are associated with different SS sets, they can be linked for the PDCCH repetition, as illustrated in Figure 3. As in the case of slot/span-based PDCCH monitoring, the two linked SS sets are restricted to have the same values of monitoringSlotPeriodicityAndOffset-r17 and duration-r17, and the same number of MOs per slot, which is one in this case. In addition, the number and the location of monitoring slots within a slot group should be the same for the linked SS sets. Thus, the size (4 or 8 bits) and the value of monitoringSlotsWithinSlotGroup-r17 should be identical for the linked SS sets.

Observation 1: For Ys=1, to enable intra-slot PDCCH repetition for multi-slot PDCCH monitoring, the two linked search space sets should have the same values for parameters including monitoringSlotPeriodicityAndOffset-r17, duration-r17, and monitoringSlotsWithinSlotGroup-r17.

Figure 3: Search space set linkage for PDCCH repetition for XS,Ys=(4,1).

Case 2) Ys>1

With Ys>1, the UE supports FG3-1-like configuration for Group (1) SS sets, and a MO should be within the first 3 OFDM symbols of a slot. Thus, for intra-slot PDCCH repetition, two MOs of different SS sets should be placed within the first 3 symbols, as illustrated in Figure 4 (a). In particular, if the two SS sets are associated with different CORESETs with different TCI states, a gap for beam switching may be required between the MOs, which makes the PDCCH repetition even more challenging for Ys>1.

Observation 2: For Ys>1, due to the limitation of FG3-1-like monitoring occasions, intra-slot PDCCH repetition may not be possible.

As such, further discussion on any potential enhancements for multi-slot PDCCH monitoring is necessary. In the following paragraphs, we discuss some alternatives of the enhancements.

Alt 1: As an optional UE capability, FG3-5b-like configuration of MOs for Ys>1 may be allowed for the UE. With the new capability, there could be up to two monitoring spans within each slot of the Ys slots, and intra-slot PDCCH repetition as shown in Figure 3 would be possible.

Alt 2: For multi-slot PDCCH monitoring, instead of intra-slot PDCCH repetition, intra-slot-group PDCCH repetition may be considered. Since the principle of multi-slot PDCCH monitoring is to change the time unit of PDCCH processing from a slot to a slot group, the intra-slot-group PDCCH repetition would be a legitimate extension of the intra-slot PDCCH repetition. Figure 4 (b) shows an example of the two linked SS sets across slots within a slot group. In this example, due to the consecutiveness restriction of monitoring slots discussed in Section 2.1, all the monitoring slots of the second SS set come after those of the first SS set. As discussed in the Rel-17 multi-TRP enhancement session, however, the SS set linkage pattern in Figure 4 (b) potentially requires a longer processing delay and a larger memory space. To avoid any potential issues, an interleaved pattern of SS sets as shown in Figure 4 (c) may also be considered. Note that, the interleaved pattern in Figure 4 (c) can be achieved more efficiently by allowing the non-consecutive monitoring slot configuration, as proposed in Proposal 1.

Proposal 6: Any potential enhancements of multi-slot PDCCH monitoring to enable PDCCH repetition should be discussed for Ys>1.

- Alt 1: FG3-5b-like optional UE capability for Ys>1

- Alt 2: Intra-slot-group PDCCH repetition as an extension of intra-slot PDCCH repetition

(a)

(b)

(c)

Figure 4: Search space set linkage for PDCCH repetition for Xs,Ys=(8,4); (a) intra-slot repetition, (b) intra-slot-group repetition with consecutive monitoring slots, (c) intra-slot-group repetition with non-consecutive monitoring slots.

# Conclusion

Observation 1: For Ys=1, to enable intra-slot PDCCH repetition for multi-slot PDCCH monitoring, the two linked search space sets should have the same values for parameters including monitoringSlotPeriodicityAndOffset-r17, duration-r17, and monitoringSlotsWithinSlotGroup-r17.

Observation 2: For Ys>1, due to the limitation of FG3-1-like monitoring occasions, intra-slot PDCCH repetition may not be possible.

  - XsYsXs,Ys

  - Ys

Proposal 1: For enhanced flexibility, the following revision for the former agreement on Group (1) SS set configuration is considered:

- For Group (1) SSs

  - The number of slots configured for multi-slot PDCCH monitoring per slot group of Xs slots should be no larger than Ys according to at least one of the Xs,Ys supported by a UE

  - In the bitmap, the separation between the first indicated slot (i.e., the fist ‘1’ in the bitmap) and the last indicated slot (i.e., the last ‘1’ in the bitmap) should be less than Ys

Proposal 2: For Type1 CSS without dedicated RRC, the number of slots configured for multi-slot PDCCH monitoring in monitoringSlotWithinGroup-r17 per slot group of Xs slots should be no larger than one.

- Xs=4 for 480 kHz SCS and Xs=8 for 960 kHz SCS

Proposal 3: If a UE is provided

- one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and

- a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI

then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot group of Xs slots, where Xs=4 for 480 kHz SCS and Xs=8 for 960 kHz SCS.

Proposal 4: When a UE supports more than one Xs,Ys combinations for multi-slot PDCCH monitoring, the applied Xs,Ys value is determined per SSSG.

- Maximum BD/CCE budgets and overbooking/dropping is applied per SSSG according to the corresponding Xs,Ys value.

- The location of Ys slots within a group of Xs slots is maintained across slot groups of a SSSG, but may be different across SSSGs.

- When two SSSGs are associated with different slot group sizes Xs, the boundary of SSSG switching is aligned with the boundary of the slot group associated with the larger Xs value between the two SSSGs.

Proposal 5: A dropping rule for PDCCH MOs may be applied for the first Ys consecutive slots after SSSG switching, if the separation between the two Ys consecutive slots before and after the SSSG switching boundary is less than Xs slots.

Proposal 6: Any potential enhancements of multi-slot PDCCH monitoring to enable PDCCH repetition should be discussed for Ys>1.

- Alt 1: FG3-5b-like optional UE capability for Ys>1

- Alt 2: Intra-slot-group PDCCH repetition as an extension of intra-slot PDCCH repetition

# References

- RP-211584, “Revised WID: Extending current NR operation to 71GHz”, RAN #92-e, Qualcomm, Intel

# Appendix

According to Proposal 1 and Proposal 3, the text proposal for Section 10.1 in TS 38.213 is provided:

## 10.1 UE procedure for determining physical downlink control channel assignment ## *** Unchanged text is omitted *** If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and - a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot. - for SCS configuration μ∈0, 1, 2, 3, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot, and - for SCS configuration μ∈5, 6, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per group of 4 slot for μ=5 and 8 slots for μ=6. ## *** Unchanged text is omitted *** - a bitmap, by monitoringSlotsWithinSlotGroup, that applies per group of slots and provides a PDCCH monitoring pattern indicating slots in a group of slots for PDCCH monitoring - a size of the group of slots is same as a size of monitoringSlotsWithinSlotGroup - for a Type1-PDCCH CSS set provided by ra-SearchSpace in dedicated RRC signaling, or for a Type3-PDCCH CSS set, or for a USS set, the PDCCH monitoring pattern indicates only consecutive slots in the group of slots for PDCCH monitoring and, at least for one combination Xs,Ys indicated by the UE as a capability, a number of the consecutive indicated slots is not larger than Ys and the separation between the first indicated slot and the last indicated slot is less than Ys ## *** Unchanged text is omitted ***