3GPP TSG-RAN WG1 #109-e	R1-2204981

e-Meeting, May 9th – May 20th  , 2022

Agenda item:    8.2.4

Source:              Qualcomm Incorporated

Title:                  Channel Access Mechanisms for NR in 52.6 to 71GHz band

Document for:  Discussion and Decision

# Introduction

The proposals in this paper are designed to reach quick consensus taking into account various company positions at the end of RAN1#108-e meeting.  The proposals are organized by topics.

The topics covered are mainly as follows:

- EDT values with LBT bandwidth is relaxed

- Short Control Signaling exemption for UL control transmission

- Multi-beam channel access

- Type 1/Type 2 LBT handling in LBT upgrade in COT sharing

- COT resuming at UE

Our emphasis on the set of solutions at this point in the work item is to reduce the impact on RRC and avoid introducing large scale RRC changes. The expectation is that other companies too see the merit in this approach and adopt solutions that reduce RRC impact.

# Adaptation for EDT value for relaxed LBT Bandwidth

By RAN1-108e it was agreed that nodes should be allowed to perform LBT over a bandwidth that includes the corresponding active BWP in that carrier.

Agreement For LBT for single carrier UL transmission, UE performs LBT over a BW that at least includes the active UL BWP bandwidth - The BW that at least includes the active UL BWP bandwidth is captured as “channel” in 37.213 Agreement For LBT for single carrier DL transmission to a UE, gNB performs LBT over a bandwidth that at least includes the active DL BWP bandwidth configured for that UE. - This does not rule out gNB implementation to perform LBT over a wider bandwidth - The BW that at least includes the active DL BWP bandwidth is captured as “channel” in 37.213 For LBT for single carrier DL transmission to multiple UEs, from each UE point of view, gNB performs LBT over a bandwidth that at least includes the active DL BWP bandwidth configured for that UE. - This does not rule out gNB implementation to perform LBT over a wider bandwidth that includes the active DL BWP of multiple UEs

It was under active discussion whether the ED Threshold used reflect the actual LBT bandwidth used or DL or UL BWP bandwidth.  The consensus seems to be leaning towards EDT to reflect the actual LBT bandwidth selected.

Yet, this implies that EDT can go without constraint as LBT bandwidth increases under the current formulation. Instead we would like to put a limit on the maximum ED threshold increase. The key value of  maximum ED threshold considered could be -47 dBm which also matches well with the ED threshold used for WiGig bandwidth of 2.16 GHz.

Proposal 1:  EDT for gNB initiated and UE initiated COT   is determined by the actual LBT bandwidth selected, with a maximum of -47 dBm.   Adopt the Text proposal A.

Text Proposal A =====For 37.213 4.4==== 4.4 Channel access procedures for frequency range 2-2 **Unchanged part omitted ** 4.4.7 Energy detection threshold adaptation procedures A gNB/UE accessing a channel on which transmission(s) on beam(s) are performed within a channel occupancy, shall set the energy detection threshold XThresh to be less than or equal to the maximum energy detection threshold XThresh_max that is determined as follows: XThresh_max_nominal=-80dBm+Pmax- Pout+ 10⋅log10BW XThresh_max=min⁡(XThresh_max_nominal, -47dBM) where: - Pmax is the RF output power limit in dBm - Pout is the maximum EIRP of the intended transmission(s) by the gNB/UE to acquire a channel occupancy in dBm where Pout≤Pmax. The maximum EIRP used for the transmission(s) by the initiating gNB/UE during the channel occupancy is limited to Pout. - BW is at least the uplink active bandwidth part bandwidth in MHz for UE and at least the downlink active bandwidth part bandwidth in MHz for gNB. =====End of TP========

# Short Control Signaling for UL Transmission

The agreements so far regarding the Short Control Signaling exemption for UL Transmission are as follows.

Agreement: - Contention Exempt Short Control Signaling rules apply to the transmission of msg1 and/or msg3 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS. - Note restriction for short control signalling transmissions apply (10% over any 100ms intervals) - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msg3/msgA transmission from one UE perspective - FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc Conclusion In Rel.17, there is no consensus to apply contention exemption short control signalling to UL transmissions other than msg1 and msgA.

Based on the positions reached at the end of RAN1-108-e, the common denominator agreement for UL short control signaling is to limit the total configured resources for transmission of msg1/msgA to at most 10ms every 100 ms. The following is proposal 2.7-2 from [1].

Proposal 2:  For Contention Exempt Short Control Signaling based transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS, the 10% over any 100ms intervals restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell.

•	This 10% allowance is separated from the 10% allowance for gNB

Proposal 3:  Adopt the text proposal B.

TP B ===================== for TS 37.213 ============= 4.4.5 Exempted transmissions from sensing In regions where channel sensing is required to access a channel for transmission and short control signalling exemption is allowed by regulation, a gNB/UE may transmit the following transmission(s) on a channel without sensing the channel: - Transmission(s) of the discovery burst by the gNB - Transmission(s) of the first message in a random access procedure by the UE When the gNB/UE transmits the above transmission(s) without sensing on a channel by utilizing the exemption above, the total duration of such transmission(s) by the gNB/UE shall not occupy the corresponding channel more than 10ms over any 100ms interval. The total configured resources for transmission(s) of the first message in a random access procedure shall not occupy the corresponding channel more than 10ms over any 100ms interval. ========================================

Given that the network may or may not choose to permit contention exempt msg1/msg A transmissions irrespective of the region of deployment, we need a RRC configuration to inform the UE of the contention exemption. This corresponds to Alt 2 in the discussion proposal 2.15-1 in [1].

Proposal 4:  gNB provides separate RRC configuration in SIB1 to indicate if contention exempt transmission of first message of the RACH procedure (msg1 or msgA) is allowed.

- It is a separate discussion if the requirement of 10% over 100ms is per UE or per cell.

# Multi-Beam Channel Access

Some of the relevant agreements regarding multi-beam COT reached by the end of RAN1-108e are as follows.

Agreement When independent per-beam LBT sensing is performed at gNB, a transmission is allowed to occur on a beam if the corresponding LBT procedure has been successful before the channel occupancy start time. - Note: For multi-beam transmission, channel occupancy start time corresponding to all Tx beams is aligned. - FFS: When independent per-beam LBT sensing is performed at UE

On the gNB as well as UE side the Multi-Beam Channel access procedure could closely mimic the Multi-Channel LBT procedure treating beams as though they are separate channels. The options available on the gNB and UE side then appear as follows. This proposal is extension of the Proposal 2.3-3b in [1], where the operation is extended to the UE.

Proposal 5: When independent per-beam LBT sensing is performed as initiators nodes at gNB or UE, each time the gNB or UE attempts to acquire a COT

- Apply independent Type 1 channel access to each beam

- the gNB and UE   shall re-initialize the counter for each beam

- the initial value of the counter is independently determined for each beam

- count-down process is independent for each beam

- Start of the channel occupancy time in all beam is aligned.

- To acquire a new COT, the applied Type 1 channel access process to each beam shall not start before the end of the previous COT

The rule for performing independent per beam sensing are described in Proposal 5. UE side independent per-beam LBT sensing eases the channel access by permitting transmission on a subset of beams where LBT is successful and therefore is desirable. On the other hand  if the network needs to restrict the behavior of UEs to either ‘all or nothing’ channel occupancy, it should have the flexibility to do so. We propose to support both modes of COT occupancy, as follows.

Proposal 6:  When independent per-beam LBT sensing is performed at UE, support both the following modes

- Mode A: The UE occupies the COT on the subset of beams where the LBT procedure is successful

- Mode B: The UE occupies the COT only if the LBT procedure is successful on all beams

The following proposal connects the EDT threshold computation for independent per beam LBT with the sensing beam used for performing the LBT operation.  This position has support from a majority of companies.

Proposal 7:  For a COT with MU-MIMO (SDM) transmission or TDM transmission of beams with beam switching, when independent per-beam LBT is performed at the start of the COT, for Pout in EDT determination of LBT for each sensing beam define Pout as the maximum EIRP of the intended transmissions “covered” by the sensing beam by the node determining EDT during a COT.

# LBT Upgrade in COT Sharing: Channel Access Type Change

The scenario for this aspect is as follows. When an uplink transmission instance falls within a COT occupied by gNB, then UE should be able to use the benefit of COT sharing and consequent ability to use Type 2 or Type 3 LBT for sensing. This needs to hold even if the uplink transmission was indicated under a scheduled grant or configured under a configured grant to use a Type 1 channel access procedure. Further, the UE and gNB need to have a common understanding of the channel access type used by the UE, and therefore something akin to an RRC configuration is required to control the type of channel access the UE will use when ‘upgrading’ its LBT type from Type 1.  At the end of RAN1-108-e, the chief debate is between unified signaling via a single SIB1 bit  or a separate  RRC Configuration. As the focus of the remaining work is to reduce changes to the RRC as much as possible, a possible way out it to use an existing indicator for determining the channel access type for such an ‘LBT Upgrade’. A prime candidate to use is the RRC Configuration for setting up DCI 0_1 (or DCI 1_1), which configures the valid Channel Access Types schemes for the UE. The UE can change the channel access type to Type 2 if Type 2 is configured as a channel access type in DCI 0_1 otherwise, the UE uses Type 3.  The idea is that if the gNB is configuring Type 2 LBT channel access type for  the UE, for example due to the requirement of the region, then the channel access type change to Type 2 should be available to the UE.  The proposal saves additional RRC modifications by two related channel access type decisions.

Proposal 8:  For an UL transmission indicated or configured to use Type 1 channel access, if the UE later finds out, through DCI 2_0 detection, the transmission falls is in a gNB COT, the UE can change the channel access type to either Type 2 channel access or Type 3 channel access, based on the following rule.

- If Type 2 Channel Access is configured as one of the entries in the configuration for DCI 0_1 (or DCI 1_1), the UE will use Type 2 channel access.

- If Type 2 Channel Access is not configured as one of the entries in the configuration for DCI 0_1, the UE will use Type 3 channel access

# COT Resumption at UE initiated COT

On the gNB side the agreements upto RAN1-108e cover the resumption of COT after a gap, whenever permitted.

Agreement Support gNB as the initiating device to resume transmission within maximum COT without a Cat 2 LBT, no matter how long the gap is from the previous transmission from initiating device or responding device - Note: This is motivated by regions where LBT is not required before each transmission (say outside Japan)? - Note: This should only be used when allowed by local regulation Agreement Support gNB as the initiating device to resume transmission with a Cat 2 LBT if there is gap longer than Y us from the previous transmission from initiating device or responding device - Note this is motivated by regions where LBT is required before each transmission (say Japan) - Y is left for initiating device implementation and should comply with local regulation but no less than 8us

The issue arises in the case where UE as the initiating device resume transmission after a gap within a COT. Two potential behaviors are possible.

- Behavior 1: Type 3 channel access (no LBT) no matter how long the gaps is from the previous transmission from initiating device or responding device

- Behavior 2: Type 2 channel access (Cat 2 LBT) if there is gap longer than Y us from the previous transmission from initiating device or responding device

If we support this functionality, we also need a mechanism to choose between the two behaviors. The following alternatives were discussed as part of RAN1-108e discussions.

- Alt 1: RRC configures between behavior 1 and behavior 2. RRC may also  need to configure Y.

- Alt 2: RRC configures if LBT is always needed for each transmission (like in Japan), and UE can adopt behavior 1 if LBT is not always required and adopt behavior 2 if LBT is always required.

- Alt 3: Leave the choice by UE implementation without additional RRC signaling.

- Not support this functionality.

Following the same principle as the previous section on Channel Access Type change, we can achieve a configurable support of this feature without RRC modification by tying the channel access type after a gap to the availability of Type 2 Channel Access Type for the RRC Configuration of DCI 0_1.  The idea is that if the network configures a UE for Type 2 Channel Access for DCI 0_1, for example, to satisfy regional requirements on sensing, the UE can use Type 2 channel access while resuming the COT after a gap. Otherwise, the UE will use Type 3 channel access.  The following proposal captures this choice that avoids further RRC modification.

Proposal 9 For a UE initiated COT, the channel access type for resuming a COT after a gap of Y us from previous transmission will be as follows:

- If Type 2 Channel Access is configured as one of the entries in the configuration for DCI 0_1 (or DCI 1_1),  the UE uses Type 2 Channel access

- If Type 2 Channel Access is not configured as one of the entries in the configuration of DCI 0_1 (or DCI 1_1) the UE uses Type 3 channel access.

# Conclusions

Following is the list of proposals from this paper.

Proposal 1:  EDT for gNB initiated and UE initiated COT   is determined by the actual LBT bandwidth selected, with a maximum of -47 dBm.   Adopt the Text proposal A.

Proposal 2:  For Contention Exempt Short Control Signaling based transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS, the 10% over any 100ms intervals restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell.

•	This 10% allowance is separated from the 10% allowance for gNB

Proposal 3:  Adopt the text proposal B.

Proposal 4:  gNB provides separate RRC configuration in SIB1 to indicate if contention exempt transmission of first message of the RACH procedure (msg1 or msgA) is allowed.

It is a separate discussion if the requirement of 10% over 100ms is per UE or per cellProposal 5: When independent per-beam LBT sensing is performed as initiators nodes at  gNB or UE, each time the gNB or UE attempts to acquire a COT

- Apply independent Type 1 channel access to each beam

- the gNB and UE   shall re-initialize the counter for each beam

- the initial value of the counter is independently determined for each beam

- count-down process is independent for each beam

- Start of the channel occupancy time in all beam is aligned.

- To acquire a new COT, the applied Type 1 channel access process to each beam shall not start before the end of the previous COT

Proposal 6:  When independent per-beam LBT sensing is performed at UE, support both the following modes

- Mode A: The UE occupies the COT on the subset of beams where the LBT procedure is successful

- Mode B: The UE occupies the COT only if the LBT procedure is successful on all beams

Proposal 7:  For a COT with MU-MIMO (SDM) transmission or TDM transmission of beams with beam switching, when independent per-beam LBT is performed at the start of the COT, for Pout in EDT determination of LBT for each sensing beam define Pout as the maximum EIRP of the intended transmissions “covered” by the sensing beam by the node determining EDT during a COT.

Proposal 8:  For an UL transmission indicated or configured to use Type 1 channel access, if the UE later finds out, through DCI 2_0 detection, the transmission falls is in a gNB COT, the UE can change the channel access type to either Type 2 channel access or Type 3 channel access, based on the following rule.

- If Type 2 Channel Access is configured as one of the entries in the configuration for DCI 0_1 (or DCI 1_1), the UE will use Type 2 channel access.

- If Type 2 Channel Access is not configured as one of the entries in the configuration for DCI 0_1, the UE will use Type 3 channel access

Proposal 9 For a UE initiated COT, the channel access type for resuming a COT after a gap of Y us from previous transmission will be as follows:

- If Type 2 Channel Access is configured as one of the entries in the configuration for DCI 0_1 (or DCI 1_1),  the UE uses Type 2 Channel access

- If Type 2 Channel Access is not configured as one of the entries in the configuration of DCI 0_1 (or DCI 1_1) the UE uses Type 3 channel access.

# References

- R1-2202685 Email discussion summary#4 for channel access of 52.6GHz to 71GHz band [108-e-NR-52-71GHz]	Moderator (Qualcomm)

- Chairperson’s notes: RAN1-108-e, Agenda 8.2.6

- Chairperson’s notes: RAN1-107bis-e, Agenda 8.2.6

- Chairperson’s notes: RAN1-107-e, Agenda 8.2.6

- Chairperson’s notes: RAN1-106-bis-e, Agenda 8.2.6

- Chairperson’s notes: RAN1-106-e, Agenda 8.2.6

- Chairperson’s notes: RAN1-105-e, Agenda 8.2.6

- 3GPP TS 38.212, Rel 16, Multiplexing and Channel Coding,  v 16.3.0