3GPP TSG RAN WG1 #109-e			    R1- 2204983

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.3.2

Source: 	Qualcomm Incorporated

Title: 	Remaining issues on intra-UE multiplexing and prioritization for IOT and URLLC

Document for:	Discussion/Decision

# Introduction

In RAN plenary #86, the work item on Enhanced Industrial Internet of Things (IIOT) and URLLC Support was agreed [1]. One of the main objectives of the work item is to study

“Intra-UE multiplexing and prioritization of traffic with different priority based on work done in Rel.16 [RAN1]:

- Specify multiplexing behavior among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH. “

In RAN1 108e, the above objective is achieved with completion of the design for the objective, except for a few minor open issues that are addressed in the reminder of this contribution.

# Remaining issues on intra-UE multiplexing

## PRI selection for HP A/N and LP A/N multiplexing

For the scenario of HP HARQ-ACK and LP HARQ-Ack multiplexing, RAN1 already agreed the following procedure to determine PUCCH resource to transmit multiplexed payload.

- Step 1: Use the HP PUCCH resource pool for multiplexed payload

- Step 2: Determine the PUCCH resource set based on sum number of bits of HP HARQ-ACK and LP HARQ-ACK

- Step 3: Pick a PUCCH resource in the HP resource pool

- Step 4: Determine number of RBs based on coding rates and number of information bits for HP HARQ-ACK and LP HARQ-ACK (agreements in RAN1 #107e).

In the above 4-step procedure, the detail of step 3 is still open. The open issue is that, whether UE should pick a PUCCH resource based on PRI in last HP DCI, or last LP DCI, or last DCI regardless of HP and LP? Given that the final PUCCH resource is selected from HP PUCCH resource pool, it makes more sense to follow the PRI in the last HP DCI.

In the current Rel-17 spec, based on the highlighted text in subclause 9.2.5.3 of TS 38.213, the PRI in the last HP DCI is used. However, RAN1 had no explicit agreement on this. Therefore, it is better to confirm the current spec with an agreement.

-----------------------------------------------------------TS 38.213 Subclause 9.2.5.3------------------------------------------------------

If a UE

-	is provided PUCCH-ConfigurationList for PUCCH transmissions with priority 0 and 1,

-	is provided UCI-MuxWithDifferentPriority, and

-	would transmit overlapping PUCCHs that include a first PUCCH with OACK,0 HARQ-ACK information bits of priority 0 and a second PUCCH with OACK,1 HARQ-ACK information bits of priority 1

-	if the PUCCH resource for the second PUCCH includes PUCCH format 2, 3, or 4 and additionally includes OSR,1 SR bits of priority 1, OACK,1 is replaced by OACK,1+OSR,1 where OSR,1 is determined according to clause 9.2.5.1

the UE

-	determines

-	a PUCCH resource set from the second PUCCH-Config using OUCI=OACK,0+OACK,1 as described in clause 9.2.1, and a PUCCH resource from the PUCCH resource set as described in clause 9.2.3 where a DCI format, if any, triggers PUCCH transmission of priority 1, or

-	a PUCCH resource from the second sps-PUCCH-AN-List using OUCI=OACK,0+OACK,1 as described in clause 9.2.1, and

-	multiplexes the OACK,0 and OACK,1 HARQ-ACK information bits in a same PUCCH using the PUCCH resource.

---------------------------------------------------------end of TS 38.213 Subclause 9.2.5.3-------------------------------------------------

Based on the above motivation to confirm the current spec, the following is proposed.

Proposal 1: for multiplexing HP HARQ-ACKs and LP HARQ-ACKs, it is confirmed that current specification is interpreted as the following.

- The PRI in the lastly received DCI (if exist), which schedules a HP HARQ-ACK involved in the multiplexing, is used to select the PUCCH resource to transmit the multiplexed UCI payload.

- Note: no spec update is needed

## Interaction between UL-CI and intra-UE multiplexing

In RAN1 108bis-e, it was agreed UL transmission cancellation due to dynamic SFI, semi-static DL symbol and SSB are handled after intra-UE multiplexing, based on the following agreement.

Agreement

For Rel-17 intra-UE multiplexing, PUCCH and PUSCH cancellation due to dynamic SFI, semi-static DL symbols and SSB symbols are performed after step 2 of Rel-17 intra-UE multiplexing.

Following the principle of above agreement, the interaction between UL-CI and intra-UE multiplexing should also be handled after step 2 of Rel-17 intra-UE multiplexing. UL-CI then should be applied to UL channels after step 2. For UL-CI, one should notice that uplinkCancellationPriority can configure UE to cancel UL PUSCH regardless of HP/LP or to only cancel UL PUSCH of LP. In case uplinkCancellationPriority is configured, A LP PUSCH has HP UCI multiplexing on it should be exempt from UL-CI.

Therefore, we have the following proposal.

Proposal 2: For Rel-17 intra-UE multiplexing, PUSCH cancellation due to UL-CI are performed after step 2 of Rel-17 intra-UE multiplexing.

- A LP PUSCH has HP UCI multiplexing on it is exempt from UL-CI, if RRC parameter uplinkCancellationPriority is configured.

## CG-UCI multiplexing on CG-PUSCH

In RAN1 108-e, regarding how CG-UCI should be multiplexing on CG-PUSCH, together with HARQ-ACK and CSI, it was a common understanding that current spec/previous agreements are not clear, and a clarification is needed. Therefore, we address this issue in this section.

For Rel-17 UCI multiplexing on PUSCH, RAN1 common understanding is that the three encoding/rate matching/RE mapping chains in Rel-15 for HARQ-ACK, CSI part 1, and CSI part 2, should be reused. Therefore, this principle should be kept for CG-UCI, HARQ-ACK, and CSI multiplexing on CG-PUSCH as well. In Rel-16, with same priority, CG-UCI is jointly encoded with HARQ-ACK. The same procedure can be reused in Rel-17, with more general scenario where other UCI might have different priorities.

In details, we can have the following 6 scenarios for CG-UCI multiplexing with CG-PUSCH, together with HARQ-ACK and CSI.

- For scenario A1 and A2, it is very simple as there is only one priority HARQ-ACK and the priority of HARQ-ACK and CG-UCI is the same. A UE can just jointly encode the CG-UCI with HARQ-ACK of the same priority and multiplex on the CG PUSCH.

- For scenario B1 and B2, there are two HARQ-ACKs of two priorities. Again, the CG-UCI can jointly encode with the HARQ-ACK of the same priority and multiplex on CG-PUSCH. The LP CSI part 2 is dropped in scenario B1 and B2.

- For scenario C1 and C2, there is only one priority HARQ-ACK and the priority of HARQ-ACK and CG-UCI is different. In this case, the CG-UCI should be treated as if it is HARQ-ACK. Then the same rule of HP HARQ-ACK, LP HARQ-ACK, and CSI multiplexing on PUSCH should be reused.

Based on the above analysis, the following proposal is made.

Proposal 3: In Rel-17 intra-UE multiplexing, treat CG-UCI of a certain priority as if a HARQ-ACK of the same priority, joint encode CG-UCI with HARQ-ACK of the same priority (if exist).

## Other FFS on details of different priorities PUCCH multiplexing

In our understanding of Rel-17 spec, most scenario for different priorities PUCCH multiplexing have been settled, except for three scenarios, as summarized in the following table.

| Overlapping different priorities PUCCHs |  | UE behavior |
| --- | --- | --- |
| HP PUCCH | LP CSI/SR | Drop LP CSI/SR |
| HP A/N | LP A/N | Mux LP A/N with HP A/N on a HP PUCCH resource |
| HP A/N | LP A/N + CSI/SR | Drop LP CSI/SR, mux LP A/N with HP A/N |
| HP SR in PF 0/1 | LP A/N in PF 0/1/2/3/4 | Drop LP A/N |
| HP SR in PF 0/1 | LP A/N+CSI/SR in PF2/3/4 | Drop LP A/N+CSI/SR |
| HP SR+A/N in PF 2/3/4 | LP A/N | Treat HP SR as HP A/N and mux with LP A/N |
| HP SR in PF 0/1 | LP A/N+ SR in PF0/1 | FFS |
| HP SR+A/N in PF 0/1 | LP A/N | FFS |
| HP SR+A/N in PF 0/1 | LP A/N+SR | FFS |

Since it is already maintenance phase for Rel-17, the FFS should be resolved following preceding similar scenarios with minimum spec impact.

For the scenario of HP SR in PUCCH format 0/1 overlapping with LP HARQ-ACK and SR in PUCCH format 0/1, we can simply drop the HARQ-ACK and SR in PUCCH format 0/1, follow the similar UE behaviour as HP SR in PUCCH format 0/1 overlapping with LP HARQ-ACK in PUCCH format 0/1/2/3/4.

Proposal 4: When a HP positive SR in PUCCH format 0/1 overlaps with LP HARQ-ACK and SR in PUCCH format 0/1, LP HARQ-ACK and SR are dropped.

For the scenario of HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, there was a heated debate in RAN1 108e without conclusion. But option 2 was supported by majority. To close this issue, it is reasonable to follow the majority view to adopt option 2.

Proposal 5: When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, option 2 is adopted.

- Option 2: 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1. The HP UCI is then multiplexed with LP HARQ-ACK

  - A UE expect that the resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is in PF2, PF3, or PF4.

Similarly, for the scenario of HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK and SR, the following is proposed.

Proposal 6: When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK and SR.

- 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1. The HP UCI is then multiplexed with LP HARQ-ACK with the LP SR dropped.

  - A UE expect that the resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is in PF2, PF3, or PF4.

# Conclusions

In summary, we have the following proposals for intra-UE multiplexing and prioritization for Rel-17 IOT and URLLC.

Proposal 1: for multiplexing HP HARQ-ACKs and LP HARQ-ACKs, it is confirmed that current specification is interpreted as the following.

- The PRI in the lastly received DCI (if exist), which schedules a HP HARQ-ACK involved in the multiplexing, is used to select the PUCCH resource to transmit the multiplexed UCI payload.

- Note: no spec update is needed

Proposal 2: For Rel-17 intra-UE multiplexing, PUSCH cancellation due to UL-CI are performed after step 2 of Rel-17 intra-UE multiplexing.

- A LP PUSCH has HP UCI multiplexing on it is exempt from UL-CI, if RRC parameter uplinkCancellationPriority is configured.

Proposal 3: In Rel-17 intra-UE multiplexing, treat CG-UCI of a certain priority as if a HARQ-ACK of the same priority, joint encode CG-UCI with HARQ-ACK of the same priority (if exist).

Proposal 4: When a HP positive SR in PUCCH format 0/1 overlaps with LP HARQ-ACK and SR in PUCCH format 0/1, LP HARQ-ACK and SR are dropped.

Proposal 5: When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, option 2 is adopted.

- Option 2: 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1. The HP UCI is then multiplexed with LP HARQ-ACK

  - A UE expect that the resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is in PF2, PF3, or PF4.

Proposal 6: When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK and SR.

- 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1. The HP UCI is then multiplexed with LP HARQ-ACK with the LP SR dropped.

  - A UE expect that the resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is in PF2, PF3, or PF4.

# References

- 3GPP TSG RAN Meeting #86, RP-193233, “3GPP Work Item Description; Enhanced Industrial Internet of Things (IIOT) and URLLC Support”, Nokia, Nokia Shanghai Bell, Sitges, Spain, December 9-12, 2019.

- 3GPP TSG SA, 3GPP TR 22.804, V16.3.0, “Technical Specification Group Services and Systems Aspects; Study on Communication for Automation in Vertical Domains (Release 16)”, July 2020.

- 3GPP TSA RAN1 Meeting #104, R1-2101459, “HARQ-ACK enhancement for IOT and URLLC,” e-Meeting, Jan. 25-Feb. 5, 2021