3GPP TSG RAN WG1 #109-e	R1-2204988

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.7.1

Source: 	Qualcomm Incorporated

Title: 	Maintenance Issues on Idle and Inactive Mode UE Power Saving

Document for:	Discussion/Decision

# Introduction

In this paper, we will discuss the maintenance issues of NR Rel-17 idle and inactive mode UE power saving techniques.

# TRS Availability Indication

It was agreed in the RAN1 #107-bis-e meeting that when SIB configures idle and inactive TRS, TRS availability indication field is present in the PEI PDCCH if configured and in the paging PDCCH [1]. Monitoring of the paging PDCCH is a mandatory feature supported by legacy UEs before Rel-17. The short message indicator in paging PDCCH indicates whether there is a paging information, short message or both for the UE to receive. It is possible that network may set the short message indicator field to ‘00’ for additional use cases such as the Rel-17 TRS availability indication.

Table 7.3.1.2.1-1: Short Message indicator [TS 38.212]

| Bit field | Short Message indicator |
| --- | --- |
| 00 | Reserved |
| 01 | Only scheduling information for Paging is present in the DCI |
| 10 | Only short message is present in the DCI |
| 11 | Both scheduling information for Paging and short message are present in the DCI |

In theory, legacy UEs should be able to handle the TRS availability indication in a paging PDCCH with short message indicator equal to ‘00’. In practice, excessive transmissions of paging PDCCH with short message indicator equal to ‘00’ may have an impact to legacy UE’s operation. For example, it is possible that the UE implementation prunes the decoded PDCCH payload in software after the decoded PDCCH candidate passes CRC in hardware. For a legacy UE, if short message indicator is equal to ‘00’, discarding such a PDCCH candidate results in a non-negligible operating cost due to the additional communication between hardware and software.

Observation 1: Excessive transmissions of paging PDCCH with short message indicator equal to ‘00’ causes higher operating cost of a legacy UE.

In the meanwhile, according to the RAN1 #107-bis-e agreement, when a TRS availability indication bit is set to ‘0’, the bit does not carry information about the presence of the associated TRS resources [1]. Then, if short message indicator is equal to ‘00’ and the TRS availability indication field contains only 0 bits, the paging PDCCH does not carry any information for either legacy operation (i.e., paging, short message) or for TRS availability indication. Then network should not transmit such a paging PDCCH without carrying any information to avoid unnecessary UE operating cost. Based on this, we make the following proposal for TRS availability carried by paging PDCCH.

Proposal 1: If network configures the Rel-17 TRS availability indicator in paging PDCCH, UE does not expect TRS availability indication field contain all 0 bits in the paging PDCCH if the short message indicator is ‘00’.

Agreement UE can receive L1 based signaling for TRS availability indication before the expiration/end of validity duration associated with previous L1 based signaling for TRS availability indication - For each bit indicated as ‘1’ in the availability indication field of the current L1 based signaling TRS availability indication, the UE assumes the corresponding TRS resource set(s) are available from the reference point until the end of the validity duration associated with the current L1 based signaling indication. - For each bit indicated as ‘0’ in the availability indication field of the current L1 based signaling, the UE keeps the existing current assumption on the availability or unavailability of the corresponding TRS resource set(s). Note: the validity duration for different group of TRS resources sets correspond to different bits in the availability indication field can be different and are maintained independently. Note: CATT has concern on the power saving gain based on this agreement.

# Conclusions

In this paper, we made the following observations and proposals for remaining issues on idle and inactive TRS design.

Observation 1: Excessive transmissions of paging PDCCH with short message indicator equal to ‘00’ causes higher operating cost of a legacy UE.

Proposal 1: If network configures the Rel-17 TRS availability indicator in paging PDCCH, UE does not expect TRS availability indication field contain all 0 bits in the paging PDCCH if the short message indicator is ‘00’.

# References

- RAN1 #107-bis-e Chairman’s notes