3GPP TSG-RAN WG1 #109-e	R1-2204989

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.7.2

Source: 	Qualcomm Incorporated

Title: 	DCI-based power saving adaptation during DRX Active Time

Document for:	Discussion/Decision

# Introduction

Throughout Rel-17 RAN1 meetings up to RAN1 #107-e, the baseline design of DCI-based PDCCH monitoring adaptation has been completed. Basically, by scheduling DCI formats, two types of PDCCH monitoring adaptation schemes can be indicated:

- Search space set group switching,

- PDCCH skipping for a certain duration.

In this contribution, we present our views on the remaining issues of the PDCCH monitoring adaptation.

# Discussion

## SR and RACH procedure during PDCCH skip duration

In RAN2 #116bis-e, RAN2 made the following agreements:

- UE ignores PDCCH skipping while the SR is pending. - If PDCCH skipping is applied to RNTI(s) monitored during RAR/MsgB window, the UE ignores PDCCH skipping during the RAR/MsgB window. - UE ignores PDCCH skipping while contention resolution timer is running.

In relation to the agreements, RAN2 sent an LS [1] to RAN1 asking RAN1’s understanding, and RAN1 sent another LS [2] in response:

RAN2 LS R2-2201960: Question 1: RAN2 would like to know whether PDCCH skipping is applied to RNTI(s) monitored during RAR/MsgB window for RAR/MsgB reception. RAN1 LS R1-2202905: Answer 1: Yes, PDCCH skipping can be applied to C-RNTI(s) and MCS-C-RNTI(s) in at least Type3 PDCCH CSS and USS monitored during RAR/MsgB window for RAR/MsgB reception based on RAN1 agreements till RAN1#107bis-e. RAN2 LS R2-2201960: Question 2: RAN2 would like to know if RAN1 prefers to capture the above agreements in RAN1 specification or prefer these to be captured in MAC specification. RAN1 LS R1-2202905: Answer 2: RAN1 would discuss and conclude how to capture the above RAN2 agreements in RAN1 specification. - It is RAN1 understanding PDCCH skipping is not applied to perform PDCCH monitoring during RAR window/MsgB window/contention resolution timer or when SR is pending.

Although it is RAN1’s understanding that PDCCH skipping does not affect PDCCH monitoring during a RAR/MsgB window, while a contention resolution timer is running, or when a SR is pending, it still needs further discussion how to capture that in RAN1’s specification. RAN2’s agreements just state that UE ‘ignores’ PDCCH skipping and defining the technical details of the UE behavior is up to RAN1.

When a SR is pending at the UE during a PDCCH skip duration, the network can realize it only after receiving a SR transmission from the UE. Thus, it would be reasonable for the UE to start ignoring PDCCH skipping only after the transmission of SR. Also, even after the UE receives an UL grant for BSR and the pending SR is cancelled, the UE may further expect to receive additional UL grants for remaining data in the buffer. In order not to delay the remaining UL data transmission, it would be desirable for the UE to ignore PDCCH skipping until the buffer is flushed out, not just while the SR is pending. This can be achieved by cancelling PDCCH skipping after the UE transmits a positive SR. Also, if the UE transmits a BSR without sending SR, e.g., by piggybacking a BSR MAC CE on another (DG or CG) PUSCH transmission, it would also be desirable for the UE to cancel PDCCH skipping.

Proposal 1: If a UE transmits a positive SR or a BSR during a PDCCH skip duration, the UE cancels PDCCH skipping.

A RACH procedure involves multiple stages of PDCCH monitoring, which follow the transmission of a RA preamble. For example, when the RACH procedure is triggered by a beam failure recovery, the UE may expect to receive multiple DL grants for resetting control and data channel beams, outside the RAR window. Thus, ignoring PDCCH skipping just for the RAR/MsgB window and during the contention resolution would not be desirable, because it could delay the completion of the BFR procedure. As in the discussion on the SR procedure, it would be fair to let the UE to cancel PDCCH skipping after transmitting the RA preamble.

Proposal 2: If a UE transmits a RA preamble for a CFRA or CBRA procedure during a PDCCH skip duration, the UE cancels PDCCH skipping.

## Monitoring of CSS and DCP during PDCCH skip duration

Throughout Rel-17 discussion, it is commonly understood that monitoring of broadcast PDCCHs (i.e., DCI format 0_0 or 1_0 with CRC scrambled by SI-RNTI, RA-RNTI, MsgB-RNTI, or P-RNTI) should not be affected by PDCCH monitoring adaptation. For example, for both in Rel-16 and Rel-17 (according to the latest CR for Section 10.4 of TS 38.213 [3]), SSSG switching is applied only for Type3 CSS or USS, not for Type0/0A/1/2 CSS.

For PDCCH skipping, another important issue is whether to allow monitoring of unicast PDCCHs (i.e., DCI format 0_0 or 1_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI) piggybacked in Type0/0A/1/2 CSSs during a skip duration. Based on the discussion in RAN1 #108-e, the following proposal has been captured in FL’s summary:

proposal 2-1 (v4) UE also skips monitoring of PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI in any CSS and USS when UE is indicated skipping PDCCH monitoring for a duration. Note: it is previously agreed that PDCCH based monitoring adaptation is applied to USS and type-3 CSS.

When the network indicates PDCCH skipping to the UE, it should guarantee that there will not be any unicast data scheduled during the indicated skip duration. Thus, it would be sensible for the UE not to expect any unicast PDCCHs during the skip duration.

Proposal 3: During an indicated PDCCH skip duration, a UE does not monitor any PDCCH candidates for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI, in any CSS and USS.

Fundamentally, this issue is about how the PDCCH skipping function is viewed and specified in the standard – there would be two different views on PDCCH skipping:

- Alt 1: During the skip duration, monitoring of PDCCHs in certain types of SS sets (i.e., Type3 CSS and USS) is restricted.

- Alt 2: During the skip duration, monitoring of PDCCHs with certain types of RNTIs in any SS sets is restricted.

With Alt 1, the UE will refrain from monitoring PDCCHs in Type3 CSS and USS, while is still monitoring Type0/0A/1/2 CSSs. However, with Alt 1, we may need to define additional UE behaviors for some exceptional cases. For instance, as discussed above, skipping of monitoring unicast PDCCHs (C-RNTI, MCS-C-RNTI, or CS-RNTI) piggybacked in Type0/0A/1/2 CSSs should be regarded as an exception for the legacy Type0/0A/1/2 CSS monitoring behavior. Another exception may be needed for monitoring DCP (DCI format 2_6), for which, a relevant discussion has been made in RAN1 #108-e:

RAN2 LS R2-2201960: Question 3: RAN2 would like to ask RAN1 whether a) Physical layer of UE reports a value of 1 for Wake-up indication bit to higher layer or b) Physical layer of UE does not report Wake-up indication bit to higher layer, in case UE cannot monitor DCP due to PDCCH skipping; or if Approach 3 can be assumed. RAN1 LS R1-2202905: Answer 3: It is RAN1 understanding that for DRX operation, - As DCP is monitored only outside active time, it cannot be missed due to PDCCH skipping as skipping applies only in active time. Note: RAN1 is discussing whether PDCCH skipping duration can apply to outside active time.

According to Alt 1, as DCI format 2_6 is transmitted in Type3 CSS, the UE will not monitor DCI format 2_6 during the skip duration. However, RAN1 has already agreed that DCP monitoring is not affected by PDCCH skipping and, hence, it should be regarded as an exception for skipping Type3 CSS monitoring. Regarding how to except DCP monitoring from PDCCH skipping, the following proposal has been captured in FL’s summary in RAN1 #108-e:

Proposal 7-3a (v1): - The PDCCH skipping applies only in active time - Down-select from the following options: - Option 1a: the PDCCH skipping duration is considered irrespective the slot(s) are in active time or not, - Option 1b: PDCCH skipping is terminated when UE enters outside active time

Since Rel-16 DCP monitoring and Rel-17 PDCCH skipping are independent power saving features, any interaction between them would be avoided to simplify the joint operation. Hence, between the two options, Option 1a is preferred.

Proposal 4: PDCCH skipping duration is counted irrespective of whether the slots are in DRX Active Time or not.

With Alt 2, the PDCCH skipping behavior can be associated with certain types of PDCCH candidates, regardless of which SS set the PDCCH candidates would be transmitted in. Unlike Alt 1, this may help avoid all the hassles of defining exceptional UE behaviors separately for each type of SS sets. For example, according to Alt 2, UE may only skip monitoring unicast PDCCHs (C-RNTI, MCS-C-RNTI, or CS-RNTI) in any SS sets, and no exceptional behavior is needed depending on the type of SS sets (CSS or USS) that the PDCCHs would be transmitted in. Furthermore, with Alt 2, monitoring of DCP (PS-RNTI) can be excluded from PDCCH skipping, which implements the agreement that DCP monitoring is not affected by PDCCH skipping. If we come to the question about which RNTIs should be impacted by PDCCH skipping according to Alt 2, we can refer to the existing standard (TS 38.321):

## 5.7 Discontinuous Reception (DRX) The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CI-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, and AI-RNTI. …

Proposal 5: During an indicated PDCCH skip duration, PDCCH monitoring activity only for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, CI-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, or AI-RNTI in any CSS and USS is impacted.

## Application delay and HARQ handling

In RAN1 #108-e, the following has been agreed:

Agreement - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH skipping (i.e., Beh 1A), select one of the following schemes - Alt 1a: the UE applies Beh 1A on an active BWP of the serving cell at the first slot after the last OFDM symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - FFS: whether the UE will monitor PDCCH when DRX Retransmission timer is running

As captured in the agreement, handling any potential retransmissions during the skip duration still needs further discussion. The potential retransmissions can either be for any PDSCH/PUSCH whose grants have been received before starting the skip duration, or for any SPS PDSCH/CG PUSCH received/transmitted during the skip duration. Based on the previous agreement, the skip duration can be configured as long as 100 milliseconds. Thus, if the UE cannot receive retransmission grants during the skip duration, it may result in a significant latency performance loss.

As a simple solution, discontinuous PDCCH monitoring by RTT and Retransmission timers may be allowed during the skip duration. Note that, during the discussion of Rel-17 UE power saving WI scope, it was commonly understood that DRX, which is the legacy PDCCH monitoring adaptation, should be the baseline. Any Rel-17 connected mode power saving schemes should not be regarded as an alternative of the DRX, but should provide additional gain on top of the DRX. Since the DRX configuration is prerequisite, the RTT and Retransmission timers are always available as parts of the DRX configuration. Thus, for HARQ handling during the skip duration, the RTT and Retransmission timers of the DRX can be reused, without introducing separate timer entities just for PDCCH skipping.

Proposal 6: During a PDCCH skip duration, the UE will discontinuously monitor PDCCH according to drx-HARQ-RTT-TimerDL/UL and drx-RetransmissionTimerDL/UL.

# Conclusions

Proposal 1: If a UE transmits a positive SR or a BSR during a PDCCH skip duration, the UE cancels PDCCH skipping.

Proposal 2: If a UE transmits a RA preamble for a CFRA or CBRA procedure during a PDCCH skip duration, the UE cancels PDCCH skipping.

Proposal 3: During an indicated PDCCH skip duration, a UE does not monitor any PDCCH candidates for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI, in any CSS and USS.

Proposal 4: PDCCH skipping duration is counted irrespective of whether the slots are in DRX Active Time or not.

Proposal 5: During an indicated PDCCH skip duration, PDCCH monitoring activity only for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, CI-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, or AI-RNTI in any CSS and USS is impacted.

Proposal 6: During a PDCCH skip duration, the UE will discontinuously monitor PDCCH according to drx-HARQ-RTT-TimerDL/UL and drx-RetransmissionTimerDL/UL.

# References

- R2-2201960, “LS on PDCCH Skipping in RRC_CONNECTED”, RAN2 #116bis-e.

- R1-2202905, “LS reply on PDCCH skipping”, RAN1 #108-e.

- R1-2112936, “Introduction of UE power savings enhancement in NR”, RAN1 #107-e, Samsung.