| 3GPP TSG-RAN WG1 Meeting #109-e e-Meeting, May 9th – May 20th, 2022 | R1-2204990 |
| --- | --- |

Agenda item:	8.8.1

Source: 	Qualcomm Incorporated

Title: 	PUSCH Enhancements

Document for:	Discussion/Decision

# Open Issues

In this document we discuss a few remaining issues related to PUSCH enhancements for coverage enhancement. We focus on available slot counting for Msg3 transmissions and available slot counting for reduced capability half-duplex UEs.

# Applicability of Available Slot Counting for Reduced Capability Half-Duplex UEs

In the last round of revision to the 38.214 specification, there was no consensus on how to capture the condition that available slot counting in only applicable when K > 1 for reduced capability half-duplex UEs. In the following proposal we provide a text proposal to capture this constraint in a precise manner.

Proposal 1: For reduced capability half-duplex UEs, restrict applicability of available slot counting to K > 1. Adopt the following text proposal:

6.1.2.1 Resource allocation in time domain <omitted text> For the case of a reduced capability half-duplex UE, and when AvailableSlotCounting is enabled and K > 1, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

# Applicability of available slot counting to Msg3 transmission

As per the current spec, it appears that available slot-based counting is used to determine the slot for Msg3 even when K=1. This can lead to a scenario where a gNB does not have full knowledge of UE behavior since it does not know if it’s a Release 17 UE that can support Msg3 repetitions. Its leads to ambiguity on whether a Msg3 transmission that overlaps with downlink symbols will be dropped or not. This ambiguity poses additional complexity for the gNB and is best avoided. We make the following proposal:

Proposal 3: For Msg3 transmission and retransmission, restrict the use of available slot-based counting only to the case when number of repetitions are greater than 1. We suggest the following text proposal:

6.1.2.1 Resource allocation in time domain <omitted text> - If K > 1, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, based on tdd-UL-DL-ConfigurationCommon and ssb-PositionsInBurst, and the TDRA information field value in the RAR UL grant. - A slot is not counted in the number of N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by RAR UL grant, if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - If K > 1, The UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, based on tdd-UL-DL-ConfigurationCommon and ssb-PositionsInBurst and the TDRA information field value in the DCI scheduling the PUSCH. - A slot is not counted in the number of N∙K slots for a PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_0 scrambled by TC-RNTI, if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.<omitted text>

# Conclusions

Based on the discussions presented in the earlier sections, we have the following proposals:

Proposal 1: For reduced capability half-duplex UEs, restrict applicability of available slot counting to K > 1. Adopt the text proposal in Section 2.

Proposal 2: For Msg3 transmission and retransmission, restrict the use of available slot-based counting only to the case when number of repetitions are greater than 1. Adopt the text proposal presented in Section 3.

# References

- Chairman notes, 3GPP TSG RAN1 meeting #107-e, e-Meeting, November 11th – 19th, 2021.

- Chairman notes, 3GPP TSG RAN1 meeting #107bis-e, e-Meeting, January 17th – 25th, 2022.

- Chairman notes, 3GPP TSG RAN1 meeting #108-e, e-Meeting, February 21st – March 3rd, 2022.