| 3GPP TSG-RAN WG1 Meeting #109-e e-Meeting, May 9th – May 20th, 2022 | R1-2204991 |
| --- | --- |

Agenda item:	8.8.2

Source: 	Qualcomm Incorporated

Title: 	Joint channel estimation for PUSCH and PUCCH

Document for:	Discussion/Decision

# Introduction

In this document we discuss some of the remaining issues on DMRS bundling for PUSCH transmissions. We focus on TPC update handling and inter-slot frequency hopping with DMRS bundling.

# Determination of Inter-slot Frequency Hopping When DMRS Bundling is Enabled

For PUSCH, although it was agreed that inter-slot frequency hopping would be based on physical slot indices when DMRS bundling is enabled, this is not currently captured in the 38.214 specification since there was no agreement on whether the hop determination should be dependent only on slot index or also depend on frame index. To better align with legacy implementations of inter-slot frequency hopping, we believe it suffices to make this dependent only on slot index and that there is no need to introduce frame index. We therefore make the following proposal:

Proposal: For PUSCH, when DMRS bundling and inter-slot frequency hopping are enabled, the RB allocations are determined only by slot index. We suggest the following text proposal:

6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots <omitted text> In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is not enabled, the starting RB during slot is given by: , where is the current slot number within a system radio frame, where a multi-slot PUSCH transmission can take place, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) and is the frequency offset in RBs between the two frequency hops. In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is enabled, the starting RB during slot is given by: [ RBstartns,fμ=RBstart RBstart+RBoffsetmodNBWPsize ns,fμ+nf∙Nslotframe,μNFH mod 2=0ns,fμ+nf∙Nslotframe,μNFH mod 2=1 ] where ns,fμ is the current slot number within a system radio frame, nf is the number of the system radio frame containing the current slot, Nslotframe,μ is the number of slots per frame for subcarrier spacing configuration μof the UL BWP that the PUSCH is transmitted on, NFH is the value of the higher layer parameter PUSCH-Frequencyhopping-Interval, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) andis the frequency offset in RBs between the two frequency hops. In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is enabled, the starting RB during slot is given by: RBstartnsμ=RBstart RBstart+RBoffsetmodNBWPsize nsμNFH mod 2=0nsμNFH mod 2=1 where is the current slot number within a system radio frame, NFH is the value of the higher layer parameter PUSCH-Frequencyhopping-Interval, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) andis the frequency offset in RBs between the two frequency hops. <omitted text>

# TPC Update Handling

In RAN1 107-e, on the issue of interaction between TPC updates and DMRS bundling, the following working assumption was agreed:

Working assumption:

- The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity.

  - If UE is configured to accumulate TPC commands,

    - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW.

  - If UE is not configured to accumulate TPC commands

    - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW.

      - FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

In subsequent meetings, there was no consensus on how to capture this in the relevant section of the specification. Rather than redefining the notion of a transmission occasion, a simpler more direct means of capturing the above agreement would be preferred. We believe the most straightforward means to accomplish this would be redefine the sets Dj that accumulate the TPC commands. We make the following proposal in this regard:

Proposal 2: Capture the agreement on deferring TPC updates to the end of nominal TDWs using the following text proposal in 38.213, Section 7.1.1:

7.1 Physical uplink shared channel <omitted text> 7.1.1 UE behaviour <omitted text> If the UE is provided PUSCH-DMRS-Bundling = ‘enabled’, and transmission occasion i is not the last transmission occasion within a nominal time domain window, then any TPC command values received via DCI format 2_2 contained in the set Di are deleted and added to the set Dj where j is a transmission occasion occurring after the end of the nominal time domain window and j>0 is the smallest integer for which KPUSCH(i) symbols before PUSCH transmission occasion i is earlier than KPUSCH(j) symbols before PUSCH transmission occasion j. <omitted text>

# Conclusion

This document discusses two remaining issues for DMRS bundling across PUSCH and PUCCH repetitions. We make the following proposals:

Proposal: For PUSCH, when DMRS bundling and inter-slot frequency hopping are enabled, the RB allocations are determined only by slot index. Adopt the text proposal in Section 2.

Proposal 2: Capture the agreement on deferring TPC updates to the end of nominal TDWs using the following text proposal in 38.213, Section 7.1.1:

“If the UE is provided PUSCH-DMRS-Bundling = ‘enabled’, and transmission occasion i is not the last transmission occasion within a nominal time domain window, then any TPC command values received via DCI format 2_2 contained in the set Di are deleted and added to the set Dj where j is a transmission occasion occurring after the end of the nominal time domain window and j>0 is the smallest integer for which KPUSCH(i) symbols before PUSCH transmission occasion i is earlier than KPUSCH(j) symbols before PUSCH transmission occasion j.”

# Reference

- Chairman notes, 3GPP TSG RAN1 meeting #107-e, e-Meeting, November 11th – 19th, 2021.

- Chairman notes, 3GPP TSG RAN1 meeting #107bis-e, e-Meeting, January 17th – 25th, 2022.

- Chairman notes, 3GPP TSG RAN1 meeting #108-e, e-Meeting, February 21st – March 3rd, 2022.