3GPP TSG RAN WG1 #109-e			R1-2204994

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.12.1

Source:	Qualcomm Incorporated

Title:	Mechanisms to improve reliability for RRC_CONNECTED UEs

Document for:	Discussion and Decision

# Introduction

In this document, we discuss the remaining details of UE feedback to improve reliability for Multicast RRC_CONNECTED UEs.

# Discussion

## Enabling/disabling of HARQ-ACK feedback for	multicast

In last RAN1#108-e meeting, we have the following conclusion and agreements related with enabling/disabling of HARQ-ACK feedback for multicast.

Conclusion

When HARQ feedback is disabled, the following fields (if present) of DCI format 4_1/4_2 can be assumed to be reserved and UE ignores them:

- PUCCH resource Indicator (PRI)

- PDSCH-to-HARQ_feedback timing indicator (k1)

Agreement

For Type-2 codebook generation, UE reports HARQ-ACK bits only for TBs with enabled HARQ-ACK by RRC or DCI.

Agreement

For Type-1 codebook generation,

- if all configured G-RNTIs are with HARQ-ACK disabled by RRC signalling, UE does not report any HARQ-ACK information in the PUCCH slot;

- For other cases, FFS between the 3 alternatives below:

  - Alt1: if at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt2: if at least one configured G-RNTI is with HARQ-ACK enabled, UE reports actual HARQ-ACK result for all G-RNTIs.

  - Alt3: UE is not expected to be configured with some G-RNTI with HARQ-ACK disabled by RRC signalling and some other G-RNTI with HARQ-ACK enabled by RRC signalling for all configured G-RNTI

  - Other alternatives are not precluded

Agreement

If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling, for Type-1 codebook generation,

- FFS between the 4 alternatives below:

  - Alt1: UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt2: UE will report ACK/NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt3: UE is not expected to be configured with G-RNTI with HARQ-ACK enabled by RRC signalling and G-CS-RNTI with HARQ-ACK enabled by RRC signalling.

  - Alt4: UE does not report any HARQ-ACK information for G-CS-RNTI with HARQ-ACK feedback disabled by RRC.

  - Other alternatives are not precluded.

Agreement

When HARQ-ACK for multicast dynamic grant PDSCHs and multicast SPS PDSCHs with ACK/NACK based feedback are multiplexed on the same PUCCH for the same priority case, the PUCCH carrying the multiplexed HARQ-ACK is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast.

For Type-2 HARQ-ACK codebook for multicast, the UE only reports the feedback for the PDSCH scheduled by the DCI with enabled multicast feedback. Therefore, for the multicast DCI with disabled feedback, UE does not report anything and the PRI/K1 fields in DCI with disabled multicast feedback are ignored. The PUCCH resource/slot is determined by the PRI/K1 indicated in the last DCI with enabled feedback.

For Type-1 HARQ-ACK codebook for multicast, the codebook size is not impacted by the enabling/disabling of HARQ-ACK feedback. RAN1 needs to discuss

- Whether/when the UE should report the HARQ-ACK bits for the disabled multicast feedback

Referring to legacy Rel-15/16 UE behavior, if UE does not detect any unicast DCI which has enabled feedback, UE does not send any unicast HARQ-ACK feedback. Similarly, if UE does not detect any DCI with enabled feedback (including unicast and multicast), UE does not report any HARQ-ACK feedback. If UE only detects multicast DCI with disabled feedback, the UE does not report HARQ-ACK feedback.

- Whether to use the PUCCH resource/slot based on the PRI/K1 indicated in the last DCI with enabled feedback or disabled feedback

If UE detects multicast DCI with enabled feedback and disabled feedback, the UE generates the Type-1 codebook for all PDSCH occasion candidates no matter whether the feedback is enabled or disabled. The PUCCH resource/slot can be determined by the PRI/K1 indicated in the last DCI with enabled or disabled feedback, i.e., the PRI/K1 in multicast DCI with disabled feedback are not ignored.

- If report the HARQ-ACK bits for the disabled multicast feedback, whether to report NACK bit or actual decoding results for the corresponding PDSCH

For disabled multicast feedback, gNB is not expecting the decoding results and UE only needs to report NACK without waiting for the decoding results of corresponding PDSCH. The pre-known dummy bits can improve the decoding performance of Reed-Mueller code and polar code at gNB side (similar as disabled feedback in NTN).

- Whether to support dynamic enabling/disabling multicast feedback for Type-1 codebook

The dynamic enabling/disabling HARQ-ACK feedback for multicast is useful for multicast feedback overhead control when Type-2 HARQ-ACK codebook is configured. For Type-1 HARQ-ACK codebook, although the codebook size is not changed because of dynamic enabling/disabling feedback, it could be improve the reliability for gNB to detect the feedback codebook with pre-known NACK bits for the disabled HARQ-ACK feedback.

Based on the above discussion, we propose:

Proposal 1:

If UE does not detect any DCI with enabled feedback (including unicast and multicast), UE does not report any HARQ-ACK feedback even if UE detects multicast DCI with disabled feedback;

If UE detects multicast DCIs with enabled and disabled feedback,

- If UE is configured with Type-1 CB for multicast,

  - UE reports multicast HARQ-ACK feedback on the PUCCH resource/slot based on last DCI with enabled or disabled feedback

  - For a multicast DCI with disabled feedback, UE reports NACK.

- If UE is configured with Type-2 CB for multicast,

  - UE reports multicast HARQ-ACK feedback on the PUCCH resource/slot based on last DCI with enabled feedback.

Considering G-RNTI and G-CS-RNTI can be associated with different multicast services, we prefer separate UE capability and independent configuration of the enabling/disabling HARQ-ACK feedback for dynamic multicast (DG) and SPS multicast. If UE detects both PDSCHs scheduled by DG multicast and SPS multicast with at least one enabled feedback, the UE reports the multicast HARQ-ACK bits for both enabled and disabled feedback in Type-1 codebook generation. According to previous RAN1 agreement, the PUCCH resource/slot is determined by DG multicast. For multicast G-CS-RNTI with disabled feedback, UE will report NACK regardless of decoding results of corresponding PDSCH.

Proposal 2: For a UE configured with Type-1 CB for multicast, if UE detects PDSCH scheduled by dynamic grant multicast and SPS multicast with at least one enabled feedback,

- UE reports multicast HARQ-ACK feedback on the PUCCH resource/slot based on the last dynamic grant multicast DCI with enabled or disabled feedback.

- For multicast G-CS-RNTI or G-RNTI with disabled feedback, UE will report NACK regardless of decoding results of corresponding PDSCH.

## Multiplexing multicast HARQ-ACK feedback in PUSCH

In RAN1#108-e meeting, we have the following agreements on the UL DAI for multicast feedback multiplexed in PUSCH, which have not been reflected in the spec yet.

Agreement

If Type-1 codebook is configured for both multicast and unicast, at least for single cell case for both unicast and multicast:

- If the UE is configured to construct the HARQ-ACK codebooks for unicast and multicast jointly, a single UL DAI bit applies for unicast and multicast

- Otherwise, 1 additional bit UL DAI is included for multicast in DCI format 0_1/0_2, in addition to the UL DAI for unicast. The 1-bit UL DAI for multicast is applied to all configured G-RNTIs.

  - FFS: additional restrictions

Agreement

If Type-2 codebook is configured for unicast and multicast, the following option2-2 (from the previous agreement) is adopted:

- Option2-2: 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast.

    - The 2-bit UL DAI for multicast is applied for all configured G-RNTIs.

        - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs.

          - Alt1-1: UL-DAI indicates the sum of DL-DAIs

          - Alt1-2: DL-DAIs have the same value.

          - Alt1-3: largest DL DAI value among the configured G-RNTIs.

          - Other alternatives are not precluded

Agreement

When Type-1 codebook is configured for unicast and Type-2 codebook is configured for multicast, or when Type-2 codebook is configured for unicast and Type-1 codebook is configured for multicast, the UL-DAI for multicast is included in DCI format 0_1/0_2, in addition to the UL-DAI field for unicast.

- The UL-DAI for multicast is 1-bit for Type-1 codebook

    - The 1-bit UL-DAI for multicast is applied to all configured G-RNTIs.

- The UL-DAI for multicast is 2-bit for Type-2 codebook applied for all configured G-RNTIs.

    - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs.

      - Alt1-1: UL-DAI indicates the sum of DL-DAIs

      - Alt1-2: DL-DAIs have the same value.

      - Alt1-3: largest DL DAI value among the configured G-RNTIs.

      - Other alternatives are not precluded

  - FFS: details of the UL DAI field

We propose the TP#1 for TS 38.212 to reflect the above agreements, where we consider

- If the joint codebook is generated for unicast and multicast feedback (i.e., Type-1 HARQ-ACK codebook using TDMed pattern for unicast and multicast), the 1-bit UL DAI in 1st DAI originally used for unicast is reused for both unicast and multicast feedback;

- Otherwise, the UL DAI for multicast feedback is separate from 1st and 2nd DAI of unicast feedback and can be specified as the 3rd DAI in UL grant.

Proposal 3: Endorse TP#1 for TS 38.212 to reflect the agreements on the UL DAI for multicast feedback.

----------------- Start of TP#1 for TS 38.212 ----------------

##### 7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE.

<Unchanged text is omitted>

-	1st downlink assignment index for unicast feedback – 1, 2 or 4 bits:

-	1 bit for semi-static HARQ-ACK codebook configured for unicast feedback but not for multicast feedback, or for semi-static HARQ-ACK codebook configured for both unicast and multicast and without the higher layer parameter fdmed-Reception-Multicast configured;

-	2 bits for dynamic HARQ-ACK codebook, or for enhanced dynamic HARQ-ACK codebook without UL-TotalDAI-Included configured for unicast feedback;

-	4 bits for enhanced dynamic HARQ-ACK codebook and with UL-TotalDAI-Included = true configured for unicast feedback.

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-1 is configured, if the bit width of the 1st downlink assignment index in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the 1st downlink assignment index in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller 1st  downlink assignment index until the bit width of the 1st downlink assignment index in DCI format 0_1 for the two HARQ-ACK codebooks are the same.

-	2nd downlink assignment index for unicast feedback – 0, 2 or 4 bits:

-	2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks, or for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and without UL-TotalDAI-Included configured for unicast feedback;

-	4 bits for enhanced dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks and with UL-TotalDAI-Included = true configured for unicast feedback;

-	0 bit otherwise.

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-1 is configured, if the bit width of the 2nd downlink assignment index in DCI format 0_1 for one HARQ-ACK codebook is not equal to that of the 2nd downlink assignment index in DCI format 0_1 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller 2nd downlink assignment index until the bit width of the 2nd downlink assignment index in DCI format 0_1 for the two HARQ-ACK codebooks are the same.

-	3rd downlink assignment index for multicast feedback – 0, 1 or 2 bits:

-	1 bit for semi-static HARQ-ACK codebook configured for multicast feedback but not for unicast feedback, or for semi-static HARQ-ACK codebook configured for both unicast and multicast and with the higher layer parameter fdmed-Reception-Multicast configured

-	2 bits for dynamic HARQ-ACK codebook configured for multicast feedback;

-	0 bit otherwise.

<Unchanged text is omitted>

##### 7.3.1.1.3	Format 0_2

DCI format 0_2 is used for the scheduling of PUSCH in one cell.

<Unchanged text is omitted>

-	Downlink assignment index – 0, 1, 2 or 4 bits

-	0 bit if the higher layer parameter downlinkAssignmentIndexDCI-0-2 is not configured;

-	1, 2 or 4 bits otherwise,

-	1st downlink assignment index for unicast feedback – 1 or 2 bits:

-	1 bit for semi-static HARQ-ACK codebook configured for unicast feedback but not for multicast feedback, or for semi-static HARQ-ACK codebook configured for both unicast and multicast and without the higher layer parameter fdmed-Reception-Multicast configured;

-	2 bits for dynamic HARQ-ACK codebook configured for unicast feedback.

-	2nd downlink assignment index for unicast feedback– 0 or 2 bits

-	2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks configured for unicast feedback;

-	0 bit otherwise.

When two HARQ-ACK codebooks are configured for the same serving cell and if higher layer parameter priorityIndicatorDCI-0-2 is configured, if the bit width of the Downlink assignment index in DCI format 0_2 for one HARQ-ACK codebook is not equal to that of the Downlink assignment index in DCI format 0_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller Downlink assignment index until the bit width of the Downlink assignment index in DCI format 0_2 for the two HARQ-ACK codebooks are the same.

-	3rd downlink assignment index for multicast feedback – 0, 1 or 2 bits:

-	1 bit for semi-static HARQ-ACK codebook configured for multicast feedback but not for unicast feedback, or for semi-static HARQ-ACK codebook configured for both unicast and multicast and with the higher layer parameter fdmed-Reception-Multicast configured

-	2 bits for dynamic HARQ-ACK codebook configured for multicast feedback;

-	0 bit otherwise.

<Unchanged text is omitted>

----------------- end of TP#1 for TS 38.212 ----------------

For multicast feedback with Type-2 HARQ-ACK feedback, there is the FFS on how to decide how to count the total number of HARQ-ACK bits of 2-bit UL DAI. From UE perspective, the UE should assume same value of UL DAI per G-RNTI and it can be up to gNB to configure UL DAI to be same as DL DAI for the G-RNTI or not.

For UL DAI of unicast feedback with Type-2 HARQ-ACK codebook, if the DAI field with value VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions for PDCCH with unicast DCI format, the UE does not multiplex HARQ-ACK information for unicast feedback in the PUSCH transmission. This UE behavior in case of VT-DAIUL=4 should be extended to multicast UL DAI as

- If the 2-bit DAI field for multicast feedback with dynamic HARQ-ACK codebook has the value of VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions with DCI format 4_1/4_2, the UE does not multiplex HARQ-ACK information for multicast feedback in the PUSCH transmission.

Proposal 4: If the 2-bit UL DAI included in UL grant is for multicast feedback with Type-2 HARQ-ACK codebook, the UE assume same value of UL DAI per G-RNTI.

  - If the 2-bit DAI field for multicast feedback with dynamic HARQ-ACK codebook has the value of VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions with DCI format 4_1/4_2, the UE does not multiplex HARQ-ACK information for multicast feedback in the PUSCH transmission.

  - Endorse TP#2 for TS 38.213

----------------- Start of TP#2 for TS 38.213 ----------------

#### 9.1.3.2	Type-2 HARQ-ACK codebook in physical uplink shared channel

<Unchanged text is omitted>

If a UE is not provided PDSCH-CodeBlockGroupTransmission and the UE is scheduled for a PUSCH transmission by DCI format that includes a DAI field with value VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions for PDCCH with unicast DCI format scheduling PDSCH receptions or SPS PDSCH release or indicating SCell dormancy on any serving cell c and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission.

If a UE is provided PDSCH-CodeBlockGroupTransmission and the UE is scheduled for a PUSCH transmission by DCI format that includes a DAI field with first value VT-DAIUL=4 or with second value VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions for PDCCH with unicast DCI format scheduling PDSCH receptions or SPS PDSCH release, or DCI format 1_1 indicating SCell dormancy, on any serving cell c and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE does not multiplex HARQ-ACK information for the first sub-codebook or for the second sub-codebook, respectively, in the PUSCH transmission.

If the UE is scheduled for a PUSCH transmission by DCI format that includes a 2-bit DAI field corresponds to multicast feedback with dynamic HARQ-ACK codebook, UE assumes same value VT-DAIUL for each configured G-RNTI. If the 2-bit DAI field for multicast feedback with dynamic HARQ-ACK codebook has the value of VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions for PDCCH with multicast DCI format, the UE does not multiplex HARQ-ACK information for multicast feedback in the PUSCH transmission.

<Unchanged text is omitted>

----------------- end of TP#2 for TS 38.213 ----------------

# 3. Conclusion

For enabling/disabling of HARQ-ACK feedback for multicast

Proposal 1:

If UE does not detect any DCI with enabled feedback (including unicast and multicast), UE does not report any HARQ-ACK feedback even if UE detects multicast DCI with disabled feedback;

If UE detects multicast DCIs with enabled and disabled feedback,

- If UE is configured with Type-1 CB for multicast,

  - UE reports multicast HARQ-ACK feedback on the PUCCH resource/slot based on last DCI with enabled or disabled feedback

  - For a multicast DCI with disabled feedback, UE reports NACK.

- If UE is configured with Type-2 CB for multicast,

  - UE reports multicast HARQ-ACK feedback on the PUCCH resource/slot based on last DCI with enabled feedback.

Proposal 2: For a UE configured with Type-1 CB for multicast, if UE detects PDSCH scheduled by dynamic grant multicast and SPS multicast with at least one enabled feedback,

- UE reports multicast HARQ-ACK feedback on the PUCCH resource/slot based on the last dynamic grant multicast DCI with enabled or disabled feedback.

- For multicast G-CS-RNTI or G-RNTI with disabled feedback, UE will report NACK regardless of decoding results of corresponding PDSCH.

For multiplexing multicast HARQ-ACK feedback in PUSCH

Proposal 3: Endorse TP#1 for TS 38.212 to reflect the agreements on the UL DAI for multicast feedback.

Proposal 4: If the 2-bit UL DAI included in UL grant is for multicast feedback with Type-2 HARQ-ACK codebook, the UE assume same value of UL DAI per G-RNTI.

- If the 2-bit DAI field for multicast feedback with dynamic HARQ-ACK codebook has the value of VT-DAIUL=4 and the UE has not received any PDCCH within the monitoring occasions with DCI format 4_1/4_2, the UE does not multiplex HARQ-ACK information for the corresponding sub-codebooks in the PUSCH transmission.

- Endorse TP#2 for TS 38.213