# 3GPP TSG RAN WG1 #109-e							R1-2204997

# e-Meeting, May 09th – May 20th, 2022

Agenda item:	8.14

Source: 	Qualcomm Incorporated

Title: 	Maintenance on IoT-NTN

Document for:	Discussion and Decision

# Background

In RAN1#108-e, the following agreements for eMTC/NB-IoT over NTN were made:

Agreement

- For IoT NTN, capture into 36.300 a stage-2 description of concept of K_offset, K-mac, UE pre-compensation of timing and frequency pre-compensation/adjustment for uplink transmission with modification as needed

NOTE: NR NTN agreements on TS 38.300 relating to a stage-2 description of concept of K_offset, K-mac, UE pre-compensation of timing and frequency pre-compensation/adjustment for uplink transmission can be adopted for TS 36.300 with modification as needed and can include aspects specific to IoT NTN as needed.

The TP below for TS 36.211 Section 8.1 is endorsed

--------------------------------------- Start of TP for 3GPP TS 36.211 ----------------------------------------

8.1	Uplink-downlink frame timing

<Unchanged Text Omitted>

Transmission of the uplink radio frame number  from the UE shall start TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs seconds before the start of the corresponding downlink radio frame at the UE.

Figure 8.1-1: Uplink-downlink timing relation

<Unchanged Text Omitted>

---------------------------------------- End of TP for 3GPP TS 36.211 ----------------------------------------

The TP below for TS 36.211 Section 8.1 is endorsed

---------------------------------------- Start of TP for 3GPP TS 36.211 ----------------------------------------

8.1	Uplink-downlink frame timing

<Unchanged Text Omitted>

The quantity NTA,adjcommon is derived from the higher-layer parameters TACommon, TACommonDrift, and TACommonDriftVariation if configured (see Clause 4.2.3 in [TS 36.213]), otherwise NTA,adjcommon=0.

<Unchanged Text Omitted>

---------------------------------------- End of TP for 3GPP TS 36.211 ----------------------------------------

Agreement

First discuss for additional values of validity timer for GEO in NR NTN AI 8.4.2. For IoT NTN, adopt the NR NTN agreement without modification for additional values of validity timer for GEO.

Conclusion

RAN1 can wait for RAN2 to conclude discussions on GNSS Measurements.

Conclusion

RAN1 can wait for RAN2 to conclude discussions on validity timer / re-acquisition on NTN-specific SIB.

Conclusion

RAN2 can further discuss when the UE-specific TA report is reported.

Agreement

Satellite velocity vector range can be discussed in NR NTN 8.4.2 first. IoT NTN can use conclusion / agreement from NR NTN without any modification.

The TP below for TS 36.211 Section 8.1 is endorsed

---------------------------------------- Start of TP for 3GPP TS 36.211 ----------------------------------------

8.1	Uplink-downlink frame timing

<Unchanged Text Omitted>

The quantity NTA,adjUE is computed by the UE based on UE position and serving satellite-ephemeris-related higher-layers parameters if configured, otherwise NTA,adjUE=0.

<Unchanged Text Omitted>

---------------------------------------- End of TP for 3GPP TS 36.211 ----------------------------------------

Agreement

Modify bit allocations for orbital parameters ephemeris format as follows:

  - Orbital parameters are indicated in 21 bytes payload:

    - Semi-major axis α (m) is 33 bits

      - Range: from 6500 km to 43000 km

      - The quantization step is 4.249 ×10-3m

    - Eccentricity e is 20 bits

      - Range: ≤ 0.015

      - The quantization step is 1.431 ×10-8

    - Argument of periapsis ω (rad) is 28 bits

      - Range: from 0 to 2π

      - The quantization step is 2.341 ×10-8 rad

    - Longitude of ascending node (Ω rad) is 28 bits

      - Range: from 0 to 2π

      - The quantization step is 2.341  ×10-8 rad

    - Inclination i (rad) is 27 bits

      - Range: from - π/2  to + π/2

      - The quantization step is 2.341  ×10-8 rad

    - Mean anomaly M (rad) at epoch time to is 28 bits

      - Range: from 0 to 2π

      - The quantization step is 2.341  ×10-8 rad

Agreement

For epoch time signaling for IoT NTN:

- When explicitly provided through SIB, Epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information.

- When provided through dedicated signaling, epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number.

- The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point.

Working assumption

Adopt NR NTN solution for negative TACommonDriftVariation values.

Working assumption

Adopt NR NTN solution for interpretation SFN indicating Epoch time.

Agreement

RAN1 kindly suggests to spec editor of TS 36.213 to move the following text to the head of section 6.1.1:

========= Unchanged Text Omitted ==========

Throughout this clause, for a BL/CE UE, if the UE is configured with the higher layer parameter CellSpecificKoffset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter CellSpecificKoffset provided by higher layers, and

KUE_offset is the parameter UESpecificKoffset provided by higher layers, otherwise KUE_offset=0

otherwise,

-	Koffset=0, Kcell_offset=0.

======== Unchanged Text Omitted ===============

Agreement

RAN1 kindly suggests to spec editor of TS 36.213 to adopt the following TP for section 9.1.5:

<<<<< Start of TP to TS 36.213 section 9.1.5 >>>>>>>

If the UE has initiated a PUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the MPDCCH UE-specific search space in a search space window starting in subframe n+4+Kmac Kmac with duration given by higher layer parameter pur-MPDCCH-SS-window-duration where Kmac is provided by higher layer parameter K-mac, otherwise Kmac=0. Upon detection of a MPDCCH with DCI format 6-0A/6-0B with CRC scrambled by PUR-RNTI intended for the UE within the search space window and the corresponding DCI is for PUR ACK/fallback indication (as defined in [4]), the UE is not required to monitor the MPDCCH UE-specific search space for the remaining search space window duration.

<<<<< End of TP to TS 36.213 section 9.1.5 >>>>>>>

Agreement

RAN1 kindly suggests to spec editor of TS 36.213 to adopt the following TP for section 16.5.1:

<<<< START of TEXT PROPOSAL for TS36.213 section 16.5.1 >>>>>>>>>>>>>

16.5.1	UE procedure for transmitting format 1 narrowband physical uplink shared channel

NPUSCH format 1 transmission can be scheduled by a NPDCCH with DCI format N0, or the transmission can correspond to using preconfigured uplink resource configured by higher layers. Transmission using preconfigured uplink resource is initiated by higher layers as specified in [14] , while retransmission of transport blocks transmitted using preconfigured uplink resource are scheduled by a NPDCCH with DCI format N0.

A UE shall upon detection on a given serving cell of a NPDCCH with DCI format N0 ending in NB-IoT DL subframe n scheduling NPUSCH intended for the UE, perform, at the end of

-	n+k0+Koffset DL subframe for FDD,

-	k0 NB-IoT UL subframes following the end of n+8+Koffset subframe for TDD,

a corresponding NPUSCH transmission using NPUSCH format 1 in N consecutive NB-IoT UL slots ni with i = 0, 1, …, N-1 according to the NPDCCH information where

-	subframe n is the last subframe in which the NPDCCH is transmitted and is determined from the starting subframe of NPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI; and

-	, where the value of is determined by the repetition number field in the corresponding DCI (see Clause 16.5.1.1), the value of is determined by the resource assignment field in the corresponding DCI (see Clause 16.5.1.1), the value of  is the number of NB-IoT UL slots of the resource unit (defined in clause 10.1.2.3 of [3]) corresponding to the  allocated number of subcarriers (as determined in Clause 16.5.1.1) in the corresponding DCI, and the value of is determined by the Number of scheduled TB for Unicast field, if present, in the corresponding DCI,  otherwise

-	n0 is the first NB-IoT UL slot starting after the end of subframe n+k0+Koffset for FDD

-	n0 is the first NB-IoT UL slot starting after k0 NB-IoT UL subframes following the end of n+8+Koffset subframe for TDD

-	value of k0 is determined by the scheduling delay field () in the corresponding DCI according to Table 16.5.1-1 for FDD and Table 16.5.1-1A for TDD

<<<< END of TEXT PROPOSAL for TS36.213 section 16.5.1 >>>>>>>>>>>>>

Agreement

RAN1 kindly suggests to spec editor of TS 36.213 to adopt the following TP for section 8.0:

<<<< START of TEXT PROPOSAL for TS36.213 section 8.0 >>>>>>>>>>>>>

A BL/CE UE shall upon detection on a given serving cell of an MPDCCH with DCI format 6-0A/6-0B scheduling PUSCH intended for the UE, perform a corresponding PUSCH transmission in subframe(s) ni = n+ki+Koffset if a transport block(s) corresponding to the HARQ process(es) of the PUSCH transmission is generated as described in [8] with i = 0, 1, …, NTBN-1 according to the MPDCCH, where

-	subframe n is the last subframe in which the MPDCCH is transmitted;

-	the value of is the number of scheduled TB determined by the corresponding DCI if present,  otherwise;

-	 and the value of  is determined by the repetition number field in the corresponding DCI, where

-	if the UE is configured with higher layer parameter ce-pdsch-puschEnhancement-config with value 'On' are given by {1,2,4,8,12,16,24,32}

-	otherwise, are given in Table 8-2b and Table 8-2c; and

-	if the UE is configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15, and the PUSCH resource assignment in the corresponding DCI is using uplink resource allocation type 5,  where N ≤ 32 for CE Mode A and N ≤ 2048 for CE Mode B,  is defined in [3] and  is determined according to procedure in clause 8.1.6,  otherwise

-	in case N>1, subframe(s) n+ki+Koffset with i=0,1,…, NTBN-1 are NTBN consecutive BL/CE UL subframe(s) starting with subframe n+x+Koffset, and in case N=1, k0=x;

<<<< Portion of specification removed >>>>>

-	for FDD, x = 4;

<<<< END of TEXT PROPOSAL for TS36.213 section 8.0 >>>>>>>>>>>>>

Agreement

In IoT NTN, the Koffset value signalled in system information (cell specific Koffset) is always used for NPDCCH and MPDCCH ordered NPRACH and PRACH timing relationships, respectively.

Agreement

RAN1 kindly suggests to spec editor of TS 36.213 to adopt the following TP for section 16.3.2:

<<<< START of TEXT PROPOSAL for TS36.213 section 16.3.2 >>>>>>>>>>>>>

In case a random access procedure is initiated by a "PDCCH order" ending in subframe n, the UE shall, if requested by higher layers, start transmission of random access preamble at the end of the first subframe n+k2+Kcell_offset, , where a NPRACH resource is available.

<<<< END of TEXT PROPOSAL for TS36.213 section 16.3.2 >>>>>>>>>>>>>

Working assumption

For IoT NTN, the unit of K_mac and Koffset when subcarrier spacing is 3.75kHz is 1 ms.

Agreement

Confirm the WA on units of Kmac and Koffset with 3.75kHz SCS:

Agreement

For IoT NTN, calculate UE-eNB RTT using the following equation: RTTUEeNB=floor(TTATf)+Kmac

where Tf = subframe duration (1ms).

Agreement

The TP below for TS 36.213 Clause 16.1.2 is endorsed

<<< Start of TP to Clause 16.1.2, TS 36.213 >>>

For a timing advance command reception ending in DL subframe n, the corresponding adjustment of the uplink transmission timing shall apply from the first available NB-IoT uplink slot following the end of n+12+Koffset DL subframe and the first available NB-IoT uplink slot is the first slot of a NPUSCH transmission.

<<< End of TP to Clause 16.1.2, TS 36.213 >>>

In this contribution, we provide our views on the remaining aspects related to IoT-NTN maintenance in Release 17.

# Gaps in segmented pre-compensation for long uplink Tx

It has been agreed that—based on a UE capability—certain UEs may need gaps between segments, when performing segmented pre-compensation for uplink transmission in NTNs. This behaviour, i.e., that a UE indicating the need of such gaps is not expected to transmit in the uplink for the duration of the gap (in slots), needs to be captured in the RAN1 specifications. Currently, uplink gaps for other purposes (e.g., the 40 ms uplink gap after 256 ms of continuous transmission), are specified in TS 36.211 Sections 5.3.4 (for eMTC, based on the capability ue-CE-NeedULGaps) and 10.1.3.6 (for NB-IoT). A similar approach may be adopted to describe the uplink gaps due to segmented pre-compensation in NTNs.

Below, we provide candidate TPs to capture this.

<TP1, Section 5.3.4, TS 36.211>

For BL/CE UEs communicating over NTN, for PUSCH transmission, for frame structure type 1, after a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a gap of Ngapprecompensation time units shall be inserted, according to the UE capability ue-CE-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331. BL/CE UL subframes within the gap of Ngapprecompensation time units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensation is one subframe.

For BL/CE UEs in CEModeB, for PUSCH transmission not associated with Temporary C-RNTI, for frame structure type 1, after a transmission duration of  time units (which may include subframes that are not BL/CE UL subframes), a gap of  time units shall be inserted, according to the UE capability ue-CE-NeedULGaps, as specified in 3GPP TS 36.331 [9]. BL/CE UL subframes within the gap of  time units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH.

</TP1>

<TP2, Section 10.1.3.6, TS 36.211>

For a UE communicating over NTN, after transmissions and/or postponements due to NPRACH of Nsegmentprecompensation time units, for frame structure type 1, a gap of Ngapprecompensation time units shall be inserted according to the UE capability ue-NBIOT-NeedSegmentedPrecompensationGaps , as specified in 3GPP TS 36.331. UL slots within the gap of Ngapprecompensation time units shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensation is one slot. The portion of a postponement due to NPRACH which coincides with a gap is counted as part of the gap.

NPRACH gaps as defined in clause 10.1.6.1 are not part of the NPRACH resource. For frame structure type 2, the valid uplink subframes which are not used for NPRACH transmission when it is not possible to map G symbol groups back-to-back are not part of the NPRACH resource. The mapping of  is then repeated until  slots have been transmitted. After transmissions and/or postponements due to NPRACH of  time units, for frame structure type 1, a gap of  time units shall be inserted where the NPUSCH transmission is postponed. The portion of a postponement due to NPRACH which coincides with a gap is counted as part of the gap.

</TP2>

Proposal 1: Specify capability-based uplink gaps due to segmented pre-compensation in TS 36.211, during which a UE indicating the need of such gaps is not expected to transmit in the uplink for the requisite number of uplink slots.

- These may be described in the “Mapping to physical resources” sections for PUSCH (Section 5.3.4) and NPUSCH (Section 10.1.3.6), analogous to the existing 40 ms gap after 256 ms of continuous uplink transmission.

- Candidate TPs capturing this—TP1 and TP2 in this contribution—may be endorsed.

- Monitoring restrictions due to half-duplex constraint

While the monitoring restrictions in the downlink (corresponding to an uplink transmission) have been captured for several cases in TS 36.213, the following additional case should reflect a similar understanding (potential edit shown in red text below).

Post-NPUSCH

- - … if the NB-IoT UE has a NPUSCH transmission ending in subframe n (accounting for uplink transmission timing), the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+3.

Proposal 2: Modify the specification for DL monitoring restrictions after NPUSCH transmission to account for uplink transmission timing.

- Physical vs logical time

In the current specifications—especially when considering the impact of large TAs in NTNs—there is considerable scope for confusion between when physical time is used, vs when logical time is used. For example, in the monitoring restrictions (described above in Section 2), the uplink transmission time is “physical time”, such that the monitoring restrictions apply in the physical neighbourhood of it. However, in most of the other cases (such as timelines involving Koffset for uplink transmission), the transmission time refers to an “index”, i.e., it is logical time.

Below are two example to highlight the incongruency: both in the context of a “PDCCH order ending in subframe n”.

Example 1.1 (Logical time, referring to an UL index)

“… In case a random access procedure is initiated by a "PDCCH order" ending in subframe n, the UE shall, if requested by higher layers, start transmission of random access preamble at the end of the first subframe n+k2+Koffset, , where a NPRACH resource is available.  …”

Example 1.2 (Physical time, inherently referring to a DL index for an UL transmission)

“… If a NB-IoT UE detects NPDCCH with DCI Format N1 for "PDCCH order" ending in subframe n, and

-	for FDD, if the corresponding NPRACH transmission starts from subframe n+k (accounting for uplink transmission timing), the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1. …”

To keep everything in logical time (DL reception follows a DL index, while UL transmission follows a UL index, with UL indices shifted back by a TA term), we had originally proposed the half-duplex monitoring restrictions to be written in terms of logical time. However, the latest CRs don’t reflect this.

To this end, we propose to clarify the issue of physical and logical times occurring in different places, either by making things uniform—specifically, by writing the half-duplex monitoring restrictions in logical time—or, by including an explanatory table in the specification to highlight the cases in which physical time is used (current wording for DL monitoring restrictions in half-duplex) and the other cases (such as UL-DL transmission timelines) where logical time is used.

Proposal 3: Clarify the use of logical/physical time in different places of the specifications.

- Option 1: For half-duplex monitoring restrictions (such as Example 1.2 in this contribution), use logical time to index uplink transmissions with UL indices, DL reception with DL indices, and use a TA term to link the two.

- Option 2: Include a table in the specifications to state which relationships use logical time, and which use physical time.

  - Currently, only the half-duplex monitoring restrictions appear to use physical time, while other timing relationships use logical time.

# Feeder Link TA drift rate compensation in GEO

Currently, the Feature Group corresponding to “Basic IoT over NTN support” (FG 2-1 in the draft version of R1-2111813) mentions the following as a component:

- “…6. UE applies common TA [in RRC_IDLE and RRC_CONNECTED] according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) (if any) …”

The common TA parameters in general include the parameters TACommon, TACommonDrift and TACommonDriftVariation, with the latter two indicating adjustments related to the common TA drift. However, in GEO NTNs (which is the intended base case corresponding to FG 2-1), the drift/variation in the common TA is negligible. To this end, for simplified operation over GEO NTNs, we propose to modify the requirements on applying common TA to only include the common TA (i.e., the term TACommon), and not the drift/variation parameters.

Proposal 4: For GEO NTNs, a UE applies common TA only in accordance with the common TA term, TACommon, and is not required to process the drift/variation parameters TACommonDrift and TACommonDriftVariation.

# Frequency pre-compensation description in TS 36.211

Current RAN1 specifications do not capture the concept of frequency pre-compensation that the UE needs to apply when communicating with an NTN, for a successful uplink transmission. For timing pre-compensation, the definition of TA does capture the NTN-specific terms, NTA,adjcommon and NTA,adjUE, in Section 8.1 of 3GPP TS 36.211. To make the RAN1 specifications clear, we propose to similarly describe the concept of frequency pre-compensation (or, frequency adjustment) for uplink transmission in TS 36.211. This may be included as a separate (new) section on uplink frequency, or, alternatively, be described in the SC-FDMA signal generation for the uplink, in Section 5.6 (eMTC) and Section 10.1.5 (NB-IoT).

Even if, later, performance requirements in RAN4 specifications implicitly capture the aspect of uplink frequency pre-compensation needed to be performed by the UE to meet the requirements, it is worth noting that the RAN4 specifications for IoT over NTN will not be ready in Release 17. As a result, it is imperative that the RAN1 specifications describe the concept of frequency pre-compensation.

Proposal 5: Describe the concept of (NTN-specific) frequency pre-compensation/adjustment for uplink transmission in the physical layer specifications (3GPP TS 36.211), analogous to the current description of NTN-specific timing pre-compensation in Section 8.1 of TS 36.211 on Timing Advance.

- Option 1: Create a new section to describe frequency pre-compensation/adjustment for uplink transmission

- Option 2: Describe frequency pre-compensation in the sections on SC-FDMA signal generation (5.6 and 10.1.5)

# Conclusion

In this contribution we presented our views on enhancements to NB-IoT/eMTC over NTN as it relates to time and frequency synchronization. We summarize our proposals below.

Proposal 1: Specify capability-based uplink gaps due to segmented pre-compensation in TS 36.211, during which a UE indicating the need of such gaps is not expected to transmit in the uplink for the requisite number of uplink slots.

- These may be described in the “Mapping to physical resources” sections for PUSCH (Section 5.3.4) and NPUSCH (Section 10.1.3.6), analogous to the existing 40 ms gap after 256 ms of continuous uplink transmission.

- Candidate TPs capturing this—TP1 and TP2 in this contribution—may be endorsed.

Proposal 2: Modify the specification for DL monitoring restrictions after NPUSCH transmission to account for uplink transmission timing.

Proposal 3: Clarify the use of logical/physical time in different places of the specifications.

- Option 1: For half-duplex monitoring restrictions (such as Example 1.2 in this contribution), use logical time to index uplink transmissions with UL indices, DL reception with DL indices, and use a TA term to link the two.

- Option 2: Include a table in the specifications to state which relationships use logical time, and which use physical time.

  - Currently, only the half-duplex monitoring restrictions appear to use physical time, while other timing relationships use logical time.

Proposal 4: For GEO NTNs, a UE applies common TA only in accordance with the common TA term, TACommon, and is not required to process the drift/variation parameters TACommonDrift and TACommonDriftVariation.

Proposal 5: Describe the concept of (NTN-specific) frequency pre-compensation/adjustment for uplink transmission in the physical layer specifications (3GPP TS 36.211), analogous to the current description of NTN-specific timing pre-compensation in Section 8.1 of TS 36.211 on Timing Advance.

- Option 1: Create a new section to describe frequency pre-compensation/adjustment for uplink transmission

- Option 2: Describe frequency pre-compensation in the sections on SC-FDMA signal generation (5.6 and 10.1.5)