| 3GPP TSG-RAN WG1 Meeting #109-e e-Meeting, May 9th – May 20th, 2022 | R1-2205005 |
| --- | --- |

Agenda item:	8.16.8

Source: 	Qualcomm Incorporated

Title: 	UE Features for NR Coverage Enhancement

Document for:	Discussion/Decision

# Discussion on UE features

We have the following remarks on the UE features for NR coverage enhancement.

General remark applicable to all features for NR coverage enhancement

Proposal 1: Unless otherwise stated, the type for a UE feature should be at least per band (if not with finer granularity type), given the potential UE testing differentiation among licensed, unlicensed, and NTN band.

On PUSCH Type A repetition enhancements

We make the following proposals to revise the UE capabilities for PUSCH Type A repetitions.

Proposal 2: UE capabilities for enhanced PUSCH Type A repetitions are revised as follows:

- The description of FG 30-2 is revised to clarify that PUSCH Type A repetitions based on available slot counting is only applicable when K > 1.

- The prerequisites for FG 30-2 are one of {5-14. 5-16, 5-17}.

On DMRS bundling for PUSCH/PUCCH repetitions

We have the following proposals on UE capability reporting for PUSCH/PUCCH DMRS bundling.

Proposal 3:  On UE features 30-4 to 30-4g: all features on DMRS Bundling (PUSCH and PUCCH) to be indicated at the per FS granularity.

Proposal 4: On UE feature 30-4 for maximum duration for DMRS bundling, since a UE may report different values for different modulation orders, choose from the following two options:

- restrict applicability of 30-4 to QPSK or lower modulation order, or

- introduce new FGs to cover the remaining modulation orders.

On TBOMS

We make the following proposals to revise the UE capabilities for TBOMS.

Proposal 5: UE capabilities for TBOMS are revised as follows:

- The prerequisites for FG 30-3 are 30-2.

- The prerequisites for FG 30-3a are 30-3.

# References

- R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, 3GPP TSG RAN1 meeting #106-e, e-Meeting, August 16th – 27th, 2021

- R1-2200254, Summary on UE features for NR coverage enhancement, TSG RAN1 meeting #107bis-e, e-meeting, January 17th – 25th, 2022

- R1-2112902, Revised RAN1 UE features list for Rel-17 NR, 3GPP TSG RAN1 meeting #107-e, e-Meeting, November 11th – 19th, 2021

# Appendix A: Agreements on UE Features for Coverage Enhancement in R1-108-e

Agreement

  - Merge FGs 30-2 and 30-2a into an FG with per band

Agreement

- Merge FGs 30-1 and 30-1a into an FG with per band

  - Value range K = {20, 24, 28, 32}

Agreement

- FG 30-3a is not separated to multiple FGs

Agreement

- FG 30-4 is updated as follows

| 30. NR_cov_enh | 30-4 | [The maximum duration for DM-RS bundling] | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH FFS dependence on modulation order FFS dependence on back-to-back vs. non-back-to-back repetitions |  | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | [Per UE] Per band | FFS N/A | No N/A | N/A | [This is applicable for QPSK or lower modulation orders] From RAN1 perspective, all modulation orders are applicable. RAN4 to decide whether this note is required not | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FFS Send an LS to ask RAN4 whether RAN4 specifies requirements for modulation order higher than QPSK

Agreement

- FG 30-4b is not separated to one for within a slot and the other for over slots

Agreement

- FG 30-5 is not split to one for PUCCH formats 1/3/4 and the other for PUCCH formats 0/2

Agreement

- Capture the following in the LS to RAN2/RAN4 on updated RAN1 UE features list for NR

  - Based on RAN4 feedback, FG 30-4 is defined without distinction of the modulation order. RAN1 has no intention to revert any existing RAN4 agreements on maximum duration. It is up to RAN4 to decide whether FG 30-4 is applicable only to QPSK and lower modulation orders

  - To RAN WG4

    - ACTION: RAN1 kindly asks RAN4 to take the above information into account for their future discussion.

- Remove the text in the note column in FG 30-4

Agreement

- FG 30-3 is not split into 2 separate FGs: 1st one for DG, 2nd one for CG

Agreement

- Type of FG 30-3 is per band

Agreement

- Type of FG 30-3a is per band

Agreement

- Add a note in FG 30-4c: “Note: If a UE reports support of FG 30-3a and 30-4c, the UE supports DMRS bundling for the repetitions of TBoMS”

Agreement

- FG 30-6 is supported as optional with capability signaling

Agreement

- FG 30-1 is updated as follows

| 30. NR_cov_enh | 30-1 | Increased maximum number of dynamic grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. Maximum value of K (the number of repetitions) = 32 For DG PUSCH, Tthe number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. For Type 1 CG PUSCH, the number of repetitions is indicated by repK-r17. For Type 2 CG PUSCH, the number of repetitions is indicated in a TDRA list or by repK-r17. A row index of the TDRA list is indicated by a DCI. A row index of the TDRA list is indicated by a Type 2 configured grant configuration. | 5-14, 5-16, or [5-17] | Yes | N/A | UE does not support more than 16 repetitions for dynamic grant PUSCH. | [Per UE] Per band | No N/A | No N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1a | Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a Type 2 configured grant configuration. FFS whether to merge with FG 30-1 FFS whether to have a separate FG for CG (including both Type 1 and Type 2) with repK-r17 | [5-16], [30-1] | Yes | N/A | UE does not support more than 16 repetitions for Type 2configurecd grant PUSCH. | [Per UE] | No | No | N/A |  | Optional with capability signalling |

Agreement

- FG 30-2 is updated as follows

| 30. NR_cov_enh | 30-2 | Dynamic grant PUSCH Type A repetitions based on available slots | Transmission occasions for [K] repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots. | One of {5-14. 5-16, [5-17], [11-6, 30-1]} | Yes | N/A | UE does not support dynamic or configured grant PUSCH repetitions counted on the basis of available slots. | [Per UE] Per band | FFS N/A | No N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. FFS whether to merge with FG 30-2 | [5-14 or 5-16], [30-2] | Yes | N/A | UE does not support configured grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |

Agreement

- Type of FG 30-5 is per UE

# Appendix B: Agreements on UE Features for Coverage Enhancement in R1-107bis-e

Agreement

- Further discuss whether/how to separate/merge FGs 30-1 and 30-1a from following options

  - Option 1: Keep current structure, i.e. FG 30-1 for DG, 30-1a for type 1 and 2 CG

  - Option 3: Merge FGs 30-1 and 30-1a into an FG

Agreement

- Further discuss whether/how to separate/merge FGs 30-2 and 30-2a from following options

  - Option 1: Keep current structure, i.e., FGs 30-2 for DG, 30-2a for type 1 and 2 CG

  - Option 3: Merge FGs 30-2 and 30-2a into an FG

Agreement

- Further discuss whether/how to separate FG 30-3 from following options

  - Option 1: Keep current structure

  - Option 3: Split 30-3 into 2 separate FGs: 1st one for DG, 2nd one for CG

Agreement

- FG 30-3 and FG30-3a are not merged

Agreement

- FG 30-4x are split to back-to-back transmission and non-back-to-back transmission

  - FFS whether to introduce as many FG as FG30-4x or one FG for all FG30-4x for non-back-to-back transmission

Agreement

- FG for intra-slot frequency hopping and inter-slot frequency hopping for TBoMS is not introduced

Agreement

- Introduce one FG for all FG30-4x for non-back-to-back transmission as follows

| 30. NR_cov_enh | 30-4h | DM-RS bundling for non-back-to-back transmission | Support DM-RS bundling for non-back-to-back transmission for consecutive slots for PUSCH and PUCCH only for corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d) | 30-4a, 30-4b, 30-4c, or 30-4d | Yes | N/A | UE does not Support DM-RS bundling for non-back-to-back transmission | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FGs 30-4 to 30-4d are not merged

  - FFS: whether to Add a note in FG 30-4c: “Note: If a UE reports support of FG [30-4a], 30-4c, [30-3] and/or 30-3a, the UE supports DMRS bundling for the repetitions of TBoMS”

Agreement

- FGs 30-4e and 30-4f are not merged

Agreement

- Component of FG 30-4d is updated as “Support DM-RS bundling for PUCCH repetitions for PUCCH formats 1/3/4”