3GPP TSG RAN WG1 #109-e		R1- 2205007

May 9th – Math 20th, 2022

Agenda item:	8.16.11

Source: 	Qualcomm

Title: 	UE Features for Sidelink Enhancements

Document for:	Discussion/Decision

- Introduction

In this contribution, we provided discussion and proposals for the remaining issues in the set of UE features for NR Sidelink Enhancements [1].

# Feature Group Types

Tranmission of inter-UE coordination requires that UE-A perform sensing and additional resource selection procedure on behalf of UE-B in Scheme 1 in addition to preparing and transmitting the inter-UE coordination packets. In Scheme 2, it requires that the UE perform the measurements and computations required for detecting conflicts as well as preparing and transmitting the Scheme 2 PSFCHs with all the associated prioritization operations. These operations place additional demands on the baseband processing of the UE and the UE’s ability to performs them is impacted by other featuresets that the UE supports in this band and in other bands. Hence, we propose that the FGs for transmitting inter-UE coordination information in Scheme 1 and Scheme 2 be defined per featureset.

Proposal 1: FG 32-5a-1 and FG 32-5b-1 are defined per featureset.

# Synchornization Reference in FG 32-4b

RAN1 agreed to introduce a featuregroup for synchronization sources with the following component highlighted in yellow for further discussion:

[6) UE supports SyncRef UE as the synchronization reference and can receive S-SSB in NR sidelink if it supports 15-1.]

The ability to use a SyncRef UE as a synchronization reference is important to ensure communications when a UE is out of gNB coverage and GNSS reception is unavailable or unreliable. Hence, we propose to support Component 6 in FG 32-4b.

Proposal 2: FG 32-4b includes Component 6 “UE supports SyncRef UE as the synchronization reference and can receive S-SSB in NR sidelink if it supports 15-1.”

# Prerequisites

In the Release 17 sidelink UE feature discussions, RAN1 expended significant time discussion how to handle and undo the Release 16 sidelink UE feature prerequisites. To avoid such discussions in the future, we propose to not introduce additional prerequisites beyond what has already been agreed.

Proposal 3: No additional prerequisites are introduced beyond the already agreed ones.

# Feature Groups for PSFCH and S-SSB Reception

In RAN #95, it was agreed to have two FGs for reception of PSFCH and S-SSB:

Agreement:

- Split FG 32-2 into two sub-FGs

  - One sub-FG for support of only S-SSB reception

  - Another sub-FG for support of PSFCH reception with pre-requisite of S-SSB reception

We propose to update the UE feature list to capture the above agreement.

Proposal 4: Replace FG 32-2 with two FGs per the RAN #95 agreement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-2a | Receiving NR sidelink of PSFCH | 1) UE can receive NR PSFCH format 0 2) UE can receive up to N PSFCH(s) resources in a slot. | 32-2b | Yes | Yes |  | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for N are {5, 15, 25, 32, 35, 45, 50, 64} If UE reports more than one FGs of 15-11 and 32-5b-2, the reported value N in each FG is the total number and the same among those FGs. | Optional with capability signalling. |
| 32. NR_SL_enh | 32-2b | Receiving NR sidelink of S-SSB | 1) UE can receive NR S-SSB. 2) UE supports SyncRef UE as the synchronization reference | None | No | Yes |  | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |

# Conclusion

Proposal 1: FG 32-5a-1 and FG 32-5b-1 are defined per featureset.

Proposal 2: FG 32-4b includes Component 6 “UE supports SyncRef UE as the synchronization reference and can receive S-SSB in NR sidelink if it supports 15-1.”

Proposal 3: No additional prerequisites are introduced beyond the already agreed ones.

Proposal 4: Replace FG 32-2 with two FGs per the RAN #95 agreement

# References

[1]  R1-2202929, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e　including remaining RAN1 issues,” Moderators  (AT&T, NTT DOCOMO, INC.), RAN1 #108-e.